3GPP TSG RAN WG2#130   	R2-2504572

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.2.2

Source:	III

Title:	Discussion on paging procedure for Ambient-IoT

Document for:	Discussion

Introduction

In the previous RAN2#129-bis meeting, some agreements were achieved in the following:

Agreements < Behavior for Parallel service requests> - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. < How to specify implicit> - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. < Paging Identifier Length> - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to tak this into account for their design.

Based on RAN2 progress, this contribution discusses different service request from different readers, transaction ID and device ID.

Discussion

Different service request is received from different reader

Different service request received from different readers was discussed before. In RAN2#129, some agreements were reached:

In RAN2#129 agreement: <Simultaneous Paging Procedures> - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed).

In device perspective, the device may receive a new service request arriving from different readers while an ongoing procedure is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s), the device focuses on the current request in progress and

ignores any other requests from the different reader until the current request completes or fails.

Option 2: The device executes the new service request, the device skips old request process and always

executes the last received request.

Option 3: The device will process higher priority request, the device determines the execution order based on

the priority of the request, ensuring that high-priority requests are processed.

FFS which solution if any for device behaviour, if it gets a new service request while one procedure is still ongoing or leave it to implementation.

Proposal 1: A new service request is received from different reader(s) while an ongoing procedure

is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s).

Option 2: The device executes the new service request.

Option 3: The device will process higher priority request.

In RAN2#129-bis offline discussion, option 1 and option 2 were discussed as option A and option B. In option A, the starting and ending point of the “ongoing procedure” should be clearly specified. The starting point is when the device receives a paging message from the reader in device point of view. The ending point can be implicitly indicating by R2D message (e.g. NACK, subsequent paging message, new paging message).

Proposal 2: The starting point of ongoing procedure is when the device receives a paging message from the reader in device point of view.

Proposal 3: The ending point of ongoing procedure can be implicitly indicating by R2D message.

There are several cases for ending point should be discussed from different R2D messages:

- When the R2D message is MSG2, the device should receive the MSG2 signal in time window (R2Dmin, R2Dmax). If the device doesn’t receive MSG2 in the time window, the device will think ongoing procedure as end and won’t transmit MSG3. The reader can’t receive MSG3 and the reader will decide to trigger re-access via an R2D message, such as figure 1.

Figure 1 MSG2 signal in time window (R2Dmin, R2Dmax)

- When the R2D message is MSG4 including NACK and received by device, device will think the ongoing procedure as end and the reader will decide to trigger re-access via an R2D message.

- The reader won’t transmit a R2D message (MSG4 including NACK) when MSG3 was correctly received by reader. The device can understand the ACK implicitly when R2D message (ex. new paging message for different transaction ID) is received and the device will think the ongoing procedure as end.

- When the R2D message is MSG4 including NACK and not received by device, device will think the ongoing procedure as end. When the reader decides to trigger re-access via an R2D message (ex. Subsequent paging message for same transaction ID). And this R2D message is received by device, the device is implicitly indicated the NACK was transmitted.

Transaction ID

In TR 38.769[1] describe, the reader transmits multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response should be avoided for the same service request. In RAN2#128 discussion, session ID/transaction ID between CN and reader (i.e., based on the candidate solutions of SA2) can be used the ID to avoid duplicated response for same service request.

In previous RAN2#128 meetings, as following:

Agreements on paging - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size

In RAN2#129 agreement, the transaction ID can be generated by reader based on CN

Agreements - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID.
We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB) of the CN Correlation ID to generate the transaction ID (but make sure X is large enough to avoid collisions),

Option 2: A mapping mechanism based on CN correlation IDs,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 1: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Ambient IoT Device ID

In the RAN2#129, agreement about paging ID:

Agreements on paging ID - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

The Ambient IoT Device ID was defined by TR 23.700-13 [2], it includes multiple identifiers.

Based on the above consideration, following components are considered necessary to compose the Ambient IoT Device ID, which is defined by 3GPP: - Home Network Identifier: an identifier used to identify the home MNO; - Owner Identifier: an identifier used to identify a 3rd party who sends service requests to trigger 5GC to perform Ambient IoT service operation; NOTE 1: The Owner Identifier is allocated by the home MNO corresponding to the Home Network Identifier. Editor’s Note: The use of the Owner Identifier is FFS and may depend on the solution for Inventory, etc. - Instance Identifier: an identifier used to identify a specific Ambient IoT device owned by the 3rd party. NOTE 2: The Instance Identifier is allocated by the home MNO which may coordinate with the 3rd party. Figure 6.2.1-2: Structure of Ambient IoT Device ID and optional 3rd Party-defined Identifier

In TS 23.003[3], the SUPI is a globally unique 5G subscription permanent identifier allocated to each subscriber in the 5G System. It is feasible to choose IMSI as SUPI type for the Ambient-IoT Device ID.

- 2.2A Subscription Permanent Identifier (SUPI) The SUPI is a globally unique 5G Subscription Permanent Identifier allocated to each subscriber in the 5G System. It is defined in clause 5.9.2 of 3GPP TS 23.501 [119]. The SUPI is defined as: - a SUPI type: in this release of the specification, it may indicate an IMSI, a Network Specific Identifier (NSI), a Global Line Identifier (GLI) or a Global Cable Identifier (GCI); and - dependent on the value of the SUPI type: - an IMSI as defined in clause 2.1; - a Network Specific Identifier (NSI), taking the form of a Network Access Identifier (NAI) as defined in clause 28.7.2; - a Global Cable Identifier (GCI) taking the form of a NAI as defined in clause 28.15.2; - a Global Line Identifier (GLI) taking the form of an NAI as defined in clause 28.16.2. NOTE: Depending on the protocol used to convey the SUPI, the SUPI type can take different formats.

Base on above, the single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness of device ID in single/multiple readers. This should be further discussed in RAN2.

Observation 2: The single device ID in paging message can be based on Ambient IoT Device ID /IMSI for uniqueness in single/multiple readers.

- Conclusion

We have the following proposal and observation:

Proposal 1: A new service request is received from different reader(s) while an ongoing procedure

is still in progress. The following options can be considered as baseline:

Option 1: The device rejects new request(s).

Option 2: The device executes the new service request.

Option 3: The device will process higher priority request.

Proposal 2: The starting point of ongoing procedure is when the device receives a paging message from the reader in device point of view.

Proposal 3: The ending point of ongoing procedure can be implicitly indicating by R2D message.

Observation 1: When the CN correlation ID was clarified by SA2, the reader can generate a transaction ID based on the CN correlation ID. We provides three options for further discussion.

Option 1: Use X bits (e.g. LSB),
Option 2: A mapping mechanism,

Option 3: The reader selects a transaction ID that has not been used recently to avoid ID duplication.

Observation 2: The single device ID in paging message can be based on IMSI / Instance Identifier for uniqueness in single/multiple readers.

- Reference:

[1] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”.

[2] TR 23.700-13,” Study on Architecture support of Ambient power-enabled Internet of Things”.

[3] TS 23.003,” Numbering, addressing and identification;”.