3GPP TSG-RAN WG2 Meeting #130	R2-2504575

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.5

Source:	Turkcell

Title:	Performance monitoring

Document for:	Discussion, Decision

# 1	Introduction

The layer 3 (L3) handover (HO) mechanism is a reactive scheme that is triggered and executed based on reported historical measurement results and events. It may work well among macro cells when the mobility of the user equipment (UE) for existing services is low. However, it could be problematic when either the UE’s mobility is high, in areas with micro cells of high density, or both, for existing services and future services, such as XR. In these cases, such a reactive scheme may lead to unintended events, such as handover failure, radio link failure, the ping-pong phenomenon, throughput loss, or premature or delayed handovers.

Conditional handover was introduced in Rel-16 to enhance handover robustness. L1/L2 triggered mobility (LTM) handover was introduced in Rel-18 to reduce the interruption time caused by frequent handover among small cells. However, these two mechanisms are inadequate because they remain reactive schemes by design. Conversely, a mechanism based on an artificial intelligence/machine learning (AI/ML) algorithm could potentially enable a proactive scheme.

L3 measurement is based on filtering L1 measurement. The study of AI/ML for air can be leveraged for mobility purposes; for example, temporal prediction can also be used to forecast beam(s)/cell(s) becoming worse, preventing unintended events like radio link failure or short-stay handover.

On the network side, new assistant information, if necessary, and statistical information based on measurement reports from UE and/or neighboring nodes can also be utilized for smart prediction. If the network could obtain certain predictive information, handover and/or RRM performance could be enhanced through proactive measures to either make better decisions or avoid unintended events.

AI/ML for mobility will focus on enhancing mobility in RRC_CONNECTED mode over the air interface by adhering to the existing mobility framework, meaning that the handover decision is always made on the network side. Mobility use cases concentrate on standalone NR PCell changes. Both UE-side and network-side AI/ML models can be considered, respectively.

Study and assess the potential benefits and gains of AI/ML-aided mobility for network-triggered L3-based handover, considering the following aspects [1]:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

  - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed

  - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

  - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we discuss further improvements for the AI/ML Mobility Study Item based on the conclusions from the 3GPP RAN2 #129bis meeting [2].

# 2	Discussion

The following items were agreed upon in the 3GPP RAN2 #129bis meeting, which could impact the performance monitoring including UE sided and NW sided model.

Agreements

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that it is generalizable for the studied cases over cell configurations with different deployment scenarios

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

- For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario than the model trained in UMa scenario when tested in UMi scenario

- No more generalization work is expected

Agreements on measurement event prediction

1.  Option 2 and Option3 outperform legacy solutions in terms of HO failure number per UE per second

2.	Capture that most companies show in their simulation results that option 3 outperforms option 2 when prediction accuracy is good enough.  There are some companies that show the opposite and indicate why.

3	For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

4	From the small number of companies that provided results for direct prediction: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

5	No further simulation results are expected for future meetings

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

- Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

Based on the RAN1#117 meeting [3], the NW-side and UE-assisted performance monitoring are selected as two options for the Beam Management (BM) use case. In our view, both options can be reused for AI/ML-aided mobility.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Two options, the NW-side and UE-assisted performance monitoring, are chosen for the BM use case. Both options can be reused for AI/ML-aided mobility.

Regarding the NW-side option, the reported measurement result can be either an L3 cell-level measurement result or an L3 beam-level measurement result. For the UE-assisted option, RAN2 needs to discuss the performance metrics calculated on the UE side, such as RSRP difference for RRM measurement prediction or F1 score for indirect measurement event prediction.

For RRM measurement and indirect event prediction, the reported measurement result for NW-side monitoring can be an L3 cell or a beam-level measurement result: actual RSRP and predicted RSRP, missed event detection, false event detection, true event prediction, true time event reporting triggered, and predicted time event reporting triggered.

For UE-assisted monitoring of RRM measurement and indirect event prediction, RAN2 requires the performance metrics calculated on the UE side, including the F1 score, RSRP difference, and the time difference between the true time event reporting triggered and the predicted time event reporting triggered.

For direct measurement event prediction, similar to indirect measurement event prediction, the UE can report the following information to the network for performance monitoring:

For direct measurement event prediction, NW-side monitoring includes missed event detection, false event detection, and true event prediction.

For direct measurement event prediction, UE-assisted monitoring can utilize the F1 score.

The prediction accuracy is evaluated based on the F1 score, which serves as a performance monitoring metric. Another consideration is whether the L3 RSRP difference can also function as a metric for measurement event prediction, as a lower L3 RSRP difference results in a higher F1 score with the indirect method.

# 3	Conclusion

In this contribution, we made the following observations in the previous section for the AI/ML mobility, which are summarized:

- Two options, the NW-side and UE-assisted performance monitoring, are chosen for the BM use case. Both options can be reused for AI/ML-aided mobility.

We propose the following based on the discussion in the previous section:

- The reported measurement result for NW-side monitoring can be an L3 cell or a beam-level measurement result: actual RSRP and predicted RSRP, missed event detection, false event detection, true event prediction, true time event reporting triggered, and predicted time event reporting triggered.

For UE-assisted monitoring, RAN2 requires the performance metrics calculated on the UE side, including the F1 score, RSRP difference, and the time difference between the true time event reporting triggered and the predicted time event reporting triggered.

NW-side monitoring includes missed event detection, false event detection, and true event prediction.

UE-assisted monitoring can utilize the F1 score.

# 4	References

RP-242393 - SID: Revised SID on AIML for mobility in NR

3GPP RAN2 #129bis Meeting: R2_129b_ChairNotes_25-04-11_16-45 final

Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0 (Fukuoka, Japan, May 20th – 24th, 2024)

3GPP TR 38.744 V0.0.4 (2024-10) Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

# 5	Annex – Simulations

TR 38.744 [4] states that RRC_CONNECTED mode encompasses RRM measurement prediction, measurement event prediction, and RLF/HOF prediction for the PCell change procedure in a standalone NR scenario. The study of the use cases is primarily driven by two objectives. The first objective is to reduce measurement efforts in the temporal, spatial, or frequency domain by utilizing predicted measurements. The second objective is to enhance handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, handover interruption).

Synthesized datasets based on channel model and deployment [4] are used for evaluation. Field data can be used optionally. In principle, once a set of simulation parameters and assumptions is settled, it should also be used for the baseline case (i.e., without AI/ML model), model training (e.g., data set generation), model validation, model testing, and inference operation [5], etc. Between the training and test datasets, different random seeds are used at least for channel modeling and UE trajectory. Simulation assumptions collected in the following table are for FR1 and FR2:

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ 1,2 or 4 TX beams are assumed. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ 8,16 or 32 TX beams are assumed |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 1RX beam is assumed | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) 4RX beams are assumed |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | 40ms | 80ms |

Predictably, as the observation window expands, more information can be utilized for prediction. When we increase the Measurement Reduction Rate in the Temporal domain (MRRT), the information used for AI model prediction decreases, along with the RSRP difference. The simulation results further prove this behavior. As the MRRT increases, the difference in RSRP also rises, consequently reducing the best cell's prediction accuracy.

The key performance indicators are evaluated below by adapting the MRRT rate according to the prediction accuracy. The computational complexity [FLOPs] is 340 K. The RSRP difference increases as the MRRT increases.

| Key Performance Indicators MRRT (Measurement Reduction Rate in the Temporal domain) | 50% | 60% | 70% | 80% |
| --- | --- | --- | --- | --- |
| Average L3 cell level RSRP difference | 0,51 dBm | 0,72 dBm | 0,94 dBm | 1,63 dBm |
| Accuracy of the predicted target cell | 98,20 % | 96,30 % | 95,90 % | 93,20 % |