3GPP TSG RAN WG2 #130	                                              R2-2504580

St Julian’s, Malta, May 19th – 23th, 2025

Source:	CEWiT

Title:	A-IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129-bis [1] meeting, the following agreements have been made on paging aspects related to A-IoT.

- Agreements - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to take this into account for their design

In this contribution, we discuss the contents, monitoring and other aspect for paging message for A-IoT.

- Discussion

2.1 Contents of A-IoT paging message

Upon receiving a request, the reader initiates communication by sending an A-IoT paging message that signals the device(s) required to respond. This paging message includes the identifiers of the targeted devices, which are crucial for ensuring that only the intended A-IoT devices are alerted and can respond accordingly. In addition to the device identifiers, the paging message can carry supplementary information to enhance the communication process. This may include the resource configurations for transmission of MSG1/MSG3, Furthermore, the message may offer guidance on generating a random number (Q) or a random device identifier, which is essential for avoiding collisions during the RACH process and establishing unique identifiers for communication.

This structured approach ensures that the A-IoT device can efficiently handle service requests while minimizing communication delays and optimizing resource usage.

Proposal 1: Support the additional following contents for paging message :

- Information related to random Number (Q) or random device ID Generation

- The total number of time domain resources and the number of the frequency domain resources for MSG1 transmission

2.2 Repeated paging request

With respect to device behavior in parallel service requests during an ongoing procedure, the following proposal was discussed in RAN2#129-bis [1] meeting, and one option needs to be selected between Option A and Option B.

- Option A: Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

- Option B: Terminate the ongoing procedure and respond to the latest request

In RAN2# 129-bis [1], the following FFS was agreed:

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

In our view, device behavior should be explicitly defined rather than left to individual implementations. This is important to avoid inconsistencies in a multi-vendor environment. Moreover, varying implementations could lead to differences in system performance. Therefore, standardizing device behavior is essential.

Proposal 2: Support to define the device behavior if it gets a new service request while one procedure is still ongoing.

In the case of parallel service request, we prefer the option A, as it allows simple device architecture and no requirement of inter reader coordination.

Observation 1: Preferred Option A i.e. ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure for the following reasons:

- No reader coordination required

- Simple device architecture

However, in Option A, it is also necessary to clearly define the scenarios in which the device is allowed to respond to new service requests during an ongoing procedure. This is because, without receiving an R2D or NACK, the device cannot ascertain whether the current procedure has failed.

The device ignores all the new service requests unless one of the following cases occurs:

- Until it’s receiving the NACK

- Not receiving a R2D message in the monitoring window

Proposal 3: Support option A i.e. device ignores all new service requests and R2D messages not associated with the ongoing procedure unless one of the following cases occurs:

- Until receiving the NACK

- Not receiving a R2D message in the monitoring window

Furthermore, in cases where the parallel service request is associated with the same ongoing procedure, the device behavior must also be clearly defined. In such scenarios, the same reader can retransmit the service request with a specific flag indicating retransmission. Accordingly, the device should respond to the service request only if it detects the retransmission flag including the above described cases otherwise, the request can be safely ignored.

Proposal 4: Support to consider following device behavior if the parallel service request is associated with same procedure:

- Respond to parallel service requests associated with same procedure if request is from same reader and contains a flag indicating the retransmission.

- Neglect parallel service request associated with same procedure unless one of the following cases occurs

- Until receiving NACK

- Not receiving a R2D message in the monitoring window

Furthermore, to enable the device to associate a service request with the correct procedure, a unique Service Request ID should be included in each service request. This helps clearly distinguish between multiple concurrent service requests. Once the service request is identified, it is also essential for the device to determine whether all subsequent R2D messages are related to the same service request. To achieve this, the Service Request ID should be included in all R2D messages corresponding to that specific request. This mechanism allows the device to verify the relevance of received messages to the ongoing procedure, even in scenarios without multi-reader support, and to safely ignore any message with a non-matching Service Request ID.

Proposal 5: Include service request ID for the device to identify the procedure associated with the service request.

Proposal 6: Include service request ID in all R2D messages corresponding to a service request, enabling the device to associate all related R2D messages with the same service request.

To avoid confusion at the device regarding the origin of a received service request, it is essential to include a Reader ID in each service request. Without this information, the device cannot determine whether the service request or corresponding R2D messages originate from the same reader or a different one. This ambiguity could lead to incorrect handling of the ongoing procedure, especially in environments with multiple readers. Including the Reader ID ensures that the device can accurately track and differentiate requests and responses based on their source.

Proposal 7: Include reader ID in service request for the device to identify the reader.

2.3 Indication of resources for Msg 1

RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access procedure. In this process, each A-IoT device randomly selects one of the time-frequency resources to transmit its A-IoT MSG1. In the RAN2 #129 [2] meeting, it is agreed that paging message can include the number of resources for MSG1. For A-IoT random access, to further select the resources for the MSG1 transmission a separate R2D trigger message is agreed after the paging message. As shown in fig. 1, a predefined time window, referred to as the RACH time window can be used for A-IoT device to select the time resources randomly from this window. This RACH window allows multiple devices to backscatter their responses. The RACH window represents the duration allocated for Msg1 transmissions, during which multiple devices can transmit at different time instances. In the RACH window different occasions for MSG1 transmission are available. The R2D trigger message can be used to indicate the start of the RACH window. Similar to RFID, the A-IoT device can use similar approach i.e. random number based on Q value approach to further chose one time resource form the RACH window for transmitting the MSG1. In this case, the A-IoT device generates a random number based on Q value, loads it to a timer, start decrementing the timer and transmits the MSG1 at the expiry of the timer. The Q value can be configured through the paging message.

Fig.1 TDMA of Msg1 Transmission

Proposal 8: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 9: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

Proposal 10: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

- Conclusion

In this contribution, we provide our views on aspects of paging message for Ambient IoT. The proposals are summarized as follows.

Proposal 1: Support the additional following contents for paging message :

- Information related to random Number (Q) or random device ID Generation

- The total number of time domain resources and the number of the frequency domain resources for MSG1 transmission

Proposal 2: Support to define the device behavior if it gets a new service request while one procedure is still ongoing.

Observation 1: Preferred Option A i.e. ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure for the following reasons:

- No reader coordination required

- Simple device architecture

Proposal 3: Support option A i.e. device ignores all new service requests and R2D messages not associated with the ongoing procedure unless one of the following cases occurs:

- Until receiving the NACK

- Not receiving a R2D message in the monitoring window

Proposal 4: Support to consider following device behavior if the parallel service request is associated with same procedure:

- Respond to parallel service requests associated with same procedure if request is from same reader and contains a flag indicating the retransmission.

- Neglect parallel service request associated with same procedure unless one of the following cases occurs

  - Until receiving NACK

  - Not receiving a R2D message in the monitoring window

Proposal 5: Include service request ID for the device to identify the procedure associated with the service request.

Proposal 6: Include service request ID in all R2D messages corresponding to a service request, enabling the device to associate all related R2D messages with the same service request.

Proposal 7: Include reader ID in service request for the device to identify the reader.

Proposal 8: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 9: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

Proposal 10: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

# References

[1]   RAN2#129-bis Chair Notes.

[2]   RAN2#129 Chair Notes.