3GPP TSG-RAN WG2 Meeting #130	R2-2504595

St.Julians, Malta, 19th May – 23th May, 2025

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

During RAN2#129bis meeting, RAN2 made some progress with the following statements:

RAN2#129bis meeting agreements:

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we will discuss the remaining issues related to the agreements

2. Discussion

Configuring multiple SMTC periodicities

In the last RAN2 meeting, RAN2 agreed to support two different SMTC periodicities with different offsets in one frequency layer for UEs in idle, inactive and connected mode. As noted in the RRC open issue, the method of configuring multiple SMTC periodicities remains FFS [1]. The following are presented as possible options.

- 1) Backwards compatible: ensures that legacy UEs can still (somewhat) operate within the cell and perform neighbour cell measurements.

  - 1a) The new SMTC periodicity is provided as an extension of SMTC4 list.

  - 1b) A new field is introduced (e.g., SMTC5) to be used by Rel-19 UEs, which support DL coverage enhancements. Network may still configure SMTC4 for legacy UEs and Rel-19 DL CE NTN UEs should ignore SMTC4 when SMTC5 is configured.

- 2) Non-backwards compatible: A new field is introduced (e.g., SMTC5) to be used by Rel-19 UEs, which support DL coverage enhancements. Network is not expected to configure legacy SMTC4. Legacy UEs may have limited operation in the cell in terms of neighbour cell measurements.

From the RAN2’s agreements so far, the network allows legacy UEs to camp on DL-CE cells at least with respect to SSB extension. Therefore, legacy SMTC4 should be configured for legacy UEs. In consideration of backward compatibility, the network can configure new SMTC periodicity as an extension of SMTC4 list (1a) or by introducing a new field (e.g., SMTC5, 1b).  However, option 1b may result in redundant signaling, as SMTC4 and SMTC5 could carry identical configurations. This is because adjacent neighbor cells are likely to be common across the UEs, regardless of UE type. Considering this, we think that option 1a is preferable to option 1b, as it maintains backward compatibility while avoiding unnecessary signaling overhead.

Proposal 1	Provide the new SMTC periodicity as an extension of the SMTC4 list.

Assistance information for UE to perform SMTC selection

During the RAN2#129bis meeting, RAN2 agreed to provide assistance information on the association between SMTC and location to help RRC idle/inactive state UE to perform SMTC selection. The specific details of the location information were left as FFS. The following two options have been suggested as possible location information:

- Serving cell’s SSB beam index

- Reference location information of neighbor cells

While some companies suggested adopting option 1 as assistance location information due to its low signalling overhead and the fact that it does not require UE location information, this option has limited applicability and raises concern regarding accuracy. Specifically, option1 cannot be applied in scenarios where the serving cell has only a single SSB beam. Since such single SSB beam configurations can occur, associating SMTC with the serving cell’s SSB beam index is not always feasible. Moreover, the correlation information between SSB beams and neighbouring cells may not be accurate depending on the UE’s location within the serving cell or the number of SSB beams in a cell. As a result, the SMTC selected by the UE based on serving cell’s SSB beam strength may not correspond to the nearest or most appropriate neighbour cells.

Proposal 2	Do not provide serving cell’s SSB beam index as assistance location information, since the smtc selection based on this association is not applicable in single SSB beam scenario and has low accuracy.

On the other hand, although option 2 requires UE location information, this is not a significant concern in the NTN system. Location-based measurement initiation of cell reselection is already supported in NTN, and the same UE location can be reused for the option 2. Furthermore, option 2 support higher accuracy, as the UE can select the SMTC based on its distance from the reference locations of neighbour cells.

Based on above, we think that option 2 offers a more general and robust solution. It is applicable regardless of the SSB beam configuration and provides accurate assistance for SMTC selection. While option 2 may incur more signalling overhead, its benefit in terms of accuracy and broad applicability make it the most reasonable form of assistance information in NTN systems.

Proposal 3	Provide assistance location information in the form of a neighboring cell reference location to UEs in RRC_IDLE/ INACTIVE mode to support the selection or prioritization of SMTCs for neighboring cell measurements.

Configuring assistance location information

Regardless of whether Option 1 and/or Option 2 is adopted as assistance location information, RAN2 should discuss how to associate location information with SMTC. Since SMTC4 can consist of a list of PCIs, it may be associated with multiple neighbour cell’s location information. For simple implementation, we suggest that the network associate the location information directly with the PCI, as the SMTC4list is configured together with the PCI.

Proposal 4	Location information is associated with each PCI in the SMTC4 list.

Maximum number of SMTCs

RAN2 has agreed that more than four SMTCs can be configured for UEs in RRC idle/inactive state. In the last meeting, there was a discussion on whether the number of SMTC should be six, considering only adjacent neighbor cells, or seven, including both the adjacent neighbor cells and the serving cell. We think that assuming all cells, including six neighboring cells and the serving cell, have different SSB periodicities and transmission timings represents an extreme worst-case scenario. Since it is unlikely in practice, we consider support for six SMTC configurations to be sufficient.

Proposal 5	The maximum number of SMTC configurations to be supported is six.

3. Conclusion

In this contribution, following statements are proposed:

Proposal 1	Provide the new SMTC periodicity as an extension of the SMTC4 list.

Proposal 2	Do not provide serving cell’s SSB beam index as assistance location information, since the smtc selection based on this association is not applicable in single SSB beam scenario and has low accuracy.

Proposal 3	Provide assistance location information in the form of a neighboring cell reference location to UEs in RRC_IDLE/ INACTIVE mode to support the selection or prioritization of SMTCs for neighboring cell measurements.

Proposal 4	Location information is associated with each PCI in the SMTC4 list.

Proposal 5	The maximum number of SMTC configurations to be supported is six.

4. References

[1] R2-25xxxxx	[Post129bis][305][R19 NR NTN] RRC CR.