3GPP TSG-RAN WG2 Meeting #130	R2-2504597

# St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	7.0.2.16

Source:	Ericsson, Qualcomm Incorporated

Title:	Discussion on DSR cancellation

Document for:	Discussion, Decision

# 1	Introduction

RAN2 introduced the Delay Status report (DSR) feature in Rel18. This reporting has a lot of similarities to the BSR. In this contribution an issue with RAN2 agreements and their implementation related to the cancellation of the DSR is discussed and potential ways to solve it proposed.

# 2	Discussion of DSR cancellation

For both BSR and DSR it has been a topic of discussion if the cancellation of the reports should be done if all the data to be reported is transmitted. In essence the question is if the report should be sent or not.

If a MAC PDU is large enough to include all PDUs/SDUs within the triggering threshold from the LCGs that have pending DSRs, the UE shall not include DSR MAC CE in the MAC PDU

For DSR there were a few agreements made. In RAN2 #124 it was decided that the DSR should not be sent if all the data to be reported can be transmitted:

It is up to UE implementation whether to cancel DSR even if the MAC PDU can accommodate all the delay-critical data but is not sufficient to include the DSR MAC CE and its subheader (i.e. similar to BSR).  Update the TP to ensure that the relaxation is only for this case.

However later in RAN2#125 the following agreement was made on the DSR cancellation:

This agreement says that in the case that all delay critical data can be included in the MAC PDU but the DSR MAC CE and its subheaders can’t be included the cancellation of the DSR should be up to UE implementation. This later agreement goes against the previous agreement. Also this later agreement states that the cancelation behaviour should be similar to BSR.

- RAN2 made multiple agreement on DSR cancellation with later agreement contradicting earlier agreement.

- Latest RAN2 agreement says that DSR cancellation behaviour should be similar to BSR.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

How this is handled for the BSR is specified by the following lines in the MAC specification (chapter 5.4.5 Buffer Status Reporting):

This implies that the BSRs may be cancelled when the UL grants can accommodate all the data to be reported but not additionally the BSR. However, the BSR does not need to be cancelled even if all the data is accommodated, i.e. the transmission can include both all the pending data and the BSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

When looking at the specification text for the DSR it states the following:

Current specification makes it clear that the DSR can’t be included (and thus will be cancelled) if all the associated data is transmitted and the only case when the DSR can be transmitted is if not all the delay critical data is included in the transmission. This differs from the behaviour of the BSR.

- Current specified cancellation behaviour between DSR and BSR differs.

The difference in the behaviour means that a DSR including zero value will never be transmitted, while an empty BSR can be sent in the BSR case. However, a zero indicating DSR may still be of value in this case for the network, as it indicates that there is no delay critical data in the buffer. Something which may be expected by the network at certain times. Sending padding bits may not be always work (as there may be more data above the threshold, i.e. that is not delay critical) and a sending only a BSR will not provide this timing information to the network. This is a reason to have aligned behaviour between DSR and BSR.

- There are reasons to have aligned behaviour between DSR and BSR.

Because of this we propose to change the normative text on DSR to align with the BSR cancellation behaviour.

Implement the TP provided in Appendix 1 to align BSR and DSR cancellation behaviour.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 made multiple agreement on DSR cancellation with later agreement contradicting earlier agreement.

Observation 2	Latest RAN2 agreement says that DSR cancellation behaviour should be similar to BSR.

Observation 3	Current specified cancellation behaviour between DSR and BSR differs.

Observation 4	There are reasons to have aligned behaviour between DSR and BSR.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Implement the TP provided in Appendix 1 to align BSR and DSR cancellation behaviour.

# Appendix 1

Changes to specification TS 38.321 (rev 18.5.0), additions highlighted in yellow:

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE.  When a MAC PDU includes all PDCP SDUs associated with all the pending DSRs the MAC PDU may contain a DSR MAC CE indicating zero buffer size.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs.