3GPP TSG RAN WG2#130	R2-2504600

St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	8.19.1

Source:	Nokia

Title:	Discussion on UE capability change

Document for: Discussion and Decision

# 1	Introduction

During IODT testing, it was observed that some UEs dynamically update their UE Radio Capability information. This involves the UE leaving the RRC_CONNECTED state without notifying the gNB and establishing a new connection for NAS registration, subsequently providing updated UE Radio Capability information to the network.

This contribution examines the problems arising from this autonomous UE behavior and proposes a way forward.

# 2	Discussion

TS 23.501 clause 5.4.4.1 describes the UE behaviour when the trigger to change the UE's UE Radio Capability happens when the UE is in CM-CONNECTED:

#### 5.4.4.1 UE radio capability information storage in the AMF […] If the UE's NG-RAN or E-UTRAN UE Radio Capability information changes while in CM-IDLE state, the UE shall perform the Registration procedure with the Registration type set to Mobility Registration Update and it also includes "UE Radio Capability Update". (For specific requirements for a UE operating in dual-registration mode see clause 5.17.2.1). When the AMF receives Mobility Registration Update Request with "UE Radio Capability Update" requested by the UE, it shall delete any UE Radio Capability information that it has stored for the UE. If the UE's NG-RAN UE Radio Capability information changes when the UE is in CM-IDLE with Suspend, NAS shall trigger AS to establish a new RRC connection and not resume the existing one in order to perform the Registration procedure with the Registration type set to Mobility Registration Update including "UE Radio Capability Update". As a result of this, the access stratum in the UE will discard the AS information and establish a new RRC connection as defined in TS 36.331 [51]. If the trigger to change the UE's NG-RAN or E-UTRAN UE Radio Capability information happens when the UE is in CM-CONNECTED state, the UE shall first enter CM-IDLE state and then perform the Registration procedure with the Registration type set to Mobility Registration Update and it also includes "UE Radio Capability Update".

When a UE autonomously transitions from RRC_CONNECTED to RRC_IDLE to dynamically update its radio capabilities, the gNB remains unaware of this RRC state change. From the gNB's perspective, the UE is still in RRC_CONNECTED. This leads to a perceived radio connection problem, as the gNB receives no responses to its downlink (DL) transmissions. Consequently, the gNB initiates unnecessary DL and UL retransmissions, increasing power consumption and lowering the modulation and coding scheme (MCS) until it detects a radio link failure (RLF). Subsequently, the gNB maintains the UE context, expecting an RRC reestablishment procedure that never occurs. Meanwhile, the UE has already established a new RRC connection for NAS registration. This results in the AMF detecting two simultaneous NGAP connections for the same UE, leading to the release of the older connection.

Although the above procedure should be considered as normal and in no way the fault of RAN, it causes degradation of RAN retainability KPIs giving operators false information about the deterioration of network performance.

Figure 1. UE Radio Capability change triggered in RRC_CONNECTED

Observation 1: When a UE autonomously transitions from RRC_CONNECTED to RRC_IDLE to update its radio capabilities, the network remains unaware of this RRC state change

Observation 2: The network continues to believe the UE is in RRC_CONNECTED, even though it has moved to RRC_IDLE. This creates a discrepancy in the network's understanding of the UE's RRC state.

Observation 3: The network continues to allocate radio resources for the UE, even though it is no longer in RRC_CONNECTED. This results in wasted resources and degradation of retainability KPIs.

To address these issues, it would be beneficial if the UE informs the gNB of its intention to leave RRC_CONNECTED due to a UE capability change. This notification would allow the network to more accurately track the UE's RRC state and avoid unnecessary resource allocation.

Proposal 1: The UE informs the gNB of its intention to leave RRC_CONNECTED due to a UE capability change

Proposal 2: Consider using UAI or new RRC message to inform the gNB UEs intention to leave RRC_CONNECTED due to a UE capability change

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: The UE informs the gNB of its intention to leave RRC_CONNECTED due to a UE capability change

Proposal 2: Consider using UAI or new RRC message to inform the gNB UEs intention to leave RRC_CONNECTED due to a UE capability change