3GPP TSG-RAN WG2 Meeting#130	R2-2504601

St Julian, Malta, May 19th-23rd, 2025

Agenda item:		8.4.4

Source:		Lenovo

Title:		 LP-WUS in RRC Connected Mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [1]. The following agreements were made at the previous meetings:

Agreements at RAN2 #127: - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. Agreements at RAN2 #127bis: - For option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LP-WUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LP-WUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. Agreements at RAN2#128 meeting: - The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. - For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. - Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE. Agreements at RAN2#129 meeting: - For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer. Agreements at RAN2#129bis meeting: - For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. - Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. - Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. - Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. - Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. - If configured, the UE can signal a preferred time offset via UAI signalling. - Ask RAN1 for further information regarding their conclusions. - FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. - For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. - For NR-DC, the LP-WUS in MCG and SCG can be configured independently. - Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

This contribution looks at the possible RRC Connected Mode procedures for an LP-WUR enabled UE including activation/ deactivation and the use of LP-WUS in Connected Mode DRX.

- Discussion

  - LP-WUS for Connected-Mode DRX

As per the R16 power saving item, in RRC Connected Mode, the MR may be configured with a DCI 2_6 (Wake-Up Signal). This DCI format 2_6 is monitored outside of the DRX active time to decide whether the MR needs to be woken up in the next drx-onDurationTimer. RAN2 should discuss how the UE may use the LP-WUS to wake up the MR such that the power saving gain achieved by using the LR must be higher than the power saving gain achieved by using only the DCI format 2_6, at the least.

- In RAN2#129bis meeting, it’s assumed that when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for option 1-1. From our side, whether the working assumption can be further confirmed depends on whether LR and MR can operate simultaneously, which relies on RAN1’s feedback. If MR and LR cannot operate simultaneously, LR is not able to monitor LP-WUS when MR is on. This will result in LR not being able to monitor LP-WUS in following cases:

- Case 1: If the LP-WUS occasion collides with C-DRX active time, UE does not monitor LP-WUS, which is agreed in RAN1#120 meeting.

- Case2: Similar to DCP, if the LP-WUS MO collides with measurement gaps, BWP switching interruption length, RAR for beam failure recovery, MR will wake up to perform legacy UE behaviours (e.g., perform measurement during measurement gap, perform downlink synchronization and channel measurement when switch to another BWP, monitor RAR) and the LR is not able to monitor LP-WUS during the MO.

- Case3: In the case of LR is in off-state during active time, it may need some time to ramp up when transits to on-state after active time. As figure 1 shows, if the duration between the end of active time and the start of the most recent MO is shorter than the required LR ramp-up time, LR cannot monitor LP-WUS during the most recent MO after active time.

- Figure 1: An example of case 3

- In such cases, to keep alignment between network and UE, whether the UE needs to start performing PDCCH monitoring during the MO-associated timer (e.g., drx_onDurationTimer in Option 1-1 etc.) needs to be specified. From our point of view, for case 2 and case 3, to avoid UE missing potential DL transmission, a simple way is that UE wakes up to perform PDCCH monitoring during the associated timer related to the MO, like legacy DCP.  In last meeting, it’s still FFS whether the similar solution can be applied to Option 1-2. From our point of view, it depends on the detail design of the new timer. Based on RAN1’s latest agreements, UE starts the new timer after a time offset configured by the network indicating a time from the first LP-WUS MO per periodicity. In this way, UE can determine the location of new timer based on the MO configuration and time offset configured by network. We prefer that similar solution can be applied to Option 1-2 that UE starts new timer if UE is not able to monitor LP-WUS occasions. Additionally, whether such behaviour is configurable by RRC can be FFS.

- Proposal 1: (MAC-2) RAN2 to confirm when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for Option 1-1, and same principle can be applied to option 1-2. FFS whether such behaviour is RRC configurable.

  - Dual DRX Case

- Serving cells of a MAC entity may be configured by RRC with two DRX groups. Each DRX group has separate DRX parameters to be applied for FR1 and FR2. Then RAN2 needs to identify whether dual DRX group can be configured along with the LP-WUS configuration. In last RAN2#129bis meeting, companies had discussion on this issue, but no consensus was reached, it is FFS on whether/how to support LP-WUS (including Option1-1 and 1-2) and dual DRX group. In legacy, the DCP is not configured with secondary DRX group simultaneously. Considering that the solutions introduced for DCP mechanism is taken as baseline for LP-WUS design for option 1-1, it is proposed that LP-WUS is not configured with secondary DRX group simultaneously at least for option1-1, to keep the solutions simple. However, since RAN1 also had discussion on this issue, RAN2 may first wait for RAN1’ s conclusions.

- Proposal 2: (MAC-1) RAN2 to wait for RAN1’s conclusion on whether to support LP-WUS and dual DRX group simultaneously.

- In the case that RAN1 supports configuration of LP-WUS with secondary DRX group simultaneously, the following options are available for configuration of LP-WUS with dual DRX:

- Opt1: Single LP-WUS Configuration:

- 1-1: LP-WUS wakes up all cell for both DRX groups

- 1-2: LP-WUS wakes up all cells for the DRX group configured with LP-WUS

- 1-3: LP-WUS indicates which DRX group is awakened, which requires LP-WUS to include addition indication.

- Opt2: LP-WUS is configured per DRX group, i.e., for FR1 and FR2 respectively:

- 2-1: LP-WUS wakes up all cells for associated DRX group.

- We think that the solution should not be overly specified to keep it simple. Since the secondary DRX group is configured with a shorter drx-onDuration and drx-Inactivity timer as compared to the default DRX group, the power saving gain achieved from configuring LP-WUS with secondary DRX group is small. Furthermore, if LP-WUS needs to be received on FR2, it may further depend on the bands supported by the LR as well as the activation/ dormancy of SCells. And to avoid additional impacts to other working groups towards the end of the release, we think that a single LP-WUS Configuration on the PCell of the default DRX group is sufficient. FFS if the LP-WUS wakes up all cells of the default DRX group only, or if the LP-WUS wakes up all cells across both DRX groups.

- Proposal 3: (MAC-1) If RAN1 confirms that LP-WUS can be configured with dual DRX group simultaneously, a single LP-WUS is configured on the PCell of the default DRX group. FFS if the LP-WUS wakes up all cells of the default DRX group only, or if the LP-WUS wakes up all cells across both DRX groups.

- Additionally, another aspect of dual DRX is how the two LP-WUS options may be configured if it is supported with dual DRX simultaneously. In general, we think that the same LP-WUS Configuration (i.e., Option 1-1 or Option 1-2) should be used across both DRX groups. That is, if Option 1-1 is configured for the default DRX group, then the secondary DRX group shall also follow Option 1-1. Similarly, if Option 1-2 is configured for the default DRX group, then the secondary DRX group shall also follow Option 1-2. With Option 1-1, this behaviour is quite straightforward and the secondary DRX group may be configured with shorter values of drx-onDuration and drx-Inactivity timers, as in legacy. With Option 1-2, the lpwus_PDCCHMonitoringTimer can either be configured with the same value in the secondary DRX group or it can be configured with a shorter timer value as compared to the default DRX group. Since Option 1-2 was designed to have a shorter latency benefit, we think it would be beneficial to have a shorter lpwus_PDCCHMonitoringTimer in the secondary DRX group for a better power saving gain. The cycle length and start of DRX in both DRX groups can be the same, similar to legacy behaviour. So we think, in principle the design of Option 1-2 with secondary DRX group (if supported) can reuse the legacy design as baseline.

- Proposal 4: (MAC-1) If RAN1 confirms that LP-WUS can be configured with dual DRX group simultaneously and both DRX groups can be woken up by a single LP-WUS configuration, the default DRX group and secondary DRX groups are both configured with the same LP-WUS DRX Option (i.e., either Option 1-1 or Option 1-2).

- Proposal 5: (MAC-1) If LP-WUS Option 1-2 can be configured with dual DRX group simultaneously and both DRX groups can be woken up by a single LP-WUS configuration, the secondary DRX group is configured with a shorter lpwus_PDCCHMonitoringTimer and the DRX Cycle start and length remain the same for both DRX groups.

  - SPS and CG impacts

- For UEs in RRC_Connected state, the resources for SPS and CG are configured by the network to UEs for potential downlink and uplink transmission. The UE may perform DL reception and UL transmission on the configured SPS and CG resources. LP-WUS impacts on the configured SPS/CG needs to be identified when UE performs LP-WUS monitoring. In previous RAN1 meetings, the issue has been discussed and corresponding conclusions have been made in RAN1, although not agreement yet.

- RAN1 Conclusions: - From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1 - From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour.

- Given the limited time for this release, it is suggested that RAN2 to discuss potential UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state based on conclusions from RAN1. Specifically, in the case of UE performs LP-WUS monitoring in RRC_CONNECTED mode, i.e., UE MR enters sleep state, at least UE behaviours on the configured CG are not impacted. We understand that same principle can also be applied to SPS resource.

- Proposal 6: In the case of LP-WUS monitoring is enabled in RRC_CONNECTED mode, UE behaviours on the configured SPS/CG resources are not impacted.

  - MR wake-up due to UL data

During the Study Item Phase, it had been agreed in RAN2 that MR needs to wake-up for PDCCH monitoring when the MR performs some UL data transmission [2].

- Agreements: UL transmission by MR also triggers PDCCH monitoring by MR.

This also means that when new UL data becomes available for transmission, it can be considered as an implicit indication to wake-up the MR, if the MR had previously been asleep. This implicit and/ or explicit indication to wake-up may result in a frequent wake-up of MR whenever new data becomes available for transmission after LP-WUS activation leading to a reduction in the power saving gain offered by configuring the LP-WUR.

- Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

Hence, some enhancements may be considered in the wake-up behavior of the MR to optimize the power saving gain even when some UL transmission needs to take place. For example, one way to avoid reduction in the power saving gain is by delaying the MR wake-up (where wake-up refers to transmission of UL data and the consequent monitoring of PDCCH) when new UL data becomes available for transmission (especially if the data that arrived is not urgent), i.e., the wake-up may be postponed until absolutely necessary.

Hence, RAN 2 should discuss how MR sleep time may be prolonged when UL data becomes available for transmission.

- Proposal 7: RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

- Conclusion

- Proposal 1: (MAC-2) RAN2 to confirm when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for Option 1-1, and same principle can be applied to option 1-2. FFS whether such behaviour is RRC configurable.

- Proposal 2: (MAC-1) RAN2 to wait for RAN1’s conclusion on whether to support LP-WUS and dual DRX group simultaneously.

- Proposal 3: (MAC-1) If RAN1 confirms that LP-WUS can be configured with dual DRX group simultaneously, a single LP-WUS is configured on the PCell of the default DRX group. FFS if the LP-WUS wakes up all cells of the default DRX group only, or if the LP-WUS wakes up all cells across both DRX groups.

- Proposal 4: (MAC-1) If RAN1 confirms that LP-WUS can be configured with dual DRX group simultaneously and both DRX groups can be woken up by a single LP-WUS configuration, the default DRX group and secondary DRX groups are both configured with the same LP-WUS DRX Option (i.e., either Option 1-1 or Option 1-2).

- Proposal 5: (MAC-1) If LP-WUS Option 1-2 can be configured with dual DRX group simultaneously and both DRX groups can be woken up by a single LP-WUS configuration, the secondary DRX group is configured with a shorter lpwus_PDCCHMonitoringTimer and the DRX Cycle start and length remain the same for both DRX groups.

- Proposal 6: In the case of LP-WUS monitoring is enabled in RRC_CONNECTED mode, UE behaviours on the configured SPS/CG resources are not impacted.

- Observation 1: Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

- Proposal 7: RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

This document discusses some procedures for a LR capable UE in the RRC Connected Mode. Following are the proposals and observations made in this document:

- References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)

[2] R2 123bis Session Notes: Report from session on Mobility Enh, Mobile IAB and LP-WUS.