3GPP TSG RAN WG2 #130                              	R2-2504606

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.3

Source: 	Sharp

Title: 	Remaining issues on OD-SIB1 Request

Document for:	Discussion and Decision

Introduction

The Rel-19 WID on enhancements to NES for NR was approved in [1], which includes the following objectives:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In addition, at RAN2#129 meeting [6], it was agreed that UEs in RRC_CONNECTED could acquire SIB1 after a radio link failure event.In this contribution, we would raise some remaining issues of OD-SIB1 request.

Discussions

Access related information

Based on R-19 NES meeting progress and current 3GPP specification [10], a UE may firstly receive and store a OD-SIB1 request configuration from the source cell. The source cell is also the cell A of the UE while the UE receives the OD-SIB1 request configuration of neighbor cells from the source cell. So, the UE would store a valid OD-SIB1 request configuration while the UE is staying in RRC_CONNECTED. Then, the UE detects a radio link failure condition is met at the UE side. Based on [10], the UE would start the timer T311 when the UE initiates a cell selection procedure and the UE would stop the timer T311 upon the UE selects a suitable cell for RRCReestablishmentRequest message transmission via a random access procedure. However, in order to support OD-SIB1 request, now the UE may select an OD-SIB1 cell while T311 is running and so additional conditions should be considered in Release-19.

Fig. 1 Signaling flow of SIB1 request configuration, SIB1 request procedure, and RRC re-establishment procedure

Firstly, after a RLF condition is considered in the UE side, the UE would start a timer T311 and initiate a cell selection procedure. Also based on [10], the running T311 would be stopped after the UE selects a suitable cell for which the UE has valid and up-to-date essential system information.

5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; 1> stop timer T311;

However, OD-SIB1 request may inevitably need to support RAN sharing scenario. In addition, the network identity list supported by the selected cell, which is also part of the essential system information, is included in its SIB1. Therefore, it may not be possible for the UE to decide the selected OD-SIB1 cell is suitable cell or not if the UE does not know the network(s) supported by the target cell. In addition, OD-SIB1 may also be supported by both public and non-public networks. If OD-SIB1 request procedure is supported by public networks/non-public networks and RAN sharing is a valid scenario to OD-SIB1 request, it is suggested that the OD-SIB1 request configuration of one OD-SIB1 cell can further include the information of the network(s) supported by the OD-SIB1 cell. By doing so, RAN2 WG can guarantte the OD-SIB1 request procedure would not increase the failure rate of RRC re-establishment procedure.

- OD-SIB1 request may support RAN sharing.

- OD-SIB1 request may be supported by public networks and non-public networks.

OD-SIB1 request configuration should include the network identities supported by OD-SIB1 cells if SIB1 request supports RAN sharing.

- Please note, Proposal 1 would also be beneficial to UEs in idle mode.

To realize proposal 1, RAN2#129bis meeting [7], RAN2 WG had discussed whther cell access related information should be included in the SIB1 request configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access. However, there was no conclusion in RAN2#129bis meeting and RAN2 WG would revisit this issue in this meeting. During the discussion, signaling overhead was a major concern to include access related information in the SIB1 request configuration.

- To facilitate the discussion, RAN2 WG is suggested to discuss existing IE “CellAccessRelatedInfo” (as shown in Fig. 2) as the baseline to dicuss the access related information in SIB1 request configuration. To a OD-SIB1 cell, serving RAN can optionally configure CellAccessRelatedInfo in the OD-SIB1 request configuration of the OD-SIB1 cell. In addition, since OD-SIB1 cell is supported by Cell A, it is common that a OD-SIB1 cell would support all or a subset networks supported Cell A.

- Fig. 2 CellAccessRelatedInfo in SIB1

- Based on the observations, Proposal 2 and Proposal 3 are proposed:

CellAccessRelatedInfo can be considered as the baseline of access related information in OD-SIB1 request configuration

The CellAccessRelatedInfo of Cell A can be applied as a reference to indicate the CellAccessRelatedInfo of OD-SIB1 cell. Details can be decided by RAN2 WG.

Cell re-selection

To control cell reselection of the UE supporting OD-SIB1, new intraFreqODSIB1-ExcludedCellList and interFreqODSIB1-ExcludedCellList are introduced. If the new excluded cell lists are provided, the UE supporting OD-SIB1 ignores the legacy excluded cell lists. However, there are different understandings on UE behaviour when the new excluded cell lists are absent. In a legacy cell, there are no OD-SIB1 related configurations, thus the UE supporting OD-SIB1 only can follow legacy excluded cell list. In a Rel-19 cell, if gNB would like to exclude the same cells for OD-SIB1 UEs as for legacy UEs, there is no need to provide the dedicated excluded cell lists. So we think if the new excluded cell lists are absent, OD-SIB1 UEs should follow the legacy excluded cell lists.

If OD-SIB1 dedicated excluded cell lists are absent, the UE supporting OD-SIB1 follows legacy excluded cell lists.

Cell type switch

In RANP#106 meeting [8], it was agreed that Cell A can switch to become a OD-SIB1 cell. We think it is nature that OD-SIB1 cell can switch to become normal cell. To avoid impacts on legacy UEs, the switching from Cell A to OD-SIB1 cell may need gNB implementation efforts, e.g. handover legacy UEs to other cells. But the similar implementation seem unnecessary for the switching from OD-SIB cell to normal cell, since legacy UEs have been barred and OD-SIB1 UEs can perform like legacy UEs in normal cell. UEs in RRC_IDLE/INACTIVE can know the switching based on MIB upon reception of Short Message, but UEs in RRC_CONNECTED does not re-aquire MIB upon reception of Short Message, then how the UEs in RRC_CONNECTED know the switching from NES cell to normal cell needs to be discussed.

RAN2 to discuss whether OD-SIB1 cell is allowed to switch to normal cell and how the UEs in RRC_CONNECTED can know the switching.

Conclusions

Based on the discussion, we have the following observations and proposals:

Firstly,  to reduce the failure rate of RRC re-establishement procedure under RAN shating scenario and factiliate the discussion of cell access related information, Proposal 1~3 are suggested:

- OD-SIB1 request may support RAN sharing.

- OD-SIB1 request may be supported by public networks and non-public networks.

- OD-SIB1 request configuration should include the network identities supported by OD-SIB1 cells if SIB1 request supports RAN sharing.

CellAccessRelatedInfo can be considered as the baseline of access related information in OD-SIB1 request configuration

The CellAccessRelatedInfo of Cell A can be applied as a reference to indicate the CellAccessRelatedInfo of OD-SIB1 cell. Details can be decided by RAN2 WG.

- Then, during cell re-selection procedure, we propose that a UE who supports OD-SIB1 should follows legacy excluded cell lists if OD-SIB1 dedicated excluded cell lists are absent.

If OD-SIB1 dedicated excluded cell lists are absent, the UE supporting OD-SIB1 follows legacy excluded cell lists.

Finally, Proposal 5 is raised to discuss how does a UE in RRC_CONNECTED be notified when a OD-SIB1 cell is switched to a normal cell.

RAN2 to discuss whether OD-SIB1 cell is allowed to switch to normal cell and how the UEs in RRC_CONNECTED can know the switching.

References

RP-242354, Revised WID: Enhancements of network energy savings for NR.

Chair’s notes RAN2#126, May 20th – May 24th, 2024

Chair’s notes RAN2#127, August 19th – August 23rd, 2024.

Chair’s notes RAN2#127bis, October 14th – October 18th, 2024.

Chair’s notes RAN2#128, November 14th – November 18th, 2024.

Chair’s notes RAN2#129, February, 17th – February 21th, 2025.

Chair’s notes RAN2#129bis, April, 7th – April 11th, 2025.

Chair’s motes RAN Plenary#106, December, 9th -12th December, 2024.

RP-243297, “Way forward on NES”

3GPP TS 38.331 V18.5.0 (2025-3-21)