3GPP TSG-RAN WG2 Meeting #130		R2-2504610

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item	: 8.6.3

Source	: Fraunhofer HHI, Fraunhofer IIS

Title	: Discussion on LTM event-triggered measurement reporting

Document for	: Discussion

# Introduction

Building on the current status of discussions in RAN2, this paper explores the remaining open issues and additional considerations related to measurement reporting for LTM.

# LTM Event Reporting Procedures

## Activation/deactivation of SP CSI-RS for LTM

During the most recent RAN2 meeting, the handling of activated semi-persistent CSI-RS resources in the context of an LTM cell switch was brought up. Although the topic was explored, no conclusion was reached, and the discussion has been left to the next meeting.

Within the legacy LTM framework, the measurement of CSI-RS resources in support of LTM procedures is already established. Clearly, SP CSI-RS resources may also serve as CSI-RS measurement resources for LTM. However, in the current legacy implementation, these SP CSI-RS resources are measured within the source cell. Consequently, once the UE executes a cell switch, it discontinues the measurement of those previously active semi-persistent CSI-RS resources.

It is technically feasible for the network to explicitly signal to the UE whether to activate or deactivate the SP CSI-RS resources after the cell switch has occurred. Alternatively, the UE could autonomously decide whether to continue or discontinue such measurements after a cell switch.

Observation 1: The NW has the capability to provide explicit signaling to instruct the UE on how to proceed with measuring activated semi-persistent CSI-RS resources following a cell switch.

Observation 2: In the absence of explicit signaling, the UE has two options: it may either continue or stop the measurement of the semi-persistent CSI-RS resources that were active prior to the cell switch.

If it is desirable for the UE to continue measuring these resources after transitioning to a new cell, then a common CSI-RS resource management strategy must be implemented across all potential target cells. This implies that all candidate cells should exchange and share their lists of SP CSI-RS resources to maintain consistency. While this approach enables seamless measurement continuity, it comes with a considerable cost, namely a significant increase in signaling overhead due to the additional coordination required between cells.

Observation 3: To allow continued measurement of activated semi-persistent CSI-RS resources after a cell switch, candidate cells must share their resource configurations. This collaborative approach increases system overhead significantly.

On the other hand, the UE may be configured to deactivate any previously active SP CSI-RS resources once the LTM cell switch is completed. This strategy reduces signaling overhead but introduces potential latency due to interruptions in the transmission and measurement process.

Observation 4: If the UE stops measurement of active semi-persistent CSI-RS resources after a cell switch, the overhead is minimized. However, this leads to increased latency caused by temporary interruption in communication.

## Report Configuration

RAN2#128 [1] agreed that a single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. This decision can be evaluated by considering scenarios where the UE is configured to measure both SSB and CSI-RS, as these reference signals serve distinct purposes. For example, SSB is primarily used for evaluating cell quality over wider beams, making it suitable for initial cell search and broad coverage assessments. CSI-RS, on the other hand, provides higher resolution measurements due to its use of narrower beams, making it more effective for precise channel state information.

Given these differences, we propose that the gNB can configure the UE to report both SSB and CSI-RS in a single measurement report, ensuring a more comprehensive network evaluation while significantly reducing overhead. Instead of requiring separate reports for different reference signals, consolidating them into a single report minimizes signaling overhead, improving overall network efficiency. This approach also optimizes resource utilization. Additionally, a unified reporting structure simplifies processing for both the UE and gNB, reducing complexity and enhancing system performance.

Proposal 1: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

According to the agreement in the previous RAN2 meeting, for event-triggered L1 LTM measurement reporting, the network controls whether beams that do not satisfy the event criteria can still be reported, following the N-beam configuration in MR MAC CE. Different scenarios and options should be considered for these N beams to ensure flexibility and efficiency in reporting.

In the existing LTM reporting framework, the following constraints apply:

- A maximum of four reference signals can be reported per cell.

- Up to four cells may be reported in a single L1 measurement report instance.

These specifications are outlined in TS 38.212 v18.2.0:

The number of reported cells and beams per cell is determined by configuration, meaning the UE always reports a fixed number of beams, four, for each configured cell. However, this approach is inefficient for several reasons.

First, some cells have stronger signals than others, making equal reporting unnecessary. Second, RAN2 has agreed that the NW controls if the beam(s) not satisfying the event can still be reported. This may necessitate reporting more than four beams per cell in certain scenarios to ensure a more comprehensive measurement.

Therefore, we propose allowing the number of beams per cell to vary. The UE should have the capability to report a different number of beams for different cells, improving flexibility and efficiency.

Proposal 2: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Similarly, this approach can also be applied to the number of reported cells (L).

Proposal 3: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

## Differential RSRP Reporting

RAN2#129 [2]  agreed that R18 differential RSRP encoding is used as baseline. For measurement metric representation, a differential format approach can be used to minimize the overall report message size. Reporting one absolute value for the largest RSRP of SSB or CSI-RS should be considered to ensure an accurate reference point for differential reporting. One absolute value for the largest RSRP of the SSB or CSI-RS measured beam and the remaining measured beams using differential L1-RSRP values are reported.

Proposal 4: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

## Prohibit timer

In RAN2#128, the following was agreed:

TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

However, there is a possibility that another beam may meet the event conditions shortly after the measurement report for the first event has been sent. If a new measurement report is triggered immediately whenever an event condition is satisfied, the overhead could increase significantly due to the high frequency of event-triggered measurement reports.

To mitigate this issue, prohibit timer should be introduced. This timer would begin once the first measurement report for a triggered event has been transmitted. By implementing this mechanism, the event-triggered measurement report mechanism can operate more efficiently by controlling the frequency of report transmissions.

Figure 1: Prohibit Timer

Proposal 5: Configure the UE with a prohibit timer that regulates the transmission of measurement reports with the following behavior:

- The prohibit timer defines the minimum duration that must pass before a new measurement report can be sent after the previous one.

- The timer starts when the first measurement report is triggered, preventing any subsequent event-triggered measurement reports from being transmitted until the timer expires.

## RSRP Measurement Reporting

The reporting range for SS-RSRP and CSI-RSRP in L3 reporting, along with the mapping of measured quantities, is defined in Table 10.1.6.1-1 of TS 38.133. Since this table is already used for the R18 LTM measurement report by CSI, it can be reused for LTM event-triggered measurement reporting.

Currently, LTM event-triggered measurements require an efficient and standardized way to report SS-RSRP and CSI-RSRP values. Reusing the existing reporting range and mapping definitions from TS 38.133 ensures alignment with current measurement frameworks, reduces implementation complexity, and maintains compatibility across different reporting layers. This approach also minimizes the need for additional specifications or modifications in reporting methodologies.

Proposal 6: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# Conclusion

Based on the discussion, we have the following observations.

Observation 1: The NW has the capability to provide explicit signaling to instruct the UE on how to proceed with measuring activated semi-persistent CSI-RS resources following a cell switch.

Observation 2: In the absence of explicit signaling, the UE has two options: it may either continue or stop the measurement of the semi-persistent CSI-RS resources that were active prior to the cell switch.

Observation 3: To allow continued measurement of activated semi-persistent CSI-RS resources post-switch, candidate cells must share their resource configurations. This collaborative approach increases system overhead significantly.

Observation 4: If the UE ceases measurement of active semi-persistent CSI-RS resources after a cell switch, the overhead is minimized. However, this leads to increased latency caused by temporary interruption in communication.

Based on the discussion, we have the following proposals.

Proposal 1: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

Proposal 2: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Proposal 3: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

Proposal 4: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

Proposal 5: Configure the UE with a prohibit timer that regulates the transmission of measurement reports with the following behavior:

- The prohibit timer defines the minimum duration that must pass before a new measurement report can be sent after the previous one.

- The timer starts when the first measurement report is triggered, preventing any subsequent event-triggered measurement reports from being transmitted until the timer expires.

Proposal 6: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# References

[1] Chair notes, 3GPP RAN2#128, Orlando, USA,18th – 22nd, 2024

[2] Chair notes, 3GPP RAN2#129, Athens, Greece,17th – 21st, 2025