3GPP TSG-RAN WG2 Meeting #130	R2-2504629

Malta, MT, May 19-23, 2025

Agenda Item:	8.8.1

Source:	THALES

Title:	Discussion on NTN MBS broadcast description in Stage 2 CR

Document for:	Discussion

# 1	Introduction

In RAN2#129bis, the stage 2 rapporteur was invited to submit a 38.300 CR to the RAN2#130 meeting including the endorsed changes for regenerative payload. RAN3 endorsed a BL CR for regenerative payload and MBS broadcast [1]. During the following post email discussion:

- [Post129bis][304][R19 NR NTN] Stage2 CR (Thales)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

Resulting to the draft running CR [2]. It was discussed how to introduce the NTN MBS broadcast feature in stage 2.

# 2	Discussion

In RAN2 draft running CR [2], MBS broadcast related parts are as follow :

7.3.1 Overview <<omitted part>> - SIBXX contains the description of the intended service area of a broadcast service in an NTN cell. <<end>>

And RAN3 BL CR [1] based description of ISA in NG-RAN signalling in [3]-§16.14.5:

16.14.5 NG-RAN signalling <<omitted part>> The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12]. <<end>>

To completely capture RAN2 agreements (see in Annex), it seems necessary to describe:

- Service area ID w.r.t. MBS broadcast service

- UE behaviour in presence of MBS broadcast service area in an NTN cell

- Others aspects (service continuity,…)

For that, multiple options are possible.

RAN2 down select one of the below options on how to capture MBS broadcast in Stage 2:

- Option 1 : NTN MBS broadcast should be described in the MBS section §16.10 as enhancements for NTN only

- Option 2 : NTN MBS broadcast should be described in a new sub-section §16.14.X in the NTN section as a new feature of NTN

- Option 3 : Keep [2] MBS broadcast descriptions for stage 2 (i.e. RAN3 endorsed BL CR and new SIB description)

# 3	Conclusion

In this contribution, the following observations and proposals was provided:

- RAN2 down select one of the below options on how to capture MBS broadcast in Stage 2:

- Option 1 : NTN MBS broadcast should be described in the MBS section §16.10 as enhancements for NTN only

- Option 2 : NTN MBS broadcast should be described in a new sub-section §16.14.X in the NTN section as a new feature of NTN

- Option 3 : Keep [2] MBS broadcast descriptions for stage 2 (i.e. RAN3 endorsed BL CR and new SIB description)

# References

R3-252523

R2-2504630

3GPP TS 38.300

# Annex : RAN2 agreements on NTN MBS broadcast

RAN2#125bis

1.	For MBS broadcast service we don’t restrict the work to any satellite constellation type

2.	We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

3.	We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

4.	The intended service area can cover the area of more than one NTN cells (or portions thereof)

5.	Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

6.	At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

RAN2#126

1.	For MBS broadcast service, both EFC and EMC are supported.

2.	RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

3.	MBS broadcast intended service area is provided via system information

4.	For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

5.	When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

RAN2#127

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

RAN2#127bis

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

RAN2#128

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

RAN2#129

1.	In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

2.	It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

3.	Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

RAN2#129bis

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.