3GPP TSG-RAN WG2 Meeting #130	R2-2504631

Malta, MT, May 19-23, 2025

Agenda Item:	8.9.1

Source:	THALES

Title:	Discussion to align IoT NTN k-Mac with IoT NTN TDD k-Mac

Document for:	Discussion

# 1	Introduction

In RAN2#129bis [1], an extension of the kmac range was agreed for IoT NTN TDD mode :

For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description) Can further discuss extending this proposal to IoT NTN and NR NTN in the next meetings

This contribution proposes to support k-Mac extension to IoT NTN.

# 2	Discussion

As explained in the discussion paper [2], the extension of the k-Mac parameter is beneficial for deployements when the number of gateway is limited in a transparent payload architecture.

It is beneficial for transparent architecture deployement with a limited number of gateway to support extension of k-Mac parameter

For IoT NTN, no limitations was found that preclude the extension of the k-mac above 512 slots (for a given SCS of 15 kHz). The Kmac range was agreed in RAN1#107-e [11]. The 9 bits was a compromise between the bit size of the SIB and the range of the considered maximum RTD to support all the transparent scenarios including the GSO. There were no further issues raised to have 276 or 1024 ms for Kmac. The Kmac is mainly used for UE to estimate the UE-gNB RTT (38.321 definition)

Furthermore, it seems coherent to align k-mac maximum value across the different NTN deployement scenarios (IoT NTN, IoT NTN and NR NTN).

K-Mac extension up to 1023 ms was agreed for IoT NTN TDD that would make an inconsistency between different NTN deployement scenarios

For IoT NTN, support k-Mac with a value range up to 1023 ms for Rel-19

# 3	Conclusion

In this contribution, the following proposals was provided:

- It is beneficial for transparent architecture deployement with a limited number of gatewey to support extension of k-Mac parameter

- K-Mac extension up to 1023 ms was agreed for IoT NTN TDD that would make an inconsistency between different NTN deployement scenarios

- For IoT NTN, support k-Mac with a value range up to 1023 ms for Rel-19

# References

R2-25XXXX

R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES, Wuhan, CN (04-2025).