3GPP TSG-RAN WG2 Meeting #130	R2-2504633

Malta, MT, May 19-23, 2025

Agenda Item:	8.17

Source:	THALES

Title:	Discussion on support of IoT-NTN TDD mode

Document for:	Discussion

# 1	Introduction

The IoT-NTN TDD mode Work Item approved in RAN#105 in September 2024 was revised in RAN#106 [1] following a study phase in RAN1. It states the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2]

The RAN1 study phase agreements can be found in the WI status report [2].

In this contribution, we the SI scheduling and other aspect discussed in RAN2#129bis.

# 2	Discussion

## 2.1	System Information Scheduling

RAN1#120 concluded that the SI message transmission in the presence of non-D subframes has to be solved by RAN2 specifications:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

The SIBs other than SIB1-NB that are required for the NB-IoT NTN system are (including those for the mobility):

- SIB2-NB : contains common radio resource configuration

- SIB5-NB : contains neighbouring cell related information for inter-frequency cell re-selection (in IDLE mode)

- SIB22-NB : contains common radio resource configuration information for paging

- SIB31-NB : contains NTN configuration information

The SIBs-NB are transmitted over NPDSCH in SI-Message within a periodically occurring SI-Window. Up to 8 SI-Message can be scheduled in SIB1-NB. The SIBs-NB transmitted in a SI-Window are indicated with the schedulingInfoList and the sib-MappingInfo. Note that SIB2-NB is always present in the first SI message listed in schedulingInfoList

The time-domain related configurations are:

- si-WindowLength (160 to 1600 ms) that indicates the SI-Window length where the SI-Message is transmitted

- si-RadioFrameOffset (0..15 radio frames) indicates the radio frame offset to calculate the SI-Window w.r.t. SFN

- si-Periodicity (640 to 40960 ms) indicated the periodicity of the SI-window in the SIB1-NB periodicity of 4096 radio frames

- si-RepetitionPattern ({everyRF,every2ndRF,every4thRF,every8thRF,every16thRF,every32ndRF,every64thRF,spare1}) indicates the repetition pattern within the SI-Window, e.g. every 2nd RF means that every 2 radio frames within the SI-Window, the SIB is transmitted. For a SI-WindowLenght of 160 ms, it implies 8 repetitions within the SI-Window of the SI-Message

- si-TB (56 to 680 bits) is used to determine the Transport Block Size of the SI-Message. It is related to the time domain : when the TBS <= 120 bits, the SI-Message is transmitted over the 2 first consecutive valid subframes SF; otherwise, the SI-Message is transmitted over the 8 consecutive valid subframes in the radio frame.

| SI configuration Parameters | value |
| --- | --- |
| si-Periodicity-r13 ENUMERATED {rf256,rf512,rf1024,rf2048,rf4096,rf8192,rf16384,spare} | 256 |
| si-RepetitionPattern-r13 ENUMERATED {everyRF,every2ndRF,every4thRF,every8thRF,every16thRF,every32ndRF,every64thRF,spare1} OPTIONAL -- Need OP | 0 |
| si-WindowLength-r13 ENUMERATED {ms160,ms320,ms640,ms960,ms1280,ms1600,spare2,spare1} | 160 |
| si-RadioFrameOffset-r13 INTEGER (1..15) OPTIONAL -- Need OP. not present = 0 | 0 |

The SI-Window starts at subframe #0 in the radio frame, from which 
(H-SFN*1024 + SFN) mod T = FLOOR(x/10) + Offset
where T is the si-Periodicity, and Offset is the si-RadioFrameOffset [5]-§5.2.3a.

After that, the NB-IoT NTN UE receives and accumulates SI message transmissions on DL-SCH over the usable NPDSCH downlink subframes until the end of the SI-window according to the si-RepetitionPatternp. The usable downlink subframes correspond to subframes not used for NPSS, NSSS, MIB-NB and SIB1-NB. Note that if SI-message was unable to decode from the accumulated SI-message transmissions by the end of the SI-Window, the reception can continue on the next SI-window occasion.

It is possible to align the beginning of the first SI-Window with a D subframe, but the others SI-Windows may not be aligned with a D subframe because si-Periodicity available configurations can not align with the 90 ms periodicity of the frame structure

Within a SI-Window, the transmission of SI w.r.t repetition pattern may not occur all the time in a D subframe and for some configurations, it may not occur at all for a given SI-Window length

The above configurations with example values are depicted below within the TDD pattern (note that any configuration of SI-WindowLength is aligned with the DL-UL-Periodicity of 90 ms):

Figure 1 - SI-Window within the TDD pattern with configurations : SI-WindowLenght=160ms; SI-Periodicity=64rf; repetitionPattern=every2ndRF

Without modifying the legacy transmission scheme, the configuration above gives 2 consecutive SI-windows, where only one SI-Message radio frame is transmitted in a usable DL (in yellow). RAN1 still discusses the maximum TBS that can be decoded by the UE for a given configuration that determines the number of available repetitions within a BCCH modification period. However in RAN2, to limit the impact on the specifications, we propose to postpone the transmission of SI message to a next valide D frame instead of introducing new periodicities adapted to the 90 ms periodicity.

The scheduling procedure of other SI messages remains unchanged

The SI-message transmission can be postponed to the next valid D frame within the SI-Window

Figure 3 - SI-Window within the TDD pattern with configurations : SI-WindowLenght=160ms; SI-Periodicity=64rf; repetitionPattern=every2ndRF for Option 1 and Option 2 postponement

An ambiguity on which SI message is postponed on the D frame is illustrated above. 4 possible SI-message are postponed to the D frame according to the si-RepetitionPattern within the SI-window. Two options are possible:

- Option 1 : If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2 : If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

Down select one of the transmission options when SI transmissions overlap:

- Option 1: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

Another issue raised by the SI-message transmission related to the TDD pattern comes from the TBS. Indeed, when the TBS <= 120 bits, 2 SFs within the SI-message radio frame are needed for transmission. TDD pattern can afford this even with 8 contiguous usable SF. But for TBS>120 bits, 8 SFs are needed. Only a maximum of 6 are available according to the DL subframe option selected.

At most 6 SF are available in D frame for a SI-message transmission, while for TBS>120 bits, SI-message are transmitted with 8 SFs

Therefore, a solution could be to : introduce a postponing mechanism for SI-message, i.e. for maximum up to 680 bits (which may contain several SIBs-NB), describe a UE behavior where the UE receiving less than 8 SF in an active DL frame waits for the next active DL frame inside the SI-Window to receive the remaining SF. If there is a collide with the SI message repetition and the remaining SF for last SI-message, the reasoning is the same as for the SI-message frame postponing :

- Option a : The remaining SI message subframe corresponding to the last transmission are dropped

- Option b : The first SI message subframe of the transmitting SI message are dropped and the subframe of the last SI message are transmitted.

The option 2 is illustrated below:

Figure 2 - Postponement of SIBs subframes principle

In the example given above, we see that the 8 SFs corresponding to a SI-message transmission could not fit in a DL subframe. Moreover, there is an offset A between the beginning of the SFN and the beginning of the transmission. It is assumed that the subframe #0 for the start of the SI-Window corresponds to the subframe#0 of the SFN#2 that is not transmitted. The UE will start to receive and accumulate at the subframe#3 instead of the subframe#1. The offset A is then implicit by the fact that subframe#1 and #2 are not a valid D subframe. It is totally feasible in the specifications since the postponement is made in a contiguous subframe excluding the non-D subframe gap. To limit the UE search space for SI-message, the SI transmissions cannot be postponed beyond the end of the corresponding SI-Window.

For the SI transmission D frame overflow case with a collision with the next SI transmission, down select one of the transmission option:

- Option a: truncate the remaining SF of the last SI transmission

- Option b: truncate the firsts SF of the new SI transmission

## Other topic

At RAN2#129bis, it was discussed if the NPDCCH period should be taken into account for the timers based on pp for supported feature in IoT NTN TDD:

- PeriodicBSR-Timer-NB-r13 (up to 128pp or infinity)

- RetxBSR-Timer-NB-r13 (up to 512pp or infinity)

- DRX onDurationTimer-r13 (up to 32pp)

- DRX drx-InactivityTimer-r13 (up to  32pp)

- DRX drx-RetransmissionTimer-r13 (up to 33p)

- DRX drx-ULRetransmissionTimer-r13 (up to 320pp)

- PUR pur-ResponseWindowTimer-r16 (up to 64pp)

- RACH ra-ResponseWindowSize-r13 (up to 10pp)

- RACH mac-ContentionResolutionTimer-r13 (up to 64pp)

- RACH mac-ContentionResolutionTimer-r15 (up to 64pp)

- MAC logicalChannelSR-ProhibitTimer-r13 (up to 2048pp)

The NPDCCH period T = Rmax * G with G (1.5,.., 128) [5] and Rmax (1..8) [4], with T >= 4. Thus, for IoT NTN TDD with 90 ms periodicity, T should be set not below 90.

RAN2#129bis

Proposal 4: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD. - QC wonders if we need to take into account that invalid subframes need to be considered. Xiaomi thinks the issue of invalid subframes is not specific of IoT NTN but NB-IoT in general and this is not considered a problem in NB-IoT. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes)

The existing value ranges of timers are also applied to IoT NTN TDD. RAN2 to clarify if a clarification is needed to take into account the impact of invalid subframes as stated above.

RAN1#120bis

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

In RAN1#120bis the NPDCCH postponing is still in discussion. If the option 2 is adopted, the periodicities will take into account the invalid subframes. Otherwise, the invalid subframes are not specific of IoT NTN but NB-IoT and were not considered as a problem.

RAN1 still discuss the NPDCCH postponing handling due to invalid subframes

No clarification needed to take into account impact of invalid subframes on NPDCCH periods

# 3	Conclusion

In this contribution, the following observations and proposals was provided:

<Other SIBs>

- It is possible to align the beginning of the first SI-Window with a D subframe, but the others SI-Windows may not be aligned with a D subframe because si-Periodicity available configurations can not align with the 90 ms periodicity of the frame structure

- Within a SI-Window, the transmission of SI w.r.t repetition pattern may not occur all the time in a D subframe and for some configurations, it may not occur at all for a given SI-Window length

- The scheduling procedure of other SI messages remains unchanged

- The SI-message transmission can be postponed to the next valid D frame within the SI-Window

- Down select one of the transmission options when SI transmissions overlap:

- Option 1: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the remaining repetitions corresponding to the first radio frame are dropped

- Option 2: If an SI transmission corresponding to a first radio frame according to si-RepetitionPattern overlaps (after postponing) with a second radio frame according to si-RepetitionPattern, the repetitions corresponding to the second radio frame are dropped and the repetitions corresponding to the first radio frame are not dropped

At most 6 SF are available in D frame for a SI-message transmission, while for TBS>120 bits, SI-message are transmitted with 8 SFs

For the SI transmission D frame overflow case with a collision with the next SI transmission, down select one of the transmission option:

- Option a: truncate the remaining SF of the last SI transmission

- Option b: truncate the firsts SF of the new SI transmission

RAN1 still discuss the NPDCCH postponing handling due to invalid subframes

No clarification needed to take into account impact of invalid subframes on NPDCCH periods

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

R1-2409440, Discussion on IoT-NTN TDD mode, Thales, RAN1#119, 2024-11, Orlando, US.

3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, v18.3.0.

3GPP TS 36.331 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”, v18.4.0.

3GPP TS 36.306 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”, v18.4.0.

3GPP TS 36-212 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”, v18.1.0.

3GPP TS 36.304 “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”, v18.3.0.

3GPP TS 24.008 “Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”, v18.7.0.

3GPP TS 36.300 “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”, v18.4.0.

R1-2200002, Report of RAN1#107-e meeting, RAN1#107-bis-e, 01-2022.