3GPP TSG-RAN WG2 Meeting #130						        			R2-2504638

St.Julians, Malta, May 19 – 23, 2025

Source: 		ZTE Corporation, Sanechips

Title:                  Further consideration on Paging procedure for A-IoT

Agenda item:   8.2.2

Document for: Discussion and Decision

# Introduction

In this contribution we take a look at the remaining open issues for paging procedure.

# Paging message contents

# Transaction ID details

RAN2 agreed that the transaction ID is generated by the reader based on the CN correlation ID. However, the exact mechanism to generate the transaction ID was left as FFS. It should be noted that the transaction ID is used to indicate the following:

- Correlation ID (i.e. service related)

- Reader ID (different readers may use different transaction IDs)

- Delta paging (paging and reaccess will be differentiated by using the transaction ID)

1 and 2 above are related to network deployment and how the CN information is processed in the network. So, these issues are not related to RAN2. For 3), all we need to specify is device behaviour for the same and different transaction ID and this is already taken care of in the MAC procedure. Based on this, we think there is no need to specify any network behaviour for how transaction ID is derived.

Proposal 1: The exact reader behaviour for how transaction ID is generated need not be specified

Then, since the transaction ID needs to cover all the above issues (correlation ID, differentiate the readers, delta paging), considering something like 2 bits for each of these, we propose that a transaction ID of 6 bits is adopted.

Proposal 2: The size of the transaction ID should be 6 bits

# MSG1 resource configuration

As described in R2-2503969, the paging message should indicate the following information for MSG1 scheduling:

- Total number of MSG1 slots (Q parameter) – RAN2 to decide the maximum value for this

- Number of AOs in time domain per MSG1 slot (X) – RAN1 agreed that this can be either 1 or 2

- Number of AOs in frequency domain per MSG1 slot (Y) – RAN1 will decide this value

So, it is proposed that the paging message signals the above values.

Proposal 3: The paging message indicates the total number of MSG1 slots (Q) and the number of AOs in time domain per MSG1 slot (X: either 1 or 2) and number of AOs in frequency domain per MSG1 slot (Y)

In RFID the total number of slots (i.e. the frame size) is 2^15. We propose that a similar value is considered also for A-IoT. This means that the reader will indicate the parameter Q and the paging frame size (i.e. the total number of MSG1 slots) is calculated as 2^Q by the device.

Proposal 4: The maximum value of Q is 15 – i.e. the total number of MSG1 slots is between 1 (i.e. 2^0) and 32768 (i.e. 2^15).

Proposal 5: RAN2 should wait for RAN1 to decide the values for number of AOs in frequency domain per MSG1 slot (Y)

# Visibility of paging ID at the reader

For the Paging procedure, the device ID information (e.g., Paging identifier) should be included in the Paging message. The device needs to compare its stored device ID information with the provided device ID information and determine whether it’s the target device and needs to respond the A-IoT operation.

When a group of devices are paged a common way for the CN/AF is to implicitly indicate a group of devices by providing information of structured device ID and the corresponding “mask/filter”.

In the case where a very large number of devices are to be inventoried (this can be a typical/special service in AIoT which is not in general IoT system), the reader may need to perform sub-grouping (mask only a subset) according to actual device density and radio resource usage. The efficiency of the slotted-Aloha algorithm suffers significantly when the system is operating in the non-linear region of the aloha procedure. In this region, the inventory times increase exponentially and hence it is absolutely essential that the reader avoids such situations. It should be noted that the actual conditions in which the algorithm operates in this region will depend on the device density and radio conditions which are not known to the CN/AF. Thus, even if the CN does some masking or sub-grouping, this may be not enough to achieve the desired results and if the only choice the reader has is to play with the group size provided by the CN, then the actual procedure over the air may suffer depending on the collision situation which may materialise during the inventory process. Moreover, the CN is responsible for the inventory control across different readers. So, it cannot be expected that the CN can always tailor the group size for each individual reader.

In Figure 1 we show the advantages of splitting large group of paged devices into sub-groups by using a mask to choose subset of devices and compare this with the case when there is no group splitting. This problem is well known also in RFID systems and hence the approach to “select” as sub-set of devices before paging the group is adopted and is popular in RFID based systems.

Figure 1: Impact of group-splitting on the inventory times

The following observations can be made from the above analysis:

Observation 1: Operating the system without group splitting (i.e. masking the paging identifier) will result in highly negative performance when the device density is high where the slotted-Aloha system operates in non-linear region

Observation 2: At low device densities, the optimal strategy is to not split the groups into sub-groups

Observation 3: The crossover point for the optimal operation/splitting depends on device density, contention levels and other radio conditions such as coverage etc

Observation 4: The CN cannot control the contention based on the device density or the actual contention levels because these are not known at the CN. Furthermore, if a wrong group size/mask is selected by the CN, then the inventory procedure at the reader will suffer significantly and may never converge

Observation 5: Existing systems such as RFID employ reader based “select” procedure which enable sub-grouping of the contending devices on the radio side

Given the above, the reader should be able to control the contention during the random-access phase by masking the device ID and selecting a sub-group of the paged devices. Thus, we propose the following:

Proposal 6: Regardless of the visibility of the paging ID at MAC, the reader should be able to apply an AS layer “mask/filter” scheme on the Paging identifier information from a CN/AF service request to select a subset of devices.

# Conclusion

The following observations and proposals are made in this contribution:

Paging message contents

Proposal 1 (Issue 1-2): The exact reader behaviour for how transaction ID is generated need not be specified

Proposal 2 (Issue 1-2): The size of the transaction ID should be 6 bits

Proposal 3 (Issue 1-4): The paging message indicates the total number of MSG1 slots (Q) and the number of AOs in time domain per MSG1 slot (X: either 1 or 2) and number of AOs in frequency domain per MSG1 slot (Y)

Proposal 4 (Issue 1-4): The maximum value of Q is 15 – i.e. the total number of MSG1 slots is between 1 (i.e. 2^0) and 2^15.

Proposal 5 (Issue 1-4): RAN2 should wait for RAN1 to decide the values for number of AOs in frequency domain per MSG1 slot (Y)

Paging ID visibility

Observation 1: Operating the system without group splitting (i.e. masking the paging identifier) will result in highly negative performance when the device density is high where the slotted-Aloha system operates in non-linear region

Observation 2: At low device densities, the optimal strategy is to not split the groups into sub-groups

Observation 3: The crossover point for the optimal operation for group splitting depends on device density, contention levels and other radio conditions such as coverage etc

Observation 4: The CN cannot control the contention based on the device density or the actual contention levels because these are not known at the CN. Furthermore, if a wrong group size/mask is selected by the CN, then the inventory procedure at the reader will suffer significantly and may never converge

Observation 5: Existing systems such as RFID employ reader based “select” procedure which enable sub-grouping of the contending devices on the radio side

Proposal 6 (Issue 1-6): The reader should be able to apply an AS layer “mask/filter” scheme on the Paging identifier information from a CN/AF service request to select a subset of devices.

- References

- 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

- 3GPP RP-250796, Revised WID: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#107, March, 2025

- TR 23.700-13 v2.0.0 (2025-02), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), January. 2025