3GPP TSG-RAN WG2 #130	R2-2504644

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.1.3

Source:	Ericsson, Nokia, Huawei, T-Mobile USA, BT Plc.

Title:	Discussion on NW-side data collection framework

Document for:	Discussion

# 1	Introduction

In RAN2#129bis it was discussed which framework to use for the radio configuration of NW-side data collection, with the following outcome:

Data collection configuration (Framework): Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

This contribution thus focuses on the framework (namely based on L1 or L3) to be used for configuring the UE with a NW-side data collection configuration.

# 2	Discussion on framework for NW-side data collection for beam management

The relevant agreements for this contribution are the following:

- From RAN2#129: Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log - From RAN2#129-bis: The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Related to the framework to be used for configuring the UE with a NW-side data collection configuration, companies started sharing views during the last RAN2 meeting. Analysing the proposals from different companies, the configuration framework can be broadly divided into two distinct approaches:

- L3 based measurement configuration for data collection

- L1 based measurement configuration for data collection.

We note that the use case for which a NW-side data collection configuration is relevant in Rel-19 is beam management, which relies on the CSI framework for the inference, performance monitoring and UE-side data collection configuration. In Rel-20, it is expected that the mobility use case requires NW-side data collection configurations.

## 2.1	L3 based measurement configuration for data collection

For the L3 based measurement configuration for NW-side data collection, the proposed solution comprises the following steps [1], [2]:

- Include L1 CSI resource configuration (e.g. CSI-ResourceConfigId) in the L3 MeasObjectNR IE; and

- Include data logging related configuration (event for logging, reporting quantity configuration) in the L3 ReportConfigNR IE, associated with a MeasId that is linked to the MeasObjectNR.

It is known that the L1 CSI measurement configuration is in the responsibility of the gNB-DU, whereas the L3 measurement configuration is in the responsibility of the gNB-CU. Hence, in order to assess the need to introduce such solutions in the specification, we have analysed the interaction between the gNB-DU and the gNB-CU in current architecture, and in particular which information is already exchanged between the gNB-DU and the gNB-CU according to the existing F1AP specification TS 38.473 [3], and what enhancements would be needed over the F1 interface to support such solution.

In the following, we highlight our concerns related to the inclusion of the L1 measurement configuration into the L3 measurement configuration for the purpose of NW-side data collection configuration:

- No need to include a L1 measurement configuration into a L3 measurement object configuration: 
First of all, we note that if the L1 data collection configuration is not based on L3 events (and is performed periodically), there is obviously no need for the UE to also be configured with a L3 measurement configuration. 
On the other hand, if the data collection configuration is based on L3 events, the UE needs to know the associated measurement object (which includes for example the filtering coefficients for the L3 measurements). However, we note that as per the agreement reached in RAN2#129, the L3 measurement events to be considered are limited to the case of “serving cell measurements becoming worse/better than a threshold”, i.e. A1/A2-type of events triggered, and the gNB-DU is already aware of the serving cell MO because that is included by the gNB-DU within the serving cell configuration (IE ServingCellConfig), as well as the UE, since the serving cell MO is included in the serving cell configuration (along with the L1 CSI measurement configuration CSI-MeasConfig). 
Hence, there is no need to associate the L1 CSI measurement configuration related to a certain serving cell with a measurement object (i.e. MeasObjectNR), because the measurement object associated to such cell is already known by the UE from the serving cell configuration. Furthermore, there is no need to associate the L1 CSI measurement configuration with a L3 reporting configuration (i.e. ReportConfigNR) because as said above the linked measurement object is already known by the UE. In addition, the measurements results related to the NW-side data collection are anyhow reported to the gNB in the UEInformationResponse message, not in the L3 measurement reports (i.e. MeasurementReport message), as in the conventional RRM. Needless to mention that the reportConfigNR includes the configuration of what to report as part of measurement report and how/when to report the measurement report.

- Change to current handling of the L1 measurement configurations at the gNB-CU: As already said, the L3 measurements are configured by the gNB-CU, whereas L1 measurements are configured by the gNB-DU. In legacy, the gNB-CU is not supposed to open and/or modify the L1 measurement configuration. In order to include the L1 configuration into the L3 measurement configuration (i.e. MeasConfig IE), the gNB-CU would be required to read the configuration sent by the gNB-DU and to incorporate it into the L3 measurement configuration. Additionally, since the L1 CSI measurement configuration may also be used for legacy BM configurations, the gNB-CU would also need to discriminate the L1 CSI measurement configuration and determine whether a received L1 measurement configuration coming from the gNB-DU contains a data collection configuration or not.

- Impact on the UE: A legacy UE performs the L1 measurements according to configurations received in the CSI-MeasConfig IE. With this approach instead, the UE would also need to consider whether the content of L3 measurement configurations contains any IEs related to L1 CSI measurements. Further, RAN1 procedures related to L1 measurements may also be impacted, because the relevant IEs may be now included outside the CSI-MeasConfig IE. 
Such impacts should be avoided, especially because as mentioned above, the UE has already all the information available to perform the necessary L3 measurements in the serving cell, so the UE already knows how to evaluate the fulfilment of the L3 events, if needed.

- Supporting of dynamic activation/deactivation of NW-side data collection configurations: Even if it is FFS whether the dynamic activation/deactivation is supported, we believe that keeping the data collection configuration into the L1 CSI measurement configuration would facilitate the introduction of such a feature, because MAC already supports the dynamic activation/deactivation of L1 measurements and related reports, whereas such a feature does not exist for the L3 measurements. We note that this feature can be beneficial to reduce the overhead of continuous reconfiguration of periodic configurations, and also to minimize the collection of redundant data, because the gNB would be able to dynamically decide based on existing measurement reports whether some data collection is needed or not.

Given the above analysis, we observe and propose the following:

Including the L1 CSI resource configuration into the L3 MeasObjectNR IE for the NW side data collection raises the following concerns:

  - Unclear benefits, given the current mechanisms for the gNB-DU and gNB-CU to configure the L1 CSI measurements, and the L3 measurements respectively, for a serving cell

  - Added complexity to the gNB-CU, to change the handling of L1 measurements configurations received from the gNB-DU

  - Added complexity to the UE, given the reception of L1 measurement configurations outside the legacy CSI-MeasConfig.

  - Added complexity if dynamic activation/deactivation of NW-side data collection configurations is supported

## 2.2	L1 based measurement configuration for data collection

An alternative solution for NW-side data collection configuration is to reuse the L1 CSI framework to configure the UE with data collection without modifying the L3 measurement configuration and without affecting current gNB-CU operations.

As already described in the previous section, in order to configure the UE to perform the L1 measurements, there is no need to include the related L1 measurement configuration into the L3 measurement configuration. Even if the network wants to configure the L3 events for triggering the L1 data collection, the UE already knows the serving cell MO to use for the L3 measurements (that is included in the serving cell configuration together with the L1 CSI measurement configuration), and the gNB-DU is already aware of the serving cell MO because, according to the existing F1AP interface, that is already exchanged between the gNB-CU and gNB-DU within the cell group configuration (see CU to DU RRC Information IE, and DU to CU RRC Information IE, as defined in TS 38.473).

Based on legacy RRC and F1AP specifications, the UE is already aware of the serving cell MO to use for L3 measurements (in case the network wants to configure the UE with L3 event based L1 data collection), as well as the gNB-DU.

The L1 CSI measurement configurations related to NW-side data collection are included within the CSI-MeasConfig. FFS which exact IE/parameter to use.

Hence, in order to support the agreement reached in RAN2#129, i.e. “Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not”, two alternatives might be considered:

- Include in the CSI measurement configuration, the L3 events that the UE would need to monitor, i.e. the A1/A2 thresholds, and the TTT.

- Upon receiving an ordinary L3 measurement report (e.g. A1/A2 event-based report), the gNB activates/deactivates or configures/releases the L1 data collection [4]. No changes are foreseen to the legacy signalling of CSI measurement configurations.

For the L3-event triggered L1 data collection, RAN2 to discuss the following alternatives:

  - The L3 event configuration (A1/A2 thresholds, and the TTT) are included in the L1 CSI measurement configurations. FFS how to ensure that the NW is transmitting the L1 CSI-RS corresponding to the logging configuration when an L3 event for L1 CSI measurement logging triggers.

  - Upon receiving an ordinary L3 measurement report, the gNB activates/deactivates the L1 data collection. No need to include L3 events (e.g. A1/A2 thresholds, and the TTT) in the CSI measurement configuration.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Including the L1 CSI resource configuration into the L3 MeasObjectNR IE for the NW side data collection raises the following concerns:

a.	Unclear benefits, given the current mechanisms for the gNB-DU and gNB-CU to configure the L1 CSI measurements, and the L3 measurements respectively, for a serving cell

b.	Added complexity to the gNB-CU, to change the handling of L1 measurements configurations received from the gNB-DU

c.	Added complexity to the UE, given the reception of L1 measurement configurations outside the legacy CSI-MeasConfig.

d.	Added complexity if dynamic activation/deactivation of NW-side data collection configurations is supported

Observation 2	Based on legacy RRC and F1AP specifications, the UE is already aware of the serving cell MO to use for L3 measurements (in case the network wants to configure the UE with L3 event based L1 data collection), as well as the gNB-DU.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The L1 CSI measurement configurations related to NW-side data collection are included within the CSI-MeasConfig. FFS which exact IE/parameter to use.

Proposal 2	For the L3-event triggered L1 data collection, RAN2 to discuss the following alternatives:

a.	The L3 event configuration (A1/A2 thresholds, and the TTT) are included in the L1 CSI measurement configurations. FFS how to ensure that the NW is transmitting the L1 CSI-RS corresponding to the logging configuration when an L3 event for L1 CSI measurement logging triggers.

b.	Upon receiving an ordinary L3 measurement report, the gNB activates/deactivates the L1 data collection. No need to include L3 events (e.g. A1/A2 thresholds, and the TTT) in the CSI measurement configuration.

# 4	References

- R2-2502414, ZTE Corporation, “Further Considerations on NW side data collection”, RAN2#129bis, Wuhan, China, 7-11 Apr, 2025.

- R2-2502126, Apple, “Remaining issues on NW-sided data collection”, RAN2#129bis, Wuhan, China, April 7-11, 2025.

- 3GPP TS 38.473 F1 application protocol (F1AP), v18.5.0

- R2-2502594, Data Collection for Training of network-side ML Models, Nokia