3GPP TSG-RAN WG2 #130	R2-2504645

St. Julians, Malta, 19 - 23 May 2025

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In RAN#106, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was updated [1].

In this contribution we look into the remaining issues of this feature.

# 2	Potential simplifications

There is only RAN2#130 and RAN2#131 left for this work item.

Considering RAN2, at RAN2#129bis, made 35 agreements using 952 words and sent RAN1 a long LS with 8 complicated questions where the reply is likely to arrive after RAN2#130 – RAN2 will obviously not be able to complete this work item in time unless we skip all non-essential parts.

Only RAN2#130 and RAN2#131 meetings remain for this work item.

At RAN2#129bis meeting, 35 agreements were made using 952 words and an LS was sent to RAN1 where reply is likely to arrive after RAN2#130.

RAN2 shall only work on the essential functions of CB-Msg3-EDT and RAN2 shall minimize work on non-essential functions for CB-Msg3-EDT.

## 2.1	eMTC

The WID does not mention eMTC or NB-IoT, but the consensus in RAN1 has been that the WID only concerns NB-IoT.

There is a high risk of not being able to conclude this WI in the short time left. There are many open issues that remains to be discussed and many of the eMTC issues have not been discussed at all. Therefore, we think that UL coverage enhancements for eMTC can be postponed to a future release.

RAN2 deprioritize work on CB-Msg3-EDT for eMTC in release 19.

## 2.2	Multi carrier NB-IoT

A simplification is to assume maximum one carrier is enough per CE level – in later releases we may extend this to multiple carriers per CE level if load on CB-Msg3-EDT is too high for one carrier.

To support multi carrier we can indicate a carrier in system info for CB-Msg3-EDT per CE level.

The CB-Msg3-EDT configuration indicates one carrier per CE level.

## 2.3	Cell specific Koffset

The CB-Msg3-EDT functionality is supposed to work for UEs in IDLE or IDLE/suspended mode and eNB cannot know what UE that is accessing, therefore all UEs must use the cell specific Koffset for Msg3 transmission on contention based resources. However, if the UE has a UE specific Koffset it would be possible to use a UE specific Koffset after Msg3 – but that would be an unnecessary complication.

The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

## 2.4	CB-RNTI simplification

Considering the CB-RNTIs agreements last meeting

For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

There is no technical gain from using multiple or changing RNTIs for Msg3 – as all UEs, that choose to transmit on the same time and frequency resource, will use the same RNTI.

If we would have an Msg3 RNTI that is different in different time and/or frequency resources – we would use more of the RNTI space for this feature.

There is a technical gain using a single RNTI, as that simplifies the specification and uses less of the RNTIs for this feature.

A single CB-RNTI used by all UEs to scramble Msg3 will minimize the number of RNTIs used for the CB-Msg3-EDT feature and simplifies the specification compared to a CB-Msg3 RNTI that is dependent on which time/frequency resources that are used for sending the CB-Msg3.

For the low load at the agreed packet loss ratio of 10%, it is reasonable to assume not too many UEs needs to be replied to in one Msg4 window, this is especially true for SA. To maximize the number of UEs that the NW can reply to in one Msg4, it is better that all UEs listen for the same RNTI – even if the Msg4 windows corresponding to different Msg3 windows overlap in time. This will maximize the NWs ability to reply to many UEs in one Msg4.

A single CB-RNTI used by all UEs to monitor for a Msg4 will maximize the NWs ability to multiplex multiple UEs in one Msg4 and thus saving DL resources for Msg4.

If the load increase such that the NW cannot reply to all Msg3 in one Msg4, the NW replies to as many as it can and the UEs with non-replied to Msg3 will need to retransmit.

The NW is aware of when this happens and can reconfigure the resources for CB-Msg3-EDT, either by decreasing the size of one Msg3 window, add more carriers, or make Msg4 window longer allowing NW to send more Msg4 messages in one Msg4 window.

The NW can handle high load on Msg4 resources by reconfigure the resources for CB-Msg3-EDT.

Therefore, one CB-RNTI is sufficient for both Msg3 scrambling and Msg4 monitoring as it minimizes number of RNTIs used, it maximizes the NWs chance of addressing multiple UEs in one Msg4, and it minimizes the specification effort. The simplest solution is if CB-RNTI is configured together with the other parameters in SIB.

One single CB-RNTI, configured in SIB, is used by all UEs for scrambling Msg3 and for monitoring for Msg4.

## 2.5	Power ramping

If the UE fails one cycle of Msg3 replica transmissions (no matching UE CR ID MAC CE is received), if multiple CE levels are configured and/or multiple tries are configured for one CE level– a UE gets another try of Msg3 transmissions and may then succeed.

Thus, as the UE sends many multiples of transmissions (replicas and whole Msg3 cycles), there is a chance that one of them go through, even with a low transmission power. Therefore, we can simplify for now and not support power ramping, and it can be added in later releases if deemed necessary.

Power ramping for CB-Msg3-EDT is not considered in release 19.

## 2.6	OCC

RAN1 has not finalized all details of OCC and there are issues that degrade OCC performance on a contention based channel (collisions may not be resolvable if the UEs are received with different power and/or different time offset and/or different frequency offsets, multi tone transmissions may not be supported).

Considering this RAN2 agreement

If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

Based on this, if supported at all, it is obvious that OCC can be added later as a backwards compatible configuration without affecting CB-Msg3-EDT not based on OCCs, therefore there is no reason to include OCC for CB-Msg3-EDT now.

RAN2 do work on OCC for CB-Msg3-EDT in release 19.

## 2.7	Modelling DSA and SA

There is no need to have special specification text for SA, when DSA is supported – SA will be DSA with a special parameter setting! This is achieved by having separate configuration of the (N)PUSCH resources and the DSA window.

For example, (N)PUSCH resource allocation has four time-domain resources times six frequency domain resources and these reoccur every five time-domain resource. For a DSA window of 2 time-domain resource and SA window of 1 time-domain resource we have illustrated the first four DSA windows and the first eight SA windows in the figure below.

When DSA is supported, SA is a special setting of the DSA parameters.

In the specification, no special provisions are captured for slotted aloha.

# 3	Msg3 considerations

RAN2 briefly discussed the periodicities of resources for CB-Msg3-EDT:

For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

We see no use for a feature where there is very long between opportunities for the UEs to send CB-Msg3.

If the load on the CB-Msg3-EDT resources is so low that an interval in the order of the H-SFN can be used, then there is no need to improve UL capacity – and regular EDT via the random access procedure can utilize the resources more efficiently without the delay to wait for the CB-Msg3 opportunity.

The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

# 4	Msg4 considerations

## 4.1	NPDCCH format for Msg4

It is reasonable to assume that the Msg4 must be scheduled in the same way as a RAR is scheduled, that is only using NPDCCH format 1 (using the two NCCEs for one DCI, for better coverage and easier NPDCCH decoding for the UEs):

The scheduling of Msg4 uses NPDCCH format 1.

## 4.2	Design of Msg4

In Msg4, the UE can repeat part of the RRC message in a UE Contention Resolution Identity MAC Control Element used for the contention resolution just as for regular random access (from LTE MAC):

6.1.3.4		UE Contention Resolution Identity MAC Control Element

The UE Contention Resolution Identity MAC control element is identified by MAC PDU subheader with LCID as specified in table 6.2.1-1. This control element has a fixed 48-bit size and consists of a single field defined as follows (figure 6.1.3.4-1)

-    UE Contention Resolution Identity: If this MAC control element is included in response to an uplink CCCH transmission, then this field contains the uplink CCCH SDU if the uplink CCCH SDU is 48 bits long. If the CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the uplink CCCH SDU. If this MAC control element is included in response to an uplink DCCH transmission (i.e. the MAC entity is configured with rach-Skip or rach-SkipSCG), then the MAC entity shall ignore the contents of this field.

Figure 6.1.3.4-1: UE Contention Resolution Identity MAC control element

The Msg4 contention resolution reuses legacy UE Contention Resolution Identity MAC Control Element.

## 4.3	Start of PDCCH monitoring for receiving Msg4

We discussed the start of Msg4 monitoring last meeting:

The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

To allow the Msg4 monitoring to overlap potential Msg3 transmission opportunities, in the same Msg3 transmission window, will lead to an uneven load on the Msg3 resources and the eNB will have an ambiguity on when it can reply to a correctly decoded Msg3.

Msg4 monitoring that overlap potential Msg3 transmission opportunities, in the same Msg3 transmission window, will lead to an uneven load on the Msg3 resources and the eNB will have an ambiguity on when it can reply to a correctly decoded Msg3.

A UE individual Msg4 window, after each UEs Msg3 replica transmission or after a few Msg3 opportunities, has less opportunities for the eNB to answer multiple Msg3 in one Msg4 and will prolong the Msg3 window in time for allowing the monitoring for Msg4.

Having only one Msg4 window that start after the end of the Msg3 window ensures plenty of Msg3s to answer in one Msg4 and gives an even load on the Msg3 opportunities.

There is one Msg4 window per Msg3 window and the Msg4 window start immediately after the Msg3 window, taking UE-eNB RTT and eNB processing into account, that is, there is never any overlap between the Msg3 window and the corresponding Msg4 window.

For legacy, PDCCH monitoring after an UL transmission is captured in 16.6 of 36.213 (see A.3.3	NPDCCH resources). The simplest is to reuse this, that is

If a Msg3 window end in uplink subframe n, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3. The UE shall start NPDCCH monitoring in the first NPDCCH subframe, after uplink subframe n+Kmac+3, that do not overlap uplink subframe n+Kmac+3. FFS eMTC timing.

## 4.4	Inclusion of C-RNTIs in Msg4

Considering successful termination of the CB-Msg3-EDT procedure, last meeting we agreed

The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

Simplest is if the existing C-RNTI MAC CE can be included in the Msg4:

The CB-Msg4 can contain one C-RNTI MAC CE per UE that is replied to.

## 4.5	Fallback options for CB-msg3-EDT

We also briefly discussed failed/successful CB-Msg3-EDT and fallback:

Proposal 5: A CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. FFS the CB-Msg4 structure.

-	Samsung thinks we need an RRC message

Come back in the next meeting

Proposal 13: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA.

-	Ericsson thinks this should be formulated in a way that failure of the procedure is indicated to upper layers. MTK agrees. ZTE wonders which UL

Continue in the next meeting

We agree some kind of fallback is needed, but we think we shall not intertwine the CB-Msg3-EDT procedures with the regular EDT – instead we can determine when the CB-Msg3-EDT procedure has failed, and UE implementation can then select appropriate actions (which may be to trigger regular EDT, or PUR or retry another CB-Msg3-EDT or inform NAS layer of the unsuccessful EDT).

Specification effort will be higher if fallback from CB-Msg3-EDT is intertwined with random access or PUR.

If the UE receives a Msg4 with matching UE CR ID MAC CE, with or without an RRC message, the UE shall consider the contention resolution of CB-Msg3-EDT procedure successfully completed. Then, if Msg4 also contains a C-RNTI MAC CE for that UE, then the UE shall monitor PDCCH for further scheduling.

If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, with an [RRCEarlyDataComplete or RRCConnectionRelease], the UE shall consider the CB-Msg3-EDT procedure successfully completed.

If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, without any RRC message the UE shall consider the CB-Msg3-EDT procedure successfully completed and behave as if an [RRCEarlyDataComplete or RRCConnectionRelease] had been received.

If the UE gets a C-RNTI and a matching UE CR ID MAC CE, then contention resolution of CB-Msg3-EDT was successful – but the network want to send further messages to the UE.

If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, and a C-RNTI MAC CE (with or without an RRC message) the UE shall consider the contention resolution of the CB-Msg3-EDT successfully completed and start the TAT (possibly implementing an received TAC first) the  and start monitoring PDCCH. FFS how long time the UE shall monitor PDCCH. FFS when/if the MAC indicate (un)successful CB-Msg3-EDT procedure to higher layers in case a C-RNTI is received.

## 4.6	Contention control

Msg3 transmission may fail due to collision or bad reception quality at the network side.

In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached MsgA-TransMax, provided that preambleTransMax is greater than MsgA-TransMax.

A similar mechanism can be introduced for CB-Msg3-EDT to handle Msg3 transmission failures.

When one CB-Msg3-EDT transmission attempt (including the replica transmissions of the one attempt) is unsuccessful (no Msg4 with matching UE CR ID MAC CE was received within the Msg4 window), the UE shall start another round of attempts for Msg3 transmission (after waiting for a random back-off) only if all the replicas in the current round have been transmitted and none of them is acknowledged by the NW.

If no successful contention resolution was received in the Msg4 window after a Msg3 window where a Msg3 was transmitted, the UE shall draw a random number from a back off indicator (indicated in a Msg4, if received, or from a back off indicator in system information) and wait the number of Msg3 windows and in the Msg3 window after the wait, attempt the CB-Msg3 replica selection and transmission of all replicas. FFS if/how power ramping is incorporated. FFS if counters for maximum number of attempts are needed.

We need a way to determine when to give up further Msg3 transmissions, that is the unsuccessful CB-Msg3-EDT case. This is if the UE does not receive a Msg4 with a matching UE CR ID MAC CE for any of a configured number of reattempts of Msg3 transmission and possible stepping of CE level and/or power ramping.

If the UE does not receive a Msg4, with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, within the Msg4 window after the UEs last CB-Msg3 uplink transmission (including any CB-Msg3 reattempt and possible stepping of CE level), the UE shall consider the CB-Msg3-EDT procedure as unsuccessful.

If MAC layer has determined that the CB-Msg3-EDT procedure is either successful or unsuccessful, the MAC layer shall inform the higher layers. FFS how/when RRC layers determines (un)successful CB-Msg3-EDT procedure.

If CB-Msg3-EDT procedure is unsuccessful, it is up to UE implementation whether to trigger further actions (for example, trigger EDT using the random access procedure or trigger PUR transmission or trigger another CB-Msg3-EDT procedure or inform NAS of unsuccessful EDT).

## 4.7	Timing Advance in Msg4

Further we agreed

Proposal 9: The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

Agreed

We think simplest is to reuse the same Timing Advance Command that is present in RAR.

The NW may include one Timing Advance Command field in Msg4 per UE replied to, the TAC field reuses the eleven-bit TAC field from RAR.

## 4.8	Support for MT-EDT and HARQ ACK of Msg4

Regarding MT-EDT, we agreed

RAN2 also intends to support CB-msg3-EDT for MT cases

Further for HARQ we agreed

Proposal 7: HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how

the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Agreed

The main characteristic of MT-EDT is that the msg4 (in random access) can contain downlink data for the UE, and the termination of the UP-EDT procedure has this: “The procedure ends with the reception of the HARQ feedback (ARQ) acknowledging the successful DL transmission.”.

To support MT-EDT in the CB-Msg3-EDT with as small changes as possible compared to legacy, we need 1) msg4 can contain MAC SDUs, and 2) each msg4 can be acknowledged by HARQ ACK.

The Msg4 can contain MAC SDUs carrying downlink data for each UE that is replied to in the Msg4.

For each UE answered in a Msg4, the successful reception of a Msg4 can be individually acknowledged with a HARQ ACK transmission.

For 3.75 kHz SCS, if there is no need for repetitions, the legacy HARQ ACK allocation can support up to sixteen UEs simultaneously (eight subcarriers and two different k0, as the NPUSCH Format 2 is 8 ms, see section A.3.1	NPUSCH resources below) but if repetitions are needed only maximum eight different UEs can be supported with HARQ ACK resources simultaneously.

For 15 kHz SCS, if there is no need for repetitions, the legacy HARQ ACK allocation can support up to twelve UEs simultaneously (four subcarriers and three different k0 that are non-overlapping, as the NPUSCH Format 2 is 2 ms, see section A.3.1	NPUSCH resources below) but if more than two repetitions are needed only maximum four different UEs can be supported simultaneously.

For 3.75 kHz SCS, the legacy HARQ ACK resource allocation can support up to eight UEs simultaneously when repetitions are needed.

For 15 kHz SCS, the legacy HARQ ACK resource allocation can support up to four UEs simultaneously when more than two repetitions are needed.

The multiplexing of many UEs in one Msg4 will increase the needed TBS, and thus risk an increase in the number of subframes and/or the number of repetitions needed.

Considering the multiplexing of many UEs in Msg4 and the drawback of large TBS and the legacy allocation of HARQ ACK resources, we think a reasonable trade-off is to multiplex up to four UEs in one Msg4. However, this may depend on many factors and the exact number can be left to the eNB implementation.

The number of Msg3 replies in one Msg4 can be left to eNB implementation.

In random access, Msg4 (and other scheduled data on NPDSCH) is acknowledged on resources that are indicated using 4 bits in the DCI that schedules the msg4.

To enable each UE to acknowledge the successful reception of a Msg4 we need to allocate one HARQ ACK resources per UE that is addressed in one Msg4. Simplest and least change compared to legacy is to reuse the same HARQ ACK/NACK resource allocation as in the DCI.

For each UE that is replied to, Msg4 has a 4 bit HARQ ACK resource allocation, reusing the existing HARQ ACK/NACK allocation signalling in the DCI. FFS eMTC

This means for 3.75 kHz SCS, the ACK/NACKs can only use the same eight subcarriers (38 to 45) and the two k0 (13 and 21) as legacy. For 15 kHz SCS, the ACK/NACKs can only use the same four subcarriers (0 to 3) and the four k0 (13, 15, 17 and 18) as legacy.

To enable to not send HARQ feedback, we note the method for dynamic HARQ feedback control, from 36.212 section 6.4.3.2:

-	HARQ-ACK resource – 4 bits as defined in clause 16.4.2 of [3]. If downlinkHARQ-FeedbackDisabledDCI-NB is configured, or if downlinkHARQ-FeedbackDisabledBitmap-NB and downlinkHARQ-FeedbackDisabledDCI-NB are configured, and the value is ‘15’, it functions as a HARQ feedback disabled indicator.

In the HARQ-ACK resources for Msg4, the value ‘15’ indicate HARQ feedback disabled. FFS eMTC

# 7	Conclusion

In the previous sections we made the following observations:

Observation 1	Only RAN2#130 and RAN2#131 meetings remain for this work item.

Observation 2	At RAN2#129bis meeting, 35 agreements were made using 952 words and an LS was sent to RAN1 where reply is likely to arrive after RAN2#130.

Observation 3	A single CB-RNTI used by all UEs to scramble Msg3 will minimize the number of RNTIs used for the CB-Msg3-EDT feature and simplifies the specification compared to a CB-Msg3 RNTI that is dependent on which time/frequency resources that are used for sendingthe CB-Msg3.

Observation 4	A single CB-RNTI used by all UEs to monitor for a Msg4 will maximize the NWs ability to multiplex multiple UEs in one Msg4 and thus saving DL resources for Msg4.

Observation 5	The NW can handle high load on Msg4 resources by reconfigure the resources for CB-Msg3-EDT.

Observation 6	When DSA is supported, SA is a special setting of the DSA parameters.

Observation 7	Msg4 monitoring that overlap potential Msg3 transmission opportunities, in the same Msg3 transmission window, will lead to an uneven load on the Msg3 resources and the eNB will have an ambiguity on when it can reply to a correctly decoded Msg3.

Observation 8	A UE individual Msg4 window, after each UEs Msg3 replica transmission or after a few Msg3 opportunities, has less opportunities for the eNB to answer multiple Msg3 in one Msg4 and will prolong the Msg3 window in time for allowing the monitoring for Msg4.

Observation 9	Having only one Msg4 window that start after the end of the Msg3 window ensures plenty of Msg3s to answer in one Msg4 and gives an even load on the Msg3 opportunities.

Observation 10	Specification effort will be higher if fallback from CB-Msg3-EDT is intertwined with random access or PUR.

Observation 11	For 3.75 kHz SCS, the legacy HARQ ACK resource allocation can support up to eight UEs simultaneously when repetitions are needed.

Observation 12	For 15 kHz SCS, the legacy HARQ ACK resource allocation can support up to four UEs simultaneously when more than two repetitions are needed.

Observation 13	In the 5G QoS model, there is an acceptable packet delay budget that 98% of the packets need to fulfill and a there is an acceptable packet loss rate.

Observation 14	There is a standardized 5QI 10 that targets NTNs. It has an RAN PDB of 1080 ms and a PER of 10^-6.

Observation 15	For 5QI 10, each transmission can have a failure rate of up to 14% without violating the QOS requirements in GEO scenarios.

Observation 16	Restrictions that limit the UEs ability to freely select transmission opportunities within a DSA window will increase the probability of collision.

Observation 17	When configuring resources for the DSA window, it is beneficial to allocate more resources in the time domain and less in frequency domain than vice versa.

Observation 18	For 3.75 kHz SCS, HARQ ACK/NACK transmission without repetitions use 8 ms on one subcarrier.

Observation 19	For 15 kHz SCS, HARQ ACK/NACK transmission without repetitions use 2 ms on one subcarrier.

Observation 20	For 3.75 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 32 ms on one subcarrier.

Observation 21	For 15 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 8 ms for one subcarrier, 4 ms for three subcarriers, 2 ms for six subcarriers and 1 ms for twelve subcarriers.

Observation 22	Increasing number of RUs can increase the UL transport block size.

Observation 23	The number of subframes for one NPUSCH transmission increase if more RUs or more repetitions are used.

Observation 24	Some of the contiguous subframes cannot be used for NPUSCH transmissions.

Observation 25	The NB-IoT UEs cannot transmit NPUSCH continuously for more than 256 ms before inserting a 40 ms silent period of postponed NPUSCH transmission subframes.

Observation 26	The NPDSCH transmissions uses 15 kHz SCS and 12 subcarriers.

Observation 27	Not all DL subframes are available for NPDSCH transmission, for example subframes with NPBCH, NPSS, NSSS, SIB1-NB and configured gaps are excluded.

Observation 28	Increasing number of subframes can increase the DL transport block size.

Observation 29	The number of subframes for one NPDSCH transmission increase if more subframes or more repetitions are used.

Observation 30	Some of the contiguous subframes cannot be used for NPDSCH transmissions.

Observation 31	The resource for HARQ ACK/NACK is indicated with 4 bits in the DCI.

Observation 32	For 3.75 kHz SCS, HARQ ACK resources can be on one of 8 different subcarriers and use one of 2 different scheduling delays.

Observation 33	For 15 kHz SCS, HARQ ACK resources can be on one of 4 different subcarriers and use one of 4 different scheduling delays.

Observation 34	There is a gap of at least 12 subframes between the the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK.

Observation 35	NPDCCH uses 6 or 12 subcarriers in one subframe (without repetitions) can carry two respectively one DCI.

Observation 36	There is a guard period of at least 3 subframes after an NPUSCH transmission (Format 1 or 2) and first possible NPDCCH reception.

Observation 37	There is a guard period of at least 12 subframes after an NPDSCH reception and the first possible NPDCCH reception.

Observation 38	Some of the contiguous subframes cannot be used for NPDCCH transmissions.

Observation 39	The UEs likely must decode every Msg4 even if each Msg3 use a different RNTIs because UE do not know which RNTI the eNB will reply to, when multiple RNTIs are used.

Observation 40	One Msg4 window can allow for multiple Msg4 transmissions consecutive in time.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 shall only work on the essential functions of CB-Msg3-EDT and RAN2 shall minimize work on non-essential functions for CB-Msg3-EDT.

Proposal 2	RAN2 deprioritize work on CB-Msg3-EDT for eMTC in release 19.

Proposal 3	The CB-Msg3-EDT configuration indicates one carrier per CE level.

Proposal 4	The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

Proposal 5	One single CB-RNTI, configured in SIB, is used by all UEs for scrambling Msg3 and for monitoring for Msg4.

Proposal 6	Power ramping for CB-Msg3-EDT is not considered in release 19.

Proposal 7	RAN2 do work on OCC for CB-Msg3-EDT in release 19.

Proposal 8	In the specification, no special provisions are captured for slotted aloha.

Proposal 9	The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

Proposal 10	The scheduling of Msg4 uses NPDCCH format 1.

Proposal 11	The Msg4 contention resolution reuses legacy UE Contention Resolution Identity MAC Control Element.

Proposal 12	There is one Msg4 window per Msg3 window and the Msg4 window start immediately after the Msg3 window, taking UE-eNB RTT and eNB processing into account, that is, there is never any overlap between the Msg3 window and the corresponding Msg4 window.

Proposal 13	If a Msg3 window end in uplink subframe n, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3. The UE shall start NPDCCH monitoring in the first NPDCCH subframe, after uplink subframe n+Kmac+3, that do not overlap uplink subframe n+Kmac+3. FFS eMTC timing.

Proposal 14	The CB-Msg4 can contain one C-RNTI MAC CE per UE that is replied to.

Proposal 15	If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, with an [RRCEarlyDataComplete or RRCConnectionRelease], the UE shall consider the CB-Msg3-EDT procedure successfully completed.

Proposal 16	If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, without any RRC message the UE shall consider the CB-Msg3-EDT procedure successfully completed and behave as if an [RRCEarlyDataComplete or RRCConnectionRelease] had been received.

Proposal 17	If the UE receives a Msg4 with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, and a C-RNTI MAC CE (with or without an RRC message) the UE shall consider the contention resolution of the CB-Msg3-EDT successfully completed and start the TAT (possibly implementing an received TAC first) the  and start monitoring PDCCH. FFS how long time the UE shall monitor PDCCH. FFS when/if the MAC indicate (un)successful CB-Msg3-EDT procedure to higher layers in case a C-RNTI is received.

Proposal 18	If no successful contention resolution was received in the Msg4 window after a Msg3 window where a Msg3 was transmitted, the UE shall draw a random number from a back off indicator (indicated in a Msg4, if received, or from a back off indicator in system information) and wait the number of Msg3 windows and in the Msg3 window after the wait, attempt the CB-Msg3 replica selection and transmission of all replicas. FFS if/how power ramping is incorporated. FFS if counters for maximum number of attempts are needed.

Proposal 19	If the UE does not receive a Msg4, with a UE contention resolution ID MAC CE that match the Msg3 that the UE sent, within the Msg4 window after the UEs last CB-Msg3 uplink transmission (including any CB-Msg3 reattempt and possible stepping of CE level), the UE shall consider the CB-Msg3-EDT procedure as unsuccessful.

Proposal 20	If MAC layer has determined that the CB-Msg3-EDT procedure is either successful or unsuccessful, the MAC layer shall inform the higher layers. FFS how/when RRC layers determines (un)successful CB-Msg3-EDT procedure.

Proposal 21	If CB-Msg3-EDT procedure is unsuccessful, it is up to UE implementation whether to trigger further actions (for example, trigger EDT using the random access procedure or trigger PUR transmission or trigger another CB-Msg3-EDT procedure or inform NAS of unsuccessful EDT).

Proposal 22	The NW may include one Timing Advance Command field in Msg4 per UE replied to, the TAC field reuses the eleven-bit TAC field from RAR.

Proposal 23	The Msg4 can contain MAC SDUs carrying downlink data for each UE that is replied to in the Msg4.

Proposal 24	For each UE answered in a Msg4, the successful reception of a Msg4 can be individually acknowledged with a HARQ ACK transmission.

Proposal 25	The number of Msg3 replies in one Msg4 can be left to eNB implementation.

Proposal 26	For each UE that is replied to, Msg4 has a 4 bit HARQ ACK resource allocation, reusing the existing HARQ ACK/NACK allocation signalling in the DCI. FFS eMTC

Proposal 27	In the HARQ-ACK resources for Msg4, the value ‘15’ indicate HARQ feedback disabled. FFS eMTC

# 8	References

RP-243278, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc., RAN#106, Dec. 2024

# Appendix A	Background

Here we list some important background information of issues discussed.

## A.1	The 5G QoS model

The 5G QoS model is described in 23.501 section 5.7.

“Standardized or pre-configured 5G QoS characteristics, are indicated through the 5QI value, and are not signalled on any interface, unless certain 5G QoS characteristics are modified as specified in clauses 5.7.3.3, 5.7.3.4, 5.7.3.6, and 5.7.3.7.”

Some of the most important characteristics are the Packet Delay Budget (PDB) and the Packet Error Rate (PER).

Among the standardized 5QIs there is one for NTNs.

### A.1.1	Standardized 5QI for NTNs

The 5QI 10 is for NTN, and the standardized parameters is copied below:

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Non-GBR (NOTE 1) | 90 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 10: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms,~21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |  |

In NOTE 1 above, it is clear delayed packets shall not be included in the PER, further for PDB we have (23.501):

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER.

In the table we see that, for 5QI 10, the PDB is 1100 ms which of 1080 ms is for RAN and 20 ms for CN. Further, an acceptable PER is 10^-6.

This means from an QoS point of view, it is acceptable if 2% or less of the packets experience a delay above 1080 ms in RAN.

In GEO, a delay of 1080 ms is one first transmission and two retransmissions if the UE has UL resources every TTI for a first transmission and there is no scheduling delay, or one first transmission and one retransmission if the UE need to send SR and wait for a grant before the first transmission.

Even if an operator have other QoS requirements than the 5QI 10, there must be an acceptable PDB and acceptable number of packet arriving late, and an acceptable level of packets lost.

In the 5G QoS model, there is an acceptable packet delay budget that 98% of the packets need to fulfill and a there is an acceptable packet loss rate.

There is a standardized 5QI 10 that targets NTNs. It has an RAN PDB of 1080 ms and a PER of 10^-6.

If we assume that the PDB allow for two attempts for each packet, then we must have a fail rate per transmission at 10^-3 or better to achieve a PER of 10^-6 or better if all packets with longer delay are lost. However, we can accept a fail rate per transmission that is higher than 10^-3 if less than 2% of the packets are delayed longer than the PDB (that is, those packets are only delayed and not lost).

Assuming a packet failure for each transmission we can calculate the probability of success after 1 transmission, and probability of success after 2 transmissions and probability that more than two transmissions are needed for success, see table below.

|  | Pn, probability of success after n transmissions |  | P[n>2], probability of more than two transmissions needed |
| --- | --- | --- | --- |
| Probability of packet failure | P1 (n = 1) | P2 (n = 2) | P[n > 2] = 1 – P1 – P2 |
| 0.001 | 0.999 | 0.000999 | 0.000001 |
| 0.01 | 0.99 | 0.0099 | 0.0001 |
| 0.03 | 0.97 | 0.0291 | 0.0009 |
| 0.05 | 0.95 | 0.0475 | 0.0025 |
| 0.1 | 0.9 | 0.09 | 0.01 |
| 0.14 | 0.86 | 0.1204 | 0.0196 |
| 0.1414 | 0.8586 | 0.12140604 | 0.019994 |

If, for example, probability of failure for each transmission attempt is 3%, then the probability that more than two attempts are needed is 9 in ten thousand (0.0009) packet attempts. And for a packet failure probability of 14% the probability to need more than 2 transmission is 1.96%.

Thus, the 5G QoS model (at least 98% of packets within the PDB) – when the PDB allows for two transmissions – we can have a probability of failure of each individual transmission attempt of more than 14%.

For 5QI 10, each transmission can have a failure rate of up to 14% without violating the QOS requirements in GEO scenarios.

## A.2	Restriction on selectable DSA resources

There may be reasons for the UEs not being able to freely select transmission opportunities inside a DSA window.

For example:

- UEs not able to transmit on more than one subcarrier (more than one RU) in one TTI

  - Due to UE filter and emission mask issues (more expensive power amplifier), or UE peak power requirements

- UEs not able to transmit on different subcarriers in consecutive TTIs

  - UE may need some time to retune the oscillator

- Selecting two consecutive TTIs would violate the requirement on 40 ms silent period every 256 ms of transmission

Any such limits will spread out the load evenly on the transmission opportunities and will therefore increase the collision probability.

As a simple example, consider a DSA window with four transmission opportunities where each UE sends two replicas, and compare two cases:

A) there are two subcarriers with two consecutive TTIs. UEs needs to randomly select one of the subcarriers in the first TTI and one of the subcarriers in the second TTI.

B) there is one subcarrier with four consecutive TTIs. The UEs can freely select two of the four TTIs.

We can calculate the probability of collision given a number of UEs, N, that are transmitting in the DSA window:

| Case A | Pn, probability of success for n UEs |  |  |  |  | M, average number of successful UEs | Pc, collision probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N, number of UEs | P0 (n = 0) | P1 (n = 1) | P2 (n = 2) | P3 (n = 3) | P4 (n = 4) | M = 0*P0+1*P1+2*P2+3*P3 | 1 – M/N |
| 2 | 0.25 | 0 | 0.75 | 0 | 0 | 1.5 | 0.25 |
| 3 | 0.0625 | 0.625 | 0.3125 | 0 | 0 | 1.25 | 0.5833 |
| 4 | 0.25 | 0.5625 | 0.1875 | 0 | 0 | 0.9375 | 0.7656 |

| Case B | Pn, probability of success of n UEs |  |  |  |  | Average number of successful UEs, M | Collision probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of UEs, N | n = 0 | n = 1 | n = 2 | n = 3 | n = 4 | M = 0*P0+1*P1+2*P2+3*P3 | 1 – M/N |
| 2 | 0.1667 | 0 | 0,8333 | 0 | 0 | 1.6667 | 0.1667 |
| 3 | 0.1389 | 0.4167 | 0.3333 | 0.1111 | 0 | 1.4167 | 0.5278 |
| 4 | 0.2407 | 0.5370 | 0.2222 | 0 | 0 | 0.9815 | 0.7546 |

Restrictions that limit the UEs ability to freely select transmission opportunities within a DSA window will increase the probability of collision.

This fact will affect the best way to configure resources for the DSA window, it is beneficial to allocate more time resources than frequency resources for the DSA window.

For example, comparing a DSA window with totally 24 possible replica resources and 12 ms DSA window periodicity allocated as a) 6 subcarriers times 4 ms, and b) 2 subcarriers for 12 ms.

If a UE cannot select two resources that are on different subcarrier at the same time, then for a) after selecting a first replica resource, there are 5 excluded resources for selecting the second replica resource while for b) after selecting a first replica resources there are only 1 excluded resource when selecting the second replica resource.

Figure A.1: Example 2 Excluded resources (red) after selecting a first replica (blue).

When configuring resources for the DSA window, it is beneficial to allocate more resources in the time domain and less in frequency domain than vice versa.

## A.3	Legacy NB-IoT resources

Here we go through the most important properties of legacy NB-IoT that will affect the CB-Msg3-EDT functionality.

### A.3.1	NPUSCH resources

The UE can only transmit in subframes that are NB-IoT UL subframes (see 36.213 section 16.5).

There are two uplink subcarrier spacings (SCS) 3.75 kHz (subcarrier 0 to 47 in one PRB) and 15 kHz (subcarrier 0 to 11 in one PRB).

There are two NPUSCH formats, Format 1 for data transmissions and Format 2 for HARQ ACK/NACK transmissions. See 36.211:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Table 10.1.2.1-1: NB-IoT parameters.

| Subcarrier spacing |  |  |
| --- | --- | --- |

The Format 2 is used for HARQ ACK/NACK transmissions. Format 2 is single tone for both 3.75 kHz and 15 kHz SCS and uses 8 ms for 3.75 kHz SCS and 2 ms for 15 kHz SCS without repetitions.

For 3.75 kHz SCS, HARQ ACK/NACK transmission without repetitions use 8 ms on one subcarrier.

For 15 kHz SCS, HARQ ACK/NACK transmission without repetitions use 2 ms on one subcarrier.

For Format 1 3.75 kHz SCS, we can only have a single tone (1 subcarrier) transmissions and these uses 32 ms without repetitions for one RU.

For Format 1 15 kHz SCS, we can have single tone or multi tone (multi subcarriers) transmissions without repetitions for one RU:

|  | RU duration |
| --- | --- |
| 1 | 8 ms |
| 3 | 4 ms |
| 6 | 2 ms |
| 12 | 1 ms |

For 3.75 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 32 ms on one subcarrier.

For 15 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 8 ms for one subcarrier, 4 ms for three subcarriers, 2 ms for six subcarriers and 1 ms for twelve subcarriers.

Each uplink transmission can use one or more RUs where each RUs follow contiguous in time. Higher nrof RUs enables higher transport block sizes. Further, each transmission can have repetitions. From 36.213:

Table 16.5.1.1-2: Number of resource units () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Increasing number of RUs can increase the UL transport block size.

The number of subframes for one NPUSCH transmission increase if more RUs or more repetitions are used.

To further complicate things when repetitions are used, there are also the uplink transmission gaps. From 36.211:

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

Thus, after 256 ms of NPUSCH transmission the UE must be silent for 40 ms – this will affect all NPUSCH transmission that are longer than 256 ms.

For example, a 1 RU, 3.75 kHz SCS uses 32 ms for one transmission and if 128 repetitions are used (the maximum) one transmission will use 32*128 ms = 16*256 ms for the transmissions but every 256 ms there is a 40 ms gap, thus the total transmission length is 16*256 + 15*40 ms.

Some of the contiguous subframes cannot be used for NPUSCH transmissions.

The NB-IoT UEs cannot transmit NPUSCH continuously for more than 256 ms before inserting a 40 ms silent period of postponed NPUSCH transmission subframes.

### A.3.2	NPDSCH resources and ACK/NACK resources

For downlink there is only 15 kHz SCS and each NPDSCH transmissions uses all the 12 subcarriers.

The NPDSCH transmissions uses 15 kHz SCS and 12 subcarriers.

The UE can only transmit in subframes that are NB-IoT DL subframes (see 36.213 section 16.4), some of the excluded subframes are NPBCH (1 ms every 10 ms), NPSS (1 ms every 10 ms), NSSS (1 ms every 20 ms), and SIB1-NB (scheduled in MIB on the NPBCH). Further gaps may be configured (36.211 10.2.3.4).

Not all DL subframes are available for NPDSCH transmission, for example subframes with NPBCH, NPSS, NSSS, SIB1-NB and configured gaps are excluded.

Each downlink transmission use one or more subframes contiguous in time. Higher nrof subframes enables higher transport block sizes. Further, each transmission can have a number of repetitions. From 36.213:

Table 16.4.1.3-1: Number of subframes () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

Table 16.4.1.5.1-1: Transport block size (TBS) table.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Increasing number of subframes can increase the DL transport block size.

The number of subframes for one NPDSCH transmission increase if more subframes or more repetitions are used.

There is a hard coded gap of four subframes between last NPDCCH subframe and the first (possible) NPDSCH subframe. To further complicate things, some of the subframes are not used for NPDSCH and are excluded. From 36.213

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N consecutive NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages or scheduling gap (if any) or processing gap (if any) where, n0<n1<…,nN-1 ,

The hard coded gap can be increased by k0 signalled in the DCI, from 36.213:

Table 16.4.1-1: for DCI format N1.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 4 | 16 |
| 2 | 8 | 32 |
| 3 | 12 | 64 |
| 4 | 16 | 128 |
| 5 | 32 | 256 |
| 6 | 64 | 512 |
| 7 | 128 | 1024 |

Table 16.4.1-1a: for DCI format N1 with DCI CRC scrambled by G-RNTI.

| 0 | 0 |
| --- | --- |
| 1 | 4 |
| 2 | 8 |
| 3 | 12 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

Some of the contiguous subframes cannot be used for NPDSCH transmissions.

#### A.3.2.1	UE procedure for reporting ACK/NACK

The legacy NPDSCH transmissions are acknowledged on resources that are indicated using 4 bits in the DCI that scheduled the NPDSCH.

The resource for HARQ ACK/NACK is indicated with 4 bits in the DCI.

From 36.213 (where k0 – 1 is the nrof subframes between last NPDSCH subframe and the first NPUSCH Format 2 RU carrying HARQ ACK/NACK for a UE):

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	 DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

…

Table 16.4.2-1: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 38 | 13 |
| 1 | 39 | 13 |
| 2 | 40 | 13 |
| 3 | 41 | 13 |
| 4 | 42 | 13 |
| 5 | 43 | 13 |
| 6 | 44 | 13 |
| 7 | 45 | 13 |
| 8 | 38 | 21 |
| 9 | 39 | 21 |
| 10 | 40 | 21 |
| 11 | 41 | 21 |
| 12 | 42 | 21 |
| 13 | 43 | 21 |
| 14 | 44 | 21 |
| 15 | 45 | 21 |

Table 16.4.2-2: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 0 | 13 |
| 1 | 1 | 13 |
| 2 | 2 | 13 |
| 3 | 3 | 13 |
| 4 | 0 | 15 |
| 5 | 1 | 15 |
| 6 | 2 | 15 |
| 7 | 3 | 15 |
| 8 | 0 | 17 |
| 9 | 1 | 17 |
| 10 | 2 | 17 |
| 11 | 3 | 17 |
| 12 | 0 | 18 |
| 13 | 1 | 18 |
| 14 | 2 | 18 |
| 15 | 3 | 18 |

There is a gap of at least 12 subframes between the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK. Further, looking into the possible allocations we note that:

For 3.75 kHz SCS, HARQ ACK resources can be on one of 8 different subcarriers and use one of 2 different scheduling delays.

For 15 kHz SCS, HARQ ACK resources can be on one of 4 different subcarriers and use one of 4 different scheduling delays.

There is a gap of at least 12 subframes between the the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK.

### A.3.3	NPDCCH resources

NB-IoT have possibility of having one or two NPDCCH in one subframe, 36.211:

One or two NPDCCHs can be transmitted in a subframe.

Table 10.2.5.1-1: Supported NPDCCH formats

| NPDCCH format | Number of NCCEs |
| --- | --- |
| - 0 | - 1 |
| - 1 | - 2 |

Further NPDCCH can also be configured with a number of repetitions, 36.211:

With Format 1, each NCCE uses 6 subcarriers, and Format 0 uses all the 12 DL subcarriers.

NPDCCH uses 6 or 12 subcarriers in one subframe (without repetitions) can carry two respectively one DCI.

If coverage is challenging, we may assume only NPDCCH format 1 with one DCI can be transmitted in each subframe when no repetitions are used.

There are limits to the timing of when the NPDCCH can earliest occur after NPUSCH transmission (which can be Format 1 with data or Format 2 with ACK/NACK), 36.213 section 16.6:

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-  and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

- if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3.

otherwise,

- If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

There are also limits to the timing of when the NPDCCH can earliest occur after NPDSCH transmission that do not require NPUSCH Format 2 for ACK/NACK, 36.213 section 16.6:

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

There is a guard period of at least 3 subframes after an NPUSCH transmission (Format 1 or 2) and first possible NPDCCH reception.

There is a guard period of at least 12 subframes after an NPDSCH reception and the first possible NPDCCH reception.

Further, all excluded subframes for NPDSCH are also excluded for NPDCCH.

Some of the contiguous subframes cannot be used for NPDCCH transmissions.

## A.4	Example of one DSA window and one Msg4 window

To explore the possibilities for the DSA window and the Msg4, we start with one possible configuration for a DSA window and a Msg4 window.

We assume a 3.75 kHz SCS GEO scenario with 544 ms RTT and with good coverage, that is, no repetitions and each transmission use one RU in uplink respectively one subframe in downlink. Each RU is then 32 ms long.

The DSA window is 12 subcarriers, and 8 RUs in time, in total 96 transmission opportunities, but not all of them can be selected at any time for one UE (to transmit Msg3 on two different subcarriers at the same time may be costly to implement for a UE as power amplifier linearity and peak power will increase, see “A.2	Restriction on selectable DSA resources”).

The DSA window is then 8*32 ms = 256 ms long.

For the timing of the Msg4, we assume the same restrictions as for legacy scheduling (for more info see A.3	Legacy NB-IoT resources). That is, a minimum of 3 ms, for eNB processing and switching time of UE from UL tx to DL rx, between last NPUSCH subframe and the first NPDCCH subframe, a minimum gap of four subframes between the last NPDCCH subframe and the first NPDSCH subframe, and a minimum gap of 12 subframes between the last NPDSCH subframe and the first NPUSCH Format 2 subframe (the HARQ ACK signalling). The NPUSCH Format 2 can also be replaced by another NPDCCH monitoring occasion.

A UEs can expect a Msg4 message earliest UE-eNB RTT (the TA plus Kmac) after the end of a Msg3, but here we assume the UE transmit two replicas in the DSA window and then wait until the end of the DSA window before switching to NPDCCH monitoring for Msg4.

To summarize all information about the Figure 1:

- The x-axis is proportional in time

- 3.75 kHz SCS

- One RU is 32 ms on a single subcarrier

- UL transmissions are illustrated at the UE

- DL transmissions are illustrated at the eNB

- UL subframes not available for UL transmission are ignored

- DL subframes not available for DL transmission are ignored

- The HARQ ACK/NACK uses 8 ms

  - The NPUSCH resources for HARQ ACK/NACK are illustrated as a window of 8*8 ms on subcarrier 0 to 11 while the actual resources for HARQ ACK/NACK are on subcarriers 38 to 45, further we can only signal the ACK/NACK to be either 13 or 21 subframes after the last NPDSCH subframe for 3.75 kHz SCS, thus a real window can only be 2*8 ms

- DSA window uses 12 subcarriers and 8 RUs in time

  - DSA window is 12*8 ms = 256 ms long

- The DSA window Periodicity is just arbitrarily selected to be 256+544+21+64 ms = 885 ms

  - it may be longer (for example if DL subframes are excluded, or if UL subframes are excluded)

  - it may be shorter (for example if we make the ACK/NACK window shorter)

- Msg3 uses one RU without repetitions and there is two replicas (in blue colour)

- Msg4 has same timings as in legacy scheduling of DL

  - 3 ms gap for eNB and UE processing between last NPUSCH subframe and the first NPDCCH subframe

  - 4 ms gap for eNB and UE processing between last NPDCCH subframe and the first NPDSCH subframe (plus the scheduling delay k0 = 0 in DCI)

  - 12 ms gap for UE processing after the last NPDSCH subframe and the first NPUSCH Format 2 subframe (the HARQ ACK signalling)

- the UEs transmit two Msg3 replicas in the DSA window and then wait until the DSA window end plus UE-eNB RTT before switching to NPDCCH monitoring

- Note that the effect Koffset is not shown (that is, figure assumes a perfect Koffset = TA), if Koffset is included likely HARQ A/N is a little delayed (especially for UEs that have lower RTT than the maximum RTT in the cell)

- Note that Kmac is assumed to be 0

Figure 1: Example of DSA window, Msg4 and HARQ A/N, no repetitions

In the next figure (Figure 2) we have the same assumptions as in Figure 1, but we have assumed the maximum number of repetitions are used for each transmission (128 for UL and 2048 for DL) and we ignore the 40 ms silent period every 256 ms of UL transmission.

To summarize all information about the Figure 2:

- Same assumptions as Figure 1 except:

  - 128 repetitions for each UL transmission

  - 2048 repetitions for each DL transmission

  - We ignore that the UE needs 40 ms silent periods every 256 ms of UL transmission

Figure 2: Example of DSA window, Msg4 and HARQ A/N, maximum repetitions

The transparent payload maximum propagation RTT (see 38.821 table 4.2-2) is about 542 ms in GEO, about 42 ms in 1200 km LEO, and about 26 ms in 600 km LEO.

The eNB do not know the UEs individual RTT, it needs to assume a UE is at the maximum RTT and all UEs will need to use the cell specific Koffset for the CB-EDT functionality.

The minimum size of the Msg4 window, as seen from the UE, is the propagation RTT + 21 + 8 ms (with one NPDSCH subframe, without repetitions) and RTT + 4115 + 1024 ms (with one NPDSCH subframe, with maximum repetitions).

Each Msg4 NPDCCH search space can contain several occasions to schedule a Msg4.

However, one method to address multiple UEs in the same Msg4 window if multiple RNTIs are used (in some way the UEs within a DSA window choose different RNTIs to listen for in the Msg4 window) is to have a NPDCCH search space and send multiple DCIs for multiple Msg4 (and use k0 to separate the corresponding NPDSCH messages in time). This method has one big drawback, as the eNB do not know when the UE sent other replicas and the UE do not know which replica that will be successful – the UEs must  search for multiple RNTIs, but eNB may reply to any of the RNTIs thus the UE must decode all RNTIs that the replicas indicate and therefore must decode every Msg4 message and look for the UE IDs to find if it was addressed in that Msg4 or not. Therefore, the Msg4 cannot overlap in time in one search space for NPDCCH, and every Msg4 that is scheduled may need to be decoded by the UEs to know whether the correct UE ID is in it.

The UEs likely must decode every Msg4 even if each Msg3 use a different RNTIs because UE do not know which RNTI the eNB will reply to, when multiple RNTIs are used.

If the UE is addressed in the DCI of a Msg4 (the DCI is scrambled with the RNTI the UE is listening for) but if the Msg4 content do not match the UE ID of the UE, then the UE can start monitoring NPDCCH after the 12 ms gap after the last NPDSCH subframe because it will never need to send a HARQ A/N if it was not addressed and any successful UEs do not need to monitor NPDCCH for another Msg4 (successful UEs can send the HARQ ACK/NACK at the same time as other UEs start monitoring NPDCCH for another Msg4).

To make sure many UEs can be addressed in a Msg4 window and to enable eNB flexibility in scheduling, we can extend the Msg4 window with multiple Msg4 transmissions opportunities from the minimum 21 + 8 ms to 21 + (21 – 3) + 8 ms (with one NPDSCH subframe, without repetitions) and from the minimum 4115 + 1024 ms to 4115 + (4115 – 3) + 1024 ms (with one NPDSCH subframe, with maximum repetitions). Note that the three subframes needed before the first NPDCCH occasion is not needed in front of the following NPDCCH subframes because the eNB has already had time to prepare the transmission after the NPUSCH transmissions, and the UEs have already switched to DL monitoring in the first NPDCCH occasion.

One Msg4 window can allow for multiple Msg4 transmissions consecutive in time.

# Appendix B	RAN2 agreements

## B.1	RAN2#125bis

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

## B.2	RAN2#126

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

## B.3	RAN2#127

Agreements:

1.	RAN2 will continue working on a CB Msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

## B.4	RAN2#127bis

In evaluation of possible methods to improve UL capacity for CB Msg3 ETD-like transmission we focus on a working point where the PLR is lower than 10% (this does not mean that we are mandating any network to work at any working point)

We use the term CB Msg3 EDT for now

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB Msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB Msg3 assuming that OCC2 might also apply to CB Msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB Msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB Msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB Msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB Msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB Msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB Msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

## B.4	RAN2#128

Agreements:

1.	Only system Information is used to provide cell-specific CB Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB Msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB Msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB Msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB Msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB Msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB Msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

## B.5	RAN2#129

Agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

3.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

4.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

5.	The UE shall at most have one ongoing CB EDT procedure at any time.

6.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

7.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

8.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

7.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

8.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

## B.5	RAN2#130

Agreements;

1.	We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

Agreements;

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

2.	The Msg4 monitoring window configuration (e.g. length) is CE level specific

3.	RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

5.	RAN2 also intends to support CB-msg3-EDT for MT cases

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).