3GPP TSG-RAN WG2 #130	R2-2504652

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

So far, RAN2 has reached several agreements regarding the shape/format of the intended service area as well as some of the related UE behaviours when located inside and outside the intended service area (ISA). However, signalling means as well as service continuity are still under discussion. This contribution presents our views on the open issues.

# 2	MBS service continuity in NTN

#### 2.1	Frequency Selection Areas (FSA) for cell reselection

##### 2.1.1	Background

The second mechanism helps the UE avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies [2]. If a neighbour cell/frequency is providing an MBS broadcast service the UE is interested in receiving, and the UE can only receive this MBS broadcast service by camping on that neighbour cell/frequency, then this mechanism allows temporary frequency prioritization to perform cell reselection towards that neighbour cell/frequency [3]. The UE can figure out the TMGI-Frequency mapping from the combination of USD/Service Announcement and SIB21.

A UE can prioritize a frequency when it is interested in an MBS broadcast service, and it can only receive that service by camping on that frequency.

A UE obtains the FSAI associated with an MBS broadcast service from the Service Announcement. A UE obtains the frequency associated with an FSAI from SIB21.

From the network perspective, MBS FSA ID and their mapping to frequencies are provided to RAN nodes via OAM [4]. During MBS Session Start for Broadcast and MBS Session Update for Broadcast, the MB-SMF may include the MBS FSA ID(s) for the MBS session and send them to the NG-RAN nodes via the AMF. The NG-RAN nodes may then use those MBS FSA ID(s) to determine cells/frequencies within the MBS service area to broadcast MBS session data [4]. Finally, the gNBs may exchange this information with their neighbours within Xn Setup messages and subsequent Xn Configuration Update messages to help with frequency layer prioritization [2].

MBS FSA ID and their mapping to frequencies are provided to RAN nodes via OAM.

RAN nodes receive MBS FSA ID related to a MBS broadcast service during MBS Session Start/Update.

SIB21 information element

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

...

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE { -> Configured in the RAN node via OAM

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

-- TAG-SIB21-STOP

-- ASN1STOP

Table 5.2.1 1: Externally defined data types used by User Service Description schema

| Data type | Comments | Reference |
| --- | --- | --- |
| Uri | A Uniform Resource Locator | TS 29.571 [30] |
| DateTime | A date–time value. |  |
| MbsServiceArea | An MBS Service Area. |  |
| MbsFsaId | An MBS Frequency Selection Area identifier. |  |
| DurationSec | A time duration expressed in seconds. |  |
| AbsoluteUrl | An absolute URL | TS 29 512 [31] |

##### 2.1.2	Association between of ISA ID and FSA ID

The inclusion of ISA information in SIB21 was extensively discussed in RAN2#129bis and the following post-meeting email discussion. Even though many companies propose to provide the mapping between ISA and FSA ID in SIB21, it is unclear which specific scenario they intend to address with such enhancement. Therefore, in the RRC open issue list, it was proposed that, for the service continuity discussion in MBS NR NTN, RAN2 takes the following scenarios as a baseline:

- Multiple frequencies, served by the same satellite, illuminating the same geographical area. Each frequency provides a different MBS service. These services are not being broadcast in the serving cell.

- Multiple frequencies, served by the same or different satellites, but not covering the same geographical area (they are neighbour cells). Each frequency provides a different MBS service. These services are not being broadcast in the serving cell.

Figure 1. Baseline scenarios for MBS Service continuity enhancements in NR NTN.

These two scenarios are depicted in Figure 1 and named scenario A and B, respectively. In Scenario A, there are two frequencies that are serving the same geographical area. In Scenario B, there are two frequencies that are serving adjacent geographical areas. In both cases, each frequency is providing a different MBS service which is associated with its own ISA, e.g., ISA 1 and ISA 2. From NTN perspective, we understand that spectrum is a scarce resource, and the most likely real deployment is Scenario B. Therefore, any enhancements that RAN2 considers on MBS service continuity should take this scenario as a baseline.

Prioritize MBS NTN service continuity enhancements based on scenario B.

In RAN2#129bis, it was agreed that the UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). A remaining open issue is how the UE obtains the association between ISA and FSA frequencies. Firstly, let’s review how the network obtains FSA ID and ISA from the core network. Similar to FSA ID, RAN nodes are informed of the ISA via the AMF in the MBS Session Start for Broadcast as part of the MBS Session Context [5,6]. In the scenarios illustrated in Figure 1, frequency A and frequency B may be provided by different gNBs. In this case, there is no existing mechanism for the involved gNBs to know the ISA(s) related to MBS services provided by the other gNB, even if they know the FSA ID associated with neighbour frequencies. Each gNB is only informed of the ISA(s) for the MBS broadcast service(s) it serves, so it does not have enough information to provide the mapping between ISA ID and FSA ID in SIB21. Here, we see two possible solutions: the core network (e.g., AMF) needs to inform not only the ISAs but also the associated FSAIs to the respective gNB and their potential neighbours, alternatively, the neighbouring gNBs need to exchange information about ISA and FSA ID. Both options have a non-negligible impact in other working groups.

A gNB is informed by the AMF of the ISA(s) and FSA ID of the MBS broadcast service(s) it serves.

A gNB is unaware of the ISA(s) associated to MBS broadcast services provided by neighbouring RAN nodes.

There is an impact to RAN3 and SA2 WG if the ISA ID needs to be broadcast in SIB21.

From UE perspective, the agreement taken in RAN2 is helpful to avoid prioritizing a frequency which provides MBS service the UE wants to receive when the UE is not located within the associated ISA(s). However, unless all cells within the FSA are provided by the same gNB, including the mapping between FSA ID and ISA ID in SIB21 increases network complexity and introduces unnecessary inter-node signalling and coordination.

Providing the association between ISA and frequencies via SIB has RAN3 impact which increases NW complexity and cannot work when SIB21 is not provided.

A clean and straightforward solution would be to rely on the Service Announcement to provide the ISA ID and the FSA ID directly from the AF to the UE. Then, the UE has all information it needs from USD and SIB21. RAN2 has previously sent a LS to SA4 and SA2 to request the inclusion of the NTN ISA in the Service Announcement. Given that the topic is still under discussion in SA4, we propose that how to provide the association between FSA ID and ISA ID is based on the LS reply from SA2/SA4 and postponed until RAN2 receives an answer.

The Service Announcement can provide the ISA ID and FSA ID directly to the UE for frequency prioritization. This solution minimizes RAN impact and inter-node coordination.

RAN2 discusses how to provide the association based on the LS reply from SA2/SA4.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE can prioritize a frequency when it is interested in an MBS broadcast service, and it can only receive that service by camping on that frequency.

Observation 2	A UE obtains the FSAI associated with an MBS broadcast service from the Service Announcement. A UE obtains the frequency associated with an FSAI from SIB21.

Observation 3	MBS FSA ID and their mapping to frequencies are provided to RAN nodes via OAM.

Observation 4	RAN nodes receive MBS FSA ID related to a MBS broadcast service during MBS Session Start/Update.

Observation 5	A gNB is informed by the AMF of the ISA(s) and FSA ID of the MBS broadcast service(s) it serves.

Observation 6	A gNB is unaware of the ISA(s) associated to MBS broadcast services provided by neighbouring RAN nodes.

Observation 7	There is an impact to RAN3 and SA2 WG if the ISA ID needs to be broadcast in SIB21.

Observation 8	Providing the association between ISA and frequencies via SIB has RAN3 impact which increases NW complexity and cannot work when SIB21 is not provided.

Observation 9	The Service Announcement can provide the ISA ID and FSA ID directly to the UE for frequency prioritization. This solution minimizes RAN impact and inter-node coordination.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Prioritize MBS NTN service continuity enhancements based on scenario B.

Proposal 2	RAN2 discusses how to provide the association based on the LS reply from SA2/SA4.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

TS 38.300

TS 38.304

TS 23.247

S2-2501328

R3-253095