3GPP TSG-RAN WG2 #130	R2-2504662

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.4

Source:	Ericsson

Title:	MBS signalling details in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

This contribution presents our views on one of the open issues identified after the email discussions following RAN2#129bis, namely:

- Nature of SIBXX defining the intended service area: cell or area specific.

# 2	Discussion

In Release 19, RAN2 has taken the following agreements related to the ISA of an MBS broadcast service:

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell.

- The intended service area can cover the area of more than one NTN cells (or portions thereof).

- A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s).

In our view, these agreements aim to limit the scope of the potential scenarios considered for enhancements in Release 19. The first agreement establishes that ISA should be smaller than an NTN cell, as exemplified by ISA 1 or ISA 3 in Figure 1. If the ISA is larger than the size of an NTN cell, the ISA concept becomes redundant in the cells that are fully included in the ISA, and existing legacy mechanisms are sufficient, given that geofencing would be unnecessary for MBS broadcast services within the serving cell. However, when the ISA is larger than a cell, even though some cells will be completely included in the ISA, there may be cells at the edge of the ISA which are only partially covered by the ISA. In these cells, the ISA is relevant and should be indicated. The second and third agreement refers to ISA 2, depicted in Figure 1, which lies within the overlapping coverage area of neighbouring cells 1 and 2. The RAN2 agreement presently allows both cells to broadcast information about ISA 2, which is crucial for minimizing system information updates in scenarios such as Earth-moving cells.

Figure 1. Typical scenario for the use of ISA in an NTN system.

Figure 1 illustrates a typical scenario for ISA utilization in an NTN system. Two cases are possible: cells served by either the same or different gNBs. In the first configuration, cell 1 and cell 2 are served by different gNBs. Unless ISA 1 and ISA 3 are linked to the same MBS broadcast service, there is no mechanism for one gNB to be aware of the ISA broadcast by another gNB, as the AMF transmits ISA information solely to the gNB responsible for broadcasting the MBS service during session creation. An exception arises when both ISAs (ISA 1 and ISA 3) are associated with the same MBS broadcast service, allowing both gNBs to be informed of the remaining areas.

It is highly likely that a gNB lacks knowledge of the ISA in neighbouring cells served by different gNBs.

In the second case, both cells are served by the same gNB. Given the typical 50 km radius of NTN cells, it is unnecessary for a UE in Cell 1 to be aware of the ISAs and associated MBS services in Cell 2, which may be distant. While such information might aid in service continuity by allowing the UE to skip SIB acquisition upon moving to Cell 2, this optimization increases signalling overhead in Cell 1, reducing its potential benefits. Provided there are benefits for cell or area specific approach, we propose that the nature of the new SIB describing the ISAs is left up to network implementation and RAN2 does not define any limitations.

It is up to network implementation whether the new SIB describing the ISA(s) is cell specific or area specific.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	It is highly likely that a gNB lacks knowledge of the ISA in neighbouring cells served by different gNBs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is up to network implementation whether the new SIB describing the ISA(s) is cell specific or area specific.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.