3GPP TSG-RAN WG2 Meeting #130	R2-2504722

# Malta, MT, May 19th – May 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2090 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MCSt for resource selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Capturing the #129bis meeting’s agreement. R2-2503014 Summary of [AT129b][101][V2X/SL] Discuss multiple candidate options for SL resource (re)selection in MCSt (LG) LG Electronics Inc. discussion Rel-18 38.321 NR_SL_enh2-Core [Session chair]: Based on that there is no technical blocking issue for any direction, would like to suggest A+W considering MCSt using single slot candidate resource is done as best effort manner. - Option (A + W) for MCSt using single slot candidate resources is agreed. To determine whether retransmission is needed or not: Option A: When an LBT failure indication is received for any resource within a MCSt, which is assigned to a given TB. To determine when resource (re)selection for retransmission is triggered: Option W: At each resource for a given TB within a MCSt. - We will see CR next meeting (LG). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The text in section 5.22.1.2c has been modified so that when LBT fails in MCSt using single slot candidate resources, the UE performs the same resource (re-)selection procedure as in the non-MCSt case. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: NR sidelink unlicensed band Interoperability: For change 1, if the network implements this CR but not the UE, there is no interoperability issue. For change 1 if the UE implements this CR but not the network, there is no interoperability issue. For change 1 if one UE implements this CR but not the other UE, resource (re-)selection procedure in MCSt using single slot candidate resources is inconsistent with each other. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE resource selection behavior in MCSt using single slot candidate resources is not supported by the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.2c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

The MAC entity shall only apply the above procedure for non Multi-consecutive slots transmission case and Multi-consecutive slots transmission case using single slot candidate resources.

END of CHANGE