3GPP TSG-RAN WG2 Meeting #130	R2-2504731

Malta, 19 – 23 May 2025

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Proposals for PL offset for RRCresume / HO (Huawei)

Document for:  Discussion and Decision

1	Introduction

[AT130][201][MIMO_Ph5] Proposals for PL offset for RRCresume / HO (Huawei)

Scope: Discuss proposals on PL offset for RRCresume / HO, taking into account R2-2504100 and R2-2503644

Intended outcome: Summary with Proposals in R2-2504731.

Deadline: before Friday CB

2	Discussion

Based on the running CR, the following parameter is captured for pathloss offset.

Joint mode

tci-StatesToAddModList SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State OPTIONAL, -- Need N tci-StatesToReleaseList SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId OPTIONAL, -- Need N TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, […], [[ pathlossOffset-r19 ENUMERATED { dB-12, dB-8, dB-4, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R ]] }

Separate mode

BWP-UplinkDedicated ::= SEQUENCE { […] [[ ul-TCI-StateList-r17 CHOICE { explicitlist SEQUENCE { ul-TCI-ToAddModList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-State-r17 OPTIONAL, -- Need N ul-TCI-ToReleaseList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-StateId-r17 OPTIONAL -- Need N }, unifiedTCI-StateRef-r17 ServingCellAndBWP-Id-r17 } TCI-UL-State-r17 ::= SEQUENCE { […], [[ pathlossOffset-r19 ENUMERATED { dB-12, dB-8, dB-4, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R ]] }

Observation 1: for pathloss offset, the ToAddModList and ToReleaseList are used.

2.1 current specification understanding

- ToAddModList and ToReleaseList

The moderator checked the RRC specification with the following captured for ToAddModList and ToReleaseList:

Use of different Need codes in different parts of a condition should be avoided. For downlink RRC message and sidelink PC5 RRC messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that. For (parent) fields without need codes in downlink RRC messages or sidelink PC5 RRC message, if the parent field is absent, UE shall follow the need codes of the child fields. Thus, if parent field is absent, the need code of each child field is followed (i.e. Need R child fields are released, Need M child fields are not modified and the actions for Need S child fields depend on the specified conditions of each field). Examples of (parent) fields in downlink RRC messages and sidelink PC5 RRC message without need codes where this rule applies are: A.3.9 Guidelines on use of ToAddModList and ToReleaseList In order to benefit from delta signalling when modifying lists with many and/or large elements, so-called add/mod- and release- lists should be used. Instead of a single list containing all elements of the list, the ASN.1 provides two lists. One list is used to convey the actual elements that are to be added to the list or modified in the list. The second list conveys only the identities (IDs) of the list elements that are to be released from the list. In other words, the ASN.1 defines only means to signal modifications to a list maintained in the receiver (typically the UE). An example is provided below: -- /example/ ASN1START AnExampleIE ::= SEQUENCE { elementsToAddModList SEQUENCE (SIZE (1..maxNrofElements)) OF Element OPTIONAL, -- Need N elementsToReleaseList SEQUENCE (SIZE (1..maxNrofElements)) OF ElementId OPTIONAL, -- Need N ... } Element ::= SEQUENCE { elementId ElementId, aField INTEG ER (0..16777215), anotherField OCTET STRING, ... } ElementId ::= INTEGER (0..maxNrofElements-1) maxNrofElements INTEGER ::= 50 maxNrofElements-1 INTEGER ::= 49 -- /example/ ASN1STOP As can be seen, the elements of the list must contain an identity (INTEGER) that identifies the elements unambiguously upon addition, modification and removal. It is recommended to define an IE for that identifier (here ElementId) so that it can be used both for a field inside the element as well as in the elementsToReleaseList. Both lists should be made OPTIONAL and flagged as "Need N". The need code reflects that the UE does not maintain the received lists as such but rather updates its configuration using the information therein. In other words, it is not possible to provide via delta signalling an update to a previously signalled elementsToAddModList or elementsToReleaseList (which Need M would imply). The update is always in relation to the UE's internal configuration. Note that the release of a field (a list element as well as any other field) releases all its sub-fields (sub-fields configured by elementsToAddModList and any other sub-field). If no procedural text is provided for a set of ToAddModList and ToReleaseList, the following generic procedure applies: The UE shall: 1> for each ElementId in the elementsToReleaseList,: 2> if the current UE configuration includes an Element with the given ElementId: 3> release the Element from the current UE configuration; 1> for each Element in the elementsToAddModList: 2> if the current UE configuration includes an Element with the given ElementId: 3> modify the configured Element in accordance with the received Element; 2> else: 3> add received Element to the UE configuration.

Based on the above, the moderator observed that UE will maintain an internal configuration for all configured Element configuration (Parent) and update the internal configuration based on the new RRC configuration.

Observation 1:

- Element is absent in elementsToAddList and the ID of this Element is absent in elementsToReleaseList

UE behaviour: UE continues to apply the internal configuration for this Element (including Need R).

- RAN2 understands UE maintains the internal configuration for this element (including Need R) in case the parent element (element in elementsToAddList) is absent as legacy.

- Element is present in elementsToAddList and the ID of this Element is absent in elementsToReleaseList

UE behaviour: UE updates the maintained internal configuration for this Element based on the Need code of child parameter.

- Element is absent in elementsToAddList and the ID of this Element is present in elementsToReleaseList

UE behaviour: UE removes the Element in maintained internal configuration.

Delta signalling for value mismatching

We think the following cases may have the value mismatching between network and UE.

- Case 1: non-handover case, i.e. RRC reconfiguration.

For the second RRC reconfiguration, the mismatch issue may happen for both Need R and Need M.

| TCI state in tci-StatesToAddList (parent) | Need code of Pathloss offset (child) |  |
| --- | --- | --- |
|  | Need R | Need M |
| Presentfollow child Need code | No value mismatch | Value mismatch in case absence of Pathloss offset |
| Absentcontinue to apply value in internal configuration | Value mismatch | Value mismatch |

- If Need code of pathloss offset is Need R, the UE follows the internal UE configuration in case TCI state is absent.

- If Need code of pathloss offset is Need M, the UE also follows the internal UE configuration in case TCI state is present and pathloss offset is absent.

For both cases, it is unclear whether or not the value kept in the internal UE configuration is updated based on the MAC CE for pathloss update.

- Case 2: handover case.

For avoiding mismatching between the UE and target node, the moderator think the PL offset value kept in UE internal configuration shall be same as the PL offset value provided from source node to target node in HandoverPreparationInformation. If MAC CE can update the UE internal configuration, some spec impact are needed in F1 (e.g. from source gNB-DU to source gNB-CU) since the source gNB-CU does not know the MAC CE value before sending the configuration from the source gNB-CU to the target gNB-CU. If MAC CE cannot update the UE internal configuration, there is no RAN3 spec.

- Case 3: resume case.

For avoiding mismatching between the UE and receiving node, the moderator think the PL offset value kept in UE internal configuration shall be same as the PL offset value provided from source node to target node. If MAC CE can update the UE internal configuration, some spec impact are needed in F1 (e.g. from anchor gNB-DU to anchor gNB-CU) since the anchor gNB-CU does not know the MAC CE value before sending the configuration from the anchor gNB-CU to the receiving gNB-CU. If MAC CE cannot update the UE internal configuration, there is no RAN3 spec.

Proposal 1: RAN2 to confirm that the PL offset value stored in the UE, i.e. in the internal UE configuration is not updated based on the MAC CE for PL update.

Proposal 2: RAN2 to confirm that the PL offset value stored in the source gNB/anchor gNB, i.e. in the UE RRC AS configuration, is not updated based on the MAC CE for PL update.

Discussion:

CATT ask how the scell activation/deactivation status is done in legacy?

- The above two proposals are agreeable.

Based on the online discussion, the common understanding is the need code of pathloss offset is Need R. For resume case, companies mentioned the pathloss offset kept in UE internal configuration may be outdated due to mobility, i.e. value C needs to be always provided or the stored value (e.g. value A) needs to be released. The moderator thinks that this can be up to network implementation on whether the stored value or new value is to be used, or the stored value is released.

- Case 1: the network is able to obtain an accurate pathloss value for target cell at resume

If network wants to provide a new value different from source cell, the network can just provide it based on the current defined signalling by setting that new pathloss offset is present. If network wants to reuse the kept value same as source cell, the network can make the tci-state as absence.

- Case 2: the network is not able to obtain an accurate pathloss value for target cell at resume

In this case the network can release the stored pathloss offset value. UE can wait for next RRC reconfiguration for providing the accurate value after RRC resume.

We do not see a need to restrict the network implementation.

Proposal 3: It is up to network implementation whether to 1) apply the stored RRC configured PL offset value to the UE in case the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is absent, or 2) release the stored RRC configured PL offset value in case PL update is absent and the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is present, or 3) configure a new RRC value to the UE for the associated tci-State during the RRC resume procedure. No specification change is needed.

Proposal 4: Need R is applied for PL offset.

- The above two proposals are agreeable.

# 3	Conclusion

we made the following proposals for agreement:

Proposal 0 (For agreement): RAN2 understands UE maintains the internal configuration for this element (including Need R) in case the parent element (element in elementsToAddList) is absent as legacy.

Proposal 1 (For agreement): RAN2 to confirm that the PL offset value stored in the UE, i.e. in the internal UE configuration is not updated based on the MAC CE for PL update.

Proposal 2 (For agreement): RAN2 to confirm that the PL offset value stored in the source gNB/anchor gNB, i.e. in the UE RRC AS configuration, is not updated based on the MAC CE for PL update.

Proposal 3 (For agreement): It is up to network implementation whether to 1) apply the stored RRC configured PL offset value to the UE in case the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is absent, or 2) release the stored RRC configured PL offset value in case PL update is absent and the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is present, or 3) configure a new RRC value to the UE for the associated tci-State during the RRC resume procedure. No specification change is needed.

Proposal 4: Need R is applied for PL offset.

# 4	Reference

# 5	Annex

- Inter-Node RRC message

11.2.3 Mandatory information in inter-node RRC messages For the AS-Config transferred within the HandoverPreparationInformation: - The source node shall include all fields necessary to reflect the current AS configuration of the UE, except for the fields sourceSCG-NR-Config, sourceSCG-EUTRA-Config and sourceRB-SN-Config, which can be omitted in case the source MN did not receive the latest configuration from the source SN. For RRCReconfiguration included in the field rrcReconfiguration, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon; - Need codes or conditions specified for subfields according to IEs defined in clause 6 do not apply. I.e. some fields shall be included regardless of the "Need" or "Cond" e.g. discardTimer; - Based on the received AS configuration, the target node can indicate the delta (difference) to the current AS configuration (as included in HandoverCommand)to the UE. The fields newUE-Identity and t304 included in ReconfigurationWithSync are not used for delta configuration purpose.

Observation 2: The source node shall include all fields necessary to reflect the current AS configuration of the UE to target node.

The above inter-node message can be applicable to both handover and inactive resume procedure.

9.1.1.9 RETRIEVE UE CONTEXT RESPONSE UE Context Information – Retrieve UE Context Response; M; 9.2.1.13. 9.2.1.13 UE Context Information – Retrieve UE Context Response RRC Context; M; OCTET STRING; Includes the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331[10] if the old and new serving NG-RAN nodes are gNBs. Includes either the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [14] or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the old and new serving NG-RAN nodes are ng-eNBs..