3GPP TSG-RAN WG2 Meeting #130	R2-2504766

# St. Julians, Malta, May 19th – 23rd , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1419 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on redirection and cell reselection to NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#129bis, it was agreed to clarify that the E-UTRAN to NR NTN redirection and cell reselection is not supported in Release 17 and 18 in the Stage-2 specification. R2-2502249 Correction on carrier frequency information in RRCConnectionRelease. CATT, Qualcomm Incorporated CR Rel-17 36.331 17.12.0 5106 - F NR_NTN_solutions-Core - Samsung wonders if we need the first change. Ericsson agrees - Samsung thinks we should rather have a clarification in Stage2. MTK and Oppo agree - QC thinks a clarification is needed and having this in Stage 2 would also be ok Not pursued We introduce a clarification in Stage2. |
| Summary of change: |  | - In 10.2.0, add clarification that redirection from E-UTRAN to NR NTN is not supported. - In 10.2.1, add clarification that cell reselection from E-UTRAN to NR NTN is not supported with priorities provided in system information or RRC connection release message. Impact analysis Impacted functionality: Redirection and Cell reselection Inter-operability: - If the UE is implemented according to this CR but the network is not, unexpected NW configuration may be received by the UE for redirection or cell reselection purpose. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unexpected NW configuration on redirection and cell reselection may be received by the UE which may suffer from unexpected UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.0, 10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

10.2	Inter RAT

10.2.0	General

Service-based redirection between GERAN / UTRAN and E-UTRAN is supported in both directions. This should not require inter-RAT reporting in RRC CONNECTION REQUEST. Redirection from E-UTRAN to NR NTN (see TS 38.300 [79]) is not supported.

10.2.1	Cell reselection

A UE in RRC_IDLE performs cell reselection. The principles of this procedure are as follows:

-	The UE makes measurements of attributes of the serving and neighbour cells to enable the reselection process:

-	For a UE to search and measure neighbouring GERAN cells, the ARFCNs of the BCCH carriers need to be indicated in the serving cell system information (i.e., an NCL). The NCL does not contain BSICs or cell specific offsets and Qrxlevmin is given per frequency band.

-	For a UE to search and measure neighbouring UTRAN cells, the serving cell can indicate an NCL containing a list of carrier frequencies and scrambling codes.

-	For a UE to search and measure neighbouring NR cells, the serving cell can indicate the measured RS types and parameters for cell quality derivation.

-	Measurements may be omitted if the serving cell attribute fulfils particular search or measurement criteria.

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	Inter-RAT reselection is based on absolute priorities where UE tries to camp on highest priority RAT available. Absolute priorities for inter-RAT reselection are provided only by the RPLMN and valid only within the RPLMN; priorities are given by the system information and valid for all UEs in a cell, specific priorities per UE can be signalled in the RRC Connection Release message. A validity time can be associated with UE specific priorities.

-	It should be possible to prevent the UE from reselecting to specific detected neighbouring cells;

-	The UE is allowed to "leave" the source E-UTRAN cell to read the target GERAN cell broadcast, in order to determine its "suitability", prior to completing the cell reselection;

-	Cell reselection can be speed dependent (speed detection based on UTRAN solution);

-	Cell reselection from E-UTRAN to NR NTN is not supported with priorities provided by system information or RRC Connection Release message.

Cell access restrictions apply as for UTRAN, which consist of access class (AC) barring and cell reservation (e.g. for cells "reserved for operator use") applicable for mobiles in RRC_IDLE mode.

When performing cell reselection while the UE is camped on another RAT, the principles of this procedure are as follows:

-	The UE measures attributes of the E-UTRA neighbouring cells:

-	Only the carrier frequencies need to be indicated to enable the UE to search and measure E-UTRA neighbouring cells;

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	For E-UTRA neighbouring cells, there is no need to indicate cell-specific cell reselection parameters i.e. these parameters are common to all neighbouring cells on an E-UTRA frequency;

-	Cell reselection parameters are applicable to all UEs in a cell, but it is possible to configure specific reselection parameters per UE group or per UE.

-	It should be possible to prevent the UE from reselecting to specific detected neighbouring cells.

CHANGE END