3GPP TSG-RAN WG2 #130	R2-2504773

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	7.0.2.20

Source:	Ericsson

Title:	draft report of [AT130][302][TEI18] In-band operation for NB-IoT (Ericsson)

Document for:	Discussion

# Introduction

[AT130][302][TEI18] In-band operation for NB-IoT (Ericsson)

Scope: discuss the proposals in R2-2504657 and R2-2504139 and draft corresponding CRs

Intended outcome: summary of the offline discussion and related CRs

Deadline for companies’ comments: Wednesday 2025-05-21 20:00

Deadline for offline discussion summary (in R2-2504773):  Thursday 2025-05-22 08:00

# 2	Discussion

## 2.1	Stage 2 clarification

During RAN2#130, RAN2 reached the following agreement:

- Clarify in Stage 2 that only the same operation mode for anchor and non-anchor carrier is supported in NTN.

From previous discussions in RAN2#129bis and RAN2#130 as well as company’s contributions, the rapporteur understands there is consensus to introduce changes to the Table 5.5a-1 in TS 36.300 to clarify operation mode supported in NB-IoT NTN. A text proposal has been included in a separate draft CR. The intention of this change is to simplify complexity and follow the principle established in previous releases that a NB-IoT is not aware of whether it is operating within NR or LTE spectrum since co-existence is defined in terms of LTE cells. Thus, even though the LS refers to the introduction of inband within NR NTN, from RAN2 perspective, it is guardband and standalone the operation modes supported currently for NB-IoT NTN. A reference has been added in the NOTE to RAN4 specifications to help the implementer differentiate which modes are really supported when NB-IoT operates within LTE and NR spectrum.

Companies are invited to provide comments on the specification change proposal in the separate CR file.

## 2.2	Introduction of a new UE capability

### 2.1	Capability with signalling

RAN4 has introduced in Release 18 the support of inband operation mode. However, in Release 17, only standalone operation mode was supported. During the meeting, it was raised the scenario where the network releases the UE to be re-directed to a different NB-IoT carrier frequency. In this case, it is beneficial that the network knows the operation mode(s) (standalone or/and inband) supported by the UE. Note that the support of standalone is mandatory. This allows to avoid re-directing the UE to carrier frequencies with an operating mode not supported by the UE. Therefore, it was proposed to introduce a new UE capability with signalling to indicate the support of inband operation mode within NR NTN. Companies are asked to comment on the necessity of this new UE capability and are invited to propose a potential description.

Introduce a new UE capability with signalling to indicate the support of in-band in NB-IoT NTN within NR NTN and facilitate release with re-direction

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | This is a non-essential enhancement. Re-direction itself is an optional feature. Without introducing capability, it can be up to NW implementation whether to re-direct from SA to inband. There is no big issue even without re-direction. In RAN4 LS, they clearly indicate there is no RAN2 impact expected. So we don’t think RAN2 should begin to discuss further enhancements. And we agree with what Rapp says in 2.2, which also applies to this question. |
| Qualcomm | Yes | This is different from TN. The operation currently can work only for NR NTN inband. The in-band indication in MIB means there will CRS puncturing etc. which is not needed with NR. the guardband operation in NTN == Always NR NTN in-band operation The new guardband operation can support carrier location both inband and guardband of NR NTN carrier. Therefore, this guardband operation in NTN is a very special operation case and to avoid any further issue in future for in-band or guardband operation, it is better to have UE capability. In addition, for redirection purpose, UE capability signaling is useful. |
| MTK | Yes | According to 36.102, In Rel-18, NTN NB-IoT UEs may optionally support in-band operation with NR NTN. We can see this as an optional feature. |
| ZTE | No | It’s unclear for us on the usage for such UE capability (with signalling). Operation mode setting is in MIB which is very fundamental. If a NB-IoT NTN UE can camp on and access to a cell which is deployed on an “in-band” NR NTN frequency band, the NW can naturally know this UE has the corresponding special capability. Is there any assumption that this capability may be mainly needed by other R18 cell which is NOT deployed on “in-band” NR NTN frequency band? Then based on such UE capability, the cell can re-direct such UE to another cell deployed on “in-band” NR NTN frequency band? Is this the possible case? We tend to think no. |
| vivo | No | Out of the discussion scope. |
| Nokia | No | This field is relevant for cell selection and camping. We don’t see need to have this signalled to network for connected mode operation. The redirection can still work without this knowledge also. |
| Ericsson | Yes | Even though it is out of scope of the current discussion, we understand that RAN4 may be considering to make in-band operation mode mandatory in future releases. Thus, forward compatibility would benefit from a UE capability. |

Proposal 1	A capability with signalling is not introduced for NB-IoT NTN in-band operation mode within NR NTN (4/7).

### 2.2	Capability without signalling

In RAN2#129bis, the following was agreed:

- We wait for RAN4 indication whether a new capability is needed

In the rapporteur’s understanding, RAN4 has already closed this TEI enhancement without determining the need for a new UE capability without signalling. Therefore, following our previous RAN2 agreement, it can be concluded that a UE capability without signalling does not need to be introduced.

## 2.3	Further RAN2 impact

In addition to the previous two aspects (stage 2 and UE capability), there was an additional comment on the potential impact in RAN2 specification. One company suggested that RAN2 should reflect in MIB-NB description (TS 36.331) that the operation mode within NTN require the network to broadcast guardand instead of inband operation mode. From rapporteur perspective, this fact is transparent to the UE which does not understand whether it operates within LTE or NR spectrum. Companies are invited to provide additional comments.

Describe in MIB-NB in TS 36.331 that the operation mode guardband in NTN refers to inband when deployed within NR spectrum.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | As indicated in the LS, this is already captured in RAN4 spec. No need for RAN2 change. |
| Qualcomm | Yes | A general note that mixed mode operation is not supported is needed. A we explained, in-band singlaing in MIB does not work so its better to add clarification with adding (note) inside table. |
| MTK | Yes | A new chapter for in-band operation with NR NTN has been added to TS 36.102. At least one reference is needed. Additionally, we think there are some differences between the guard-band in TN and in-band operation with NR NTN. The in-band operation with NR NTN is optionally supported, while the guard-band operation in TN is mandatory. For TN guard-band, the DL EARFCN formulation is the same as that of standalone and in-band mode. A UE does not need separate calculations for each operation mode. However, for NTN in-band operation with NR NTN, according to 36.102 5.4B.2 the DL EARFCN formulation is different than that of standalone. The UE needs to be aware of being in NTN in-band operation mode to use a different formulation from that of standalone. This difference will also lead to a potential issue, specifically for the neighbor cell EARFCN in the SIB5, how does the UE determine the operation mode of neighbor cell, so that it can select the correct EARFCN formulation to obtain the frequency? This potential issue may also have impacts on different UE behaviors. For reference, the DL EARFCN formulation of standalone in NTN is: FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) and the DL EARFCN formulation of in-band operation with NR NTN is: FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1) Therefore, we think it is best not to consider transparency between the guard-band in TN and in-band operation with NR NTN. We propose the following change: 36.331 6.7.2 operationModeInfo Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23]. Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports. Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id. guardband indicates a guard-band deployment in TN (see TS 36.101 [42]) and in-band operation with NR NTN in NTN (see TS 36.102 [113]). standalone indicates a standalone deployment. |
| ZTE | No | RAN4 only give us the clue that NB-IoT NTN is allowed to be deployed on a frequency which is inside NR NTN frequency band and in this case, the operation mode needs to be set to “guardband”. But there is no any information on whether NB-IoT NTN can be deployed on the real guardband of NR NTN frequency band and how to set operation mode for this case. Shortly to say, we only know guardband can be used in some special “in-band” case, but we RAN2 cannot conclude that “guardband” setting in NTN will always means “in-band”. So we disagree to introduce any unclear restriction to stage-3 configuration. |
| vivo | No | RAN4 will further specify the actual in-band details in Rel-19. We propose not to add clarifications on "inband" for Rel-18 in RAN2. Otherwise, additional clarifications in RAN2 would still be required later. |
| Nokia | No | As per RAN4 conclusion, the parameters of guard-band mode is used to convey information related to NR NB-IoT inband which is also needed only for some cases. From RAN2 spec if we clarify guard-band mode is allowed from Rel-18 it is sufficient for the above. The details can be specified in RAN4 spec. For the above stage-2 change is sufficient. |
| Ericsson | No | From RAN2 and RAN1 perspective, the fact that UE is operating in NR or LTE is transparent. The indication in MIB to configure the UE with a set of parameters and timing relationships. Regarding MTK’s comment, the DL EARFCN is defined in RAN4 specs and depends on the operation mode set in MIB-NB with the value guardband, not inband. Thus, we lack to see any RAN2 impact. |

Proposal 2	RAN2 does not introduce Stage 3 changes for NB-IoT NTN in-band operation mode within NR NTN (5/7).

# 4	Conclusion

Proposal 1	A capability with signalling is not introduced for NB-IoT NTN in-band operation mode within NR NTN (4/7).

Proposal 2	RAN2 does not introduce Stage 3 changes for NB-IoT NTN in-band operation mode within NR NTN (5/7).