# 3GPP TSG-RAN WG2 Meeting #130															R2-2504799

# St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2075 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TA validation for SRS transmission in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - First change: In RAN1#112, an area-specific RSRP change threshold for TA validation is introduced: Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold. According to the above agreement, TS 38.331 defines inactivePosSRS-ValidityAreaRSRP for TA validation as follows: AreaValidityTA-Config-r18 ::= SEQUENCE { inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, inactivePosSRS-ValidityAreaRSRP-r18 RSRP-ChangeThreshold-r17 OPTIONAL, -- Need R autonomousTA-AdjustmentEnabled-r18 ENUMERATED {true} OPTIONAL -- Need R } inactivePosSRS-ValidityAreaRSRP RSRP threshold for the increase/decrease of RSRP for validity area time alignment validation as specified in TS 38.321 [3].. In the definition of inactivePosSRS-ValidityAreaRSRP in TS 38.331, a reference to TS 38.321 exists (as also highlighted in yellow above). However, current wording of TS 38.321 does not capture the referred behaviour for the inactivePosSRS-ValidityAreaRSRP for TA validation. Therefore, currently there is a misalignment between TS 38.331 and TS 38.321 regarding inactivePosSRS-ValidityAreaRSRP for TA validation and the usage of inactivePosSRS-ValidityAreaRSRP for TA validation is unclearly specified. - Second change: Two different terminologies are used throughout TS 38.321 when referring to the configured validity area for SRS positioning: in §5.2 of TS 38.321, it is refered as “SRS positioning validity area” while in §5.26.2 of TS 38.321, it is refered as “positioning validity area”. It is preferable to use a consistent name to avoid confusions, for example “SRS positioning validity area”. |
| Summary of change: |  | - First change [in §5.26.2]: - Add inactivePosSRS-ValidityAreaRSRP for TA validation. - Add the definition of inactivePosSRS-ValidityAreaRSRP as part of parameters that RRC configures for validation for SRS transmission in RRC_INACTIVE. - Second change [in §5.26.2]: Update the reference of “positioning validity area” to “SRS positioning validity area”. Impact analysis Impacted functionality: LPHAP Impacted 5G architecture options: NR SA Inter-operability: - If the network is implemented according to the CR and the UE is not, if the UE is configured with validity area for SRS positioning and if inactivePosSRS-ValidityAreaRSRP is configured, the UE mistakenly determine the TA validity and UE’s operation for positioning SRS transmission may fail. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the UE is configured with validity area for SRS positioning and if inactivePosSRS-ValidityAreaRSRP is configured, the UE mistakenly determine the TA is valid according to TS 38.321, as the impact of the inactivePosSRS-ValidityAreaRSRP for TA validation is missing. Ambiguity of the terminologies used in TS 38.321 when referring to the positioning validity area. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.26.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.26	Positioning SRS transmission in RRC_INACTIVE

### 5.26.1	General

Periodic and semi-persistent Positioning SRS with or without positioning SRS bandwidth aggregation can be configured for Positioning SRS transmission in RRC_INACTIVE.

SRS for positioning Tx frequency hopping as in clause 5.32 can also be configured for Positioning SRS transmission in RRC_INACTIVE.

The MAC entity shall,

1>	if the TA of the configured Positioning SRS is valid according to clause 5.26.2, and the conditions for positioning SRS transmission in clause 7.3.1 of TS 38.213 [6] and clause 6.2.1.4 of TS 38.214 [7] are satisfied:

2>	if the UE is configured with UTW and the UplinkTimeWindowTimer is running according to clause 5.32; or

2>	if the UE is not configured with UTW:

3>	instruct to the lower layer according to TS 38.214 [7] to transmit Periodic or Semi-Persistent Positioning SRS that is activated according to clause 5.18.17 or clause 5.18.37.

### 5.26.2	TA validation for SRS transmission in RRC_INACTIVE

RRC configures the following parameters for validation for SRS transmission in RRC_INACTIVE:

-	inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation;

-	inactivePosSRS-ValidityAreaRSRP: RSRP threshold for the increase/decrease of RSRP for time alignment validation when SRS positioning validity area is configured.

The MAC entity shall:

1>	if the UE receives configuration for SRS transmission in RRC_INACTIVE:

2>	store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5].

1>	else if the UE is configured with SRS transmission in RRC_INACTIVE:

2>	if Timing Advance Command MAC CE is received as in clause 5.2, or;

2>	if Timing Advance Command or Absolute Timing Advance Command is received for Random Access procedure that is successfully completed:

3>	update the stored the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference.

2>	if the UE is configured Positioning with SRS with validity area and the upper layer indicates the MAC to update the stored RSRP:

3>	update the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5].

The MAC entity shall consider the TA to be valid when the following conditions are fulfilled:

1>	compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference of the camped cell as specified in TS 38.331 [5] has not increased/decreased by more than inactivePosSRS-RSRP-ChangeThreshold, if configured, or inactivePosSRS-ValidityAreaRSRP, if configured, when SRS positioning validity area is configured; and

1>	inactivePosSRS-TimeAlignmentTimer is running or inactivePosSRS-ValidityAreaTAT is running when SRS positioning validity area is configured.

END OF CHANGE