3GPP TSG-RAN WG2 Meeting #129bis	R2-2504902

# Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2081 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung, Qualcomm, Nokia, China Telecom, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PUCCH spatial relation Activation/deactivation for multiple TRP PUCCH Repetition MAC CE is introduced for Rel-17 multiple TRP PUCCH Repetition, among the field descriptions of the MAC CE, the description of the field Spatial Relation Info IDi is shown as below: - Spatial Relation Info IDi: This field contains PUCCH-SpatialRelationInfoId–r16 where PUCCH-SpatialRelationInfoId is the identifier of the PUCCH Spatial Relation Info in PUCCH-Config in which the PUCCH Resource ID is configured, as specified in TS 38.331 [5], where i is the index of the activated spatial relation info ID. The length of the field is 6 bits; First of all, in RRC specification the PUCCH-SpatialRelationInfoId-r16 is only used for presenting in the toReleaselist but not associated with any configuration of PUCCH-spatialRelationInfo or PUCCH-spatialRelationInfoExt Which means, a PUCCH-SpatialRelationInfoId-r16 quote in the field description of Spatial Relation Info IDi cannot be associated with any PUCCH-SatialRelationInfo configuration. Secondly, the spatial relation Info ID field only have 6 bits that indicate the value range only from 0 to 63, however, the value range of PUCCH-SpatialRelationInfo-Id-r16 is from 1 to 64, which means the PUCCH-SpatialRelationInfo =64 cannot be indicated in the MAC CE, please see below: PUCCH-SpatialRelationInfoId-r16 ::= INTEGER (1..maxNrofSpatialRelationInfos-r16) maxNrofSpatialRelationInfos-r16 INTEGER ::= 64 Above all, the field shall indicate the PUCCH-SpatialRelationInfoId –1 instead of PUCCH-SpatialRelationInfoId-r16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the suffix ‘-r16’ of PUCCH-SpatialRelationInfoId-r16 - Clarify the Spatial Relation Info IDi field of the PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH Repetition MAC CE contains PUCCH-SpatialRelationInfoId –1 Impact Analysis Impacted 5G architecture options: NR SA,NR-DC Impacted functionality: Multiple PUCCH Repetition Inter-operability: - If the network is implemented according to the CR and the UE is not, UE may not activate the correct spatial relationship for a PUCCH resource that is actually requested by NW.. - If the UE is implemented according to the CR and the NW is not, NW may send the MAC CE with a wrong value of spatial relation information for UE to activate. This CR is mandatory to implement for UEs and networks supporting PUCCH spatial Relation activation/deactivation for mTRP PUCCH repetition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not approved: 1: UE may not find the indicated pucch spatial relationship info configuration via the field of spatial relation info Idi in the MAC CE. 1: NW cannot indicate UE to activate the pucch spatial relation info configuration with Id=64. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.45 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.3.45	PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE

The PUCCH Spatial Relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	C: This field indicates whether single or two spatial relation info(s) is activated for the indicated PUCCH Resource ID. If this field is set to 1, octet containing the second spatial relation info for the indicated PUCCH Resource is present. If this field is set to 0, octet containing the second spatial relation info for the indicated PUCCH Resource is not present;

-	PUCCH Resource ID: This field contains an identifier of the PUCCH resource ID identified by PUCCH-ResourceId as specified in TS 38.331 [5], which is to be activated with a spatial relations indicated by Spatial Relation Info IDi fields in the subsequent octet(s). The length of the field is 7 bits. If the indicated PUCCH Resource ID is included in a PUCCH Resource Group (configured via resourceGroupToAddModList as specified in TS 38.331 [5]) of the indicated UL BWP, no other PUCCH Resources within the same PUCCH Resource group are indicated in the MAC CE, and this MAC CE applies to all the PUCCH Resources in the PUCCH Resource group;

-	Spatial Relation Info IDi: This field contains PUCCH-SpatialRelationInfoId – 1 where PUCCH-SpatialRelationInfoId is the identifier of the PUCCH Spatial Relation Info in PUCCH-Config in which the PUCCH Resource ID is configured, as specified in TS 38.331 [5], where i is the index of the activated spatial relation info ID. The length of the field is 6 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.45-1: PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE