3GPP TSG RAN WG2#130	R2-2504964

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121	R1-2504934

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	LS on NR-NTN TP for TS 38.300

Release:	Rel-19

Work Item:	NR_NTN_Ph3

Agenda Item:	9.11

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Mohamed EL JAAFARI

E-mail Address:	mohamed.el-jaafari@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

As part of the WI on NR NTN enhancement phase 3, RAN1 discussed the following text proposal for inclusion in 3GPP TS 38.300.

<Unchanged text is omitted>

## 16.14.9 Support for NR NTN coverage enhancements

<Unchanged text is omitted>

To improve NR downlink coverage in NTN, the following enhancements are supported for FR1-NTN:

- Repetition of PDCCH CSS.

- Repetitions of PDSCH carrying Msg4.

- Repetitions within 20 ms duration of PDSCH carrying SIB1.

To extend the NR downlink coverage in NTN, for FR1-NTN and FR2-NTN: extended periodicity of 160 ms (apart from the existing 20ms value) for the half frames with SS/PBCH blocks is assumed by UE during initial cell selection.

<Unchanged text is omitted>

## 16.14.11 Support for NR NTN uplink capacity enhancement

To improve NR-NTN uplink capacity in NTN, the following enhancements are supported for PUSCH transmission:

- Inter-slot Orthogonal Cover Codes (OCC) with OCC length 2 to multiplex up to two UEs

- Inter-slot OCC with OCC length 4 to multiplex up to four UEs

## 16.14.12 Support of Half-Duplex UE in NR NTN

To support Reduced Capability (RedCap) UEs and enhanced RedCap (eRedCap) UEs in NR NTN operating in FR1-NTN bands, enhancements to address issues caused by Timing Advance (TA) mismatch for Half-Duplex UE in paired spectrum are specified. This mismatch occurs between the actual TA used by the UE and the assumed TA for the UE at the gNB. These enhancements are specifically targeted at mitigating the issues in HD collision cases at the UE side, including scenarios where semi-statically configured downlink reception collides with semi-statically configured uplink transmission, and where dynamically scheduled downlink reception collides with dynamically scheduled uplink transmission.

---------- TEXT PROPOSAL END ---------

# 2. Actions:

Action to TSG RAN WG2:

RAN1 respectfully asks RAN2 to consider the above TP in NR-NTN Stg2 Running CR for TS 38.300 on NTN enhancement phase 3.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #122	25 Aug – 29 Aug 2025	Bengaluru, IN

TSG-RAN WG1 Meeting #122-bis	13 Oct – 17 Oct 2025	Prague, CZ