3GPP TSG-RAN WG2 Meeting #130	DRAFT_R2-2504675

St. Julian’s, Malta, May 19th – 23rd, 2025

Source: 	Session chair (Huawei)

Title:	Report from session on R19 XR and LTE Broadcast

[AT130][500][XR] Organizational – Session on R19 XR and LTE Broadcast (Session chair)

Scope:

      - Share plans and list of ongoing email discussions for the session

      - Share meeting notes and agreements for review and endorsement

[AT130][501][XR] Reply LS on TTNB (QCM)

Scope: Find proper wording to express the RAN2 agreement in the LS

Intended outcome: Agreeable LS in R2-2504812

Deadline:  Friday 2025-05-23, 08:00

[AT130][502][XR] Reply to SA4 LS on RTP retransmission (Nokia)

Scope: Discuss the reply LS considering the online discussion

Intended outcome: Agreeable reply LS in R2-2504813

Deadline:  CB session on Thursday

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and agreed as individual CRs

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

For each R19 feature, 1 additional tdoc on top of the limit is allowed for a primary co-sourcing company for co-sourced contribution with 4 or more companies.

Open issues

- CR Rapporteurs (as indicated in email discussion scope) are expected to provide open issue list

- Please refer to RAN2 chair guidance document in [POST129bis][001][Organizational] Open issue list

- Companies should focus on addressing these open issues first and clearly indicate the open issue number they are addressing in their section and proposal, e.g. Proposal x: (RRC-1) Agree to bla bla

- Companies can discuss UE capabilities in their topic-specific Tdocs

Tdoc request/submission for RAN2#130 deadlines:

- Tdoc Submission deadline: May. 9th, 1000 UTC

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 7

#### 7.0.2.16	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

IPA XR

R2-2503842	Correction to UE capability for retx-less CG	Huawei, HiSIlicon, Apple, Futurewei, Qualcomm	CR	Rel-18	38.306	18.5.0	1279	-	F	NR_XR_enh-Core

CR is agreed

R2-2503843	Correction to RRC for retx-less CG	Huawei, HiSIlicon, Apple, Futurewei, Qualcomm	CR	Rel-18	38.331	18.5.1	5348	-	F	NR_XR_enh-Core

CR is agreed

Other

R2-2504597	Discussion on DSR cancellation	Ericsson, Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

The CR is not pursued

- Nokia disagrees with the observations. Prefer not to change behaviour because there are no issues with the existing text.

- LGE thinks that intention was not to include both all data and DSR.

- NEC, Samsung agrees with Nokia and LGE.

- Samsung sees no contradiction in the current text.

- Xiaomi see this as an optimization and it was discussed already.

- MTK thought it was useful and sees some contradiction in the current text.

- QCM thinks such relaxation in the UE behaviour is beneficial. Zero DSR can be used as an indication to the network. It requires the UE to perform LCP twice in case it turns out all data can be in the MAC PDU.

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-250107)

Time budget: 2 TU

Tdoc Limitation: 5 tdocs

General guidelines:

- Contributions should focus on the open issues identified in the post-meeting CR review discussions

- Companies can discuss UE capabilities in their topic-specific Tdocs

### 8.7.1	Organizational

LS, rapporteur input, workplan, running CRs, open issues lists etc.Including outcome of [POST129bis][507][XR] Incoming LS(es) from SA4 (Qualcomm)

Incoming LSes

R2-2503323	Reply LS on uplink rate control (R3-252491; contact: Meta)	RAN3	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN2, SA2

Noted

R2-2503328	Reply LS on UE assistance information (R4-2504972; contact: Nokia)	RAN4	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN1, RAN2

Noted

RAN2 assumes granularity of indication should be decided by RAN4. If no input from RAN4 is received, then we will decide this in RAN2 next meeting.

- Nokia thinks that it is a bit unclear whether granularity is supposed to be discussed in RAN2 or in RAN4.

- QCM thinks granularity can be discussed in RAN2, because of time constraints. There is enough expertise in RAN2.

- Huawei indicates that RAN4 is discussing this and this is R4 feature. Lenovo, Ericsson agrees.

- ZTE thinks that some agreement is needed, but it seems there is not much input on this in RAN4.

R2-2503336	LS reply on uplink rate control (S2-2504252; contact: vivo)	SA2	LS in	Rel-19	XRM_Ph2, NR_XR_Ph3-Core	To:RAN2	Cc:RAN3, SA4

Noted

R2-2503339	LS on the accuracy of PDU Set size and data burst size indication (S4-250736; contact: Qualcomm)	SA4	LS in	Rel-19	5G_RTP_Ph2	To:RAN2	Cc:SA2, RAN3

Noted

R2-2503340	LS on Indicating Time to the Next Data Burst (TTNB) (S4-250737; contact: Qualcomm)	SA4	LS in	Rel-19	5G_RTP_Ph2	To:RAN2	Cc:SA2, RAN3

Noted

Draft reply LSes

R2-2503362	Draft reply LS to SA4 on accuracy of PDU Set size and data burst size	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Change the last sentence in bullet 1 to: “An accuracy of 5% is sufficient.”

With this change the LS is approved in R2-2504811

R2-2503363	Draft reply LS to SA4 on accuracy of TTNB	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Most of the contents of the LS are agreeable

We will clarify that in case 0.125ms is infeasible, it is acceptable for RAN2 to have less accurate indication.

Exact wording to be discussed offline

[AT130][501][XR] Reply LS on TTNB (QCM)

Scope: Find proper wording to express the RAN2 agreement in the LS

Intended outcome: Agreeable LS in R2-2504812

Deadline:  Friday 2025-05-23, 08:00

DISCUSSION on two LSes above:

- QCM indicates there were some comments from Ericsson and Futurewei.

- Huawei thinks that for scheduling purposes accuracy of 2% would be better.

- Futurewei thinks it is important to note that accuracy should not be increased at an expense of packet delay. We should make sure that there is balance between accuracy and additional delay.

- Lenovo thinks that there are some inaccuracies for BSR as well, so we do not have to be too detailed.

- Nokia clarifies that so far it seems it was not accurate, so the point from SA4 is to get some order of magnitude, not exact number.

- Ericsson thinks that we can simply say 5% is sufficient and we do not need more accuracy.

- LGE thinks just indicating no more than 5% is OK, we do not have to be very detailed.

- Huawei reminds that CT will use this to specify the codepoints.

- FTW thinks the unit in SA4 specs for TTNB is in ms, so now we are asking them to do better than that. Xiaomi shares the concern, not sure if we need to be that accurate. Ericsson agrees.

R2-2504609	Discussion on SA2 LS on UL rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Rate control applies for both GBR and non-GBR bearers.

Proposal 2	When gNB sends the rate control to UE, it could inform SMF/UPF in order to coordinate with CN for congestion handling.

- OPPO agrees with proposal 1. P2 is not needed, we can focus on questions SA2 asked us.

- LGE, Huawei, Samsung, Nokia, ZTE agrees we should focus on P1.

- Vivo thinks if we indicated something to SA2, it could help them.

- QCM agrees that we may focus on the question but thinks some coordination between L4S and XR rate control mechanisms is needed.

- Ericsson agrees we should focus on P1 and asks whether we will capture sth in stage-2 regarding this.

From RAN2 perspective rate control applies for both GBR and non-GBR bearers.

[AT130][503][XR] Reply LS to SA2 on GBR/non-GBR (vivo)

Scope: Agree reply LS

Intended outcome: Agreeable LS

Deadline:  Friday 2025-05-23, 08:00

Rapporteur input

R2-2503563	Rapporteur Inputs	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Running CRs

R2-2503360	Introduction to R19 XR enhancements	Qualcomm Incorporated	draftCR	Rel-19	38.321	18.5.0	NR_XR_Ph3-Core

R2-2503564	Draft Stage 2 CR for XR	Nokia (Rapporteur)	draftCR	Rel-19	38.300	18.5.0	B	NR_XR_Ph3-Core

R2-2503620	RLC running CR for R19 XR	vivo	draftCR	Rel-19	38.322	18.2.0	B	NR_XR_Ph3-Core

R2-2503696	PDCP running CR for R19 XR	LG Electronics Inc. (Rapporteur)	draftCR	Rel-19	38.323	18.5.0	NR_XR_Ph3-Core

R2-2503787	Running RRC CR for R19 XR	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_XR_Ph3-Core

R2-2503436	Draft 38.306 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.306	18.5.0	B	NR_XR_Ph3-Core

R2-2503437	Draft 38.331 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.331	18.5.1	B	NR_XR_Ph3-Core

The CRs above are endorsed as baseline for further updates after this meeting

In MAC CR, FFS whether some changes are needed due to using single/multiple entry DSR MAC CE terminology

Open issues lists

R2-2503361	List of open issues in MAC	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Noted

- CR Rapporteur: whether to use multiple entry DSR MAC CE and enhanced DSR MAC CE needs to be decided. There are 3 options:

  - Single and multiple entry DSR MAC CE

  - Enhanced DSR MAC CE and legacy stays as DSR MAC CE

  - We have a separate section for the new DSR

- Ofinno thinks that sometimes new DSR will only have one entry, so multiple entry DSR MAC CE is not appropriate. Option 2 is preferred.

- Apple would like to rule out option 3. Option 2 is preferred.

- LGE thinks it’s important to differentiate between legacy and new DSR. Otherwise

- Samsung thinks we should not spend too much time on this

- Ofinno indicates that option 1 impacts R18 DSR. Sharp indicates it is just a name.

We go with option 1, i.e. Single and multiple entry DSR MAC CE

R2-2503438	Open issues of Rel-19 XR UE capabilities	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

An optional UE capability with signalling (e.g. delayStatusReportNonDelayReportingData-r19) is introduced to indicate the support of including non-delay-reporting data ahead of delay-reporting data in the buffer size calculation for enhanced delay status report. A UE supporting this feature shall also indicate support of enhanced delay status report (enhancedDelayStatusReport-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

An optional UE capability with signalling (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message (in UL Rate Control MAC CE) from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE (ul-RateControl-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

R2-2503565	Stage 2 Open Issues	Nokia (Rapporteur)	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1 (Stage2-1): send an LS to SA2 to clarify PDU set handling when the gNB is not provided with any PDU Set QoS Parameters from the SMF but still being provided with PDU Set information from the UPF.

- Nokia clarifies that SA2 agreed to update their specs, so there should be no ambiguity any more and we can close this open issue.

- FTW indicates that we should also update our stage-2 specs.

We will update 38.300 in line with SA2 updated specs

R2-2504649	Draft LS to SA2 on gNB PDU Set based handling without QoS parameters	Futurewei	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2	Cc:RAN3

R2-2503697	Discussion of [POST129bis][504][XR] PDCP running CR	LG Electronics Inc. (Rapporteur)	report	Rel-19	NR_XR_Ph3-Core

Noted

R2-2503767	Discussion summary and list of RLC open issue for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Open issue RLC-1 (essential): Terminology for avoiding unnecessary retransmission, e.g. “obsolete”, or “outdated”, or “discard”

Proposal 1: (10/13) Use the term of “discard” for avoiding unnecessary retransmission, e.g. “stopReTxDiscardedSDU”, “t-RxDiscard”.

Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

Rapporteur: The latest running CR will reflect this proposal. Companies have concern/different views on it could provide your further comments during the meeting.

Open issue RLC-2 (not essential, but important): whether further changes are needed for SR triggered by t-RxDiscard expires.

Rapporteur suggests to follow the majority, i.e. there is no need on further change for SR triggered by t-RxDiscard expires.

Rapporteur: The latest running CR will reflect this suggestion. Companies have concern/different views on it could provide your further comments during the meeting.

Open issue RLC-3 (essential): whether use the terminology of “autonomous retransmission” or others.

Proposal 2: (7/13) The term “remaining time based retransmission” is used for autonomous retransmission in RLC.

Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

Rapporteur: The latest running CR will reflect this proposal. Companies have concern/different views on it could provide your further comments during the meeting.

Open issue RLC-4 (essential): whether merge the autonomous retransmission procedure in clause 5.x into 5.3.2 or capture it separately.

Rapporteur suggests to follow the clear majority, i.e. merge autonomous retransmission procedure in 5.x into section 5.3.2 “retransmission”.

Rapporteur: The latest running CR will reflect this suggestion. Companies have concern/different views on it could provide your further comments during the meeting.

- RLC rapporteur clarifies that open issues RLC-2 and RLC-4 are already closed so there is no need to discuss them.

Use the term of “discard” for avoiding unnecessary retransmission, e.g. “stopReTxDiscardedSDU”, “t-RxDiscard”.

- Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

The term “remaining time based retransmission” is used for autonomous retransmission in RLC.

- Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

R2-2503788	Summary of [POST129bis][503][XR] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

### 8.7.2	Multi-modality support

No contributions are expected for this AI

### 8.7.3	RRM measurement gaps/restrictions related enhancements

Focus on remaining details of UAI as agreed by RAN4 (see LS in R4-2504972), e.g. when to trigger UAI, need of prohibit timer etc.

UAI triggering and configuration (RRC-5, RRC-6)

R2-2504121	Discussion on UAI for MG skipping	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received or when the assistance information changes since the last report.

Proposal 3: (RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

Proposal 4: (RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio.

Proposal 5: RAN2 to wait for RAN4 progress on the granularity (e.g., per FR or per UE) for the recommended gap cancellation ratio.

UE capability (UE capability-03)

R2-2503653	Discussion on RRM measurement gaps enhancements of XR traffic	Xiaomi Communications	discussion

Proposal 5	RAN2 defines a UE capability for reporting the ratio of gap occasions. And its prerequisite condition is that it supports enabling TX/RX during measurement gap scheduling restrictions by DCI.

R2-2503364	Discussion on measurement gap enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503425	Discussion on RRM Measurement Gaps Restrictions Related Enhancements	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503494	Discussion on Measurement Gap enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503508	UAI and Measurement Gap Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503521	RRM Measurement Gaps Enhancements for XR	Ofinno	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503621	Discussion on RRM measurement gaps enhancements	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503792	Discussion on RRM measurement gaps/restrictions related enhancements	China Telecom	discussion	Rel-19

R2-2503892	Enabling TX/RX for XR during measurement gaps/restrictions	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503971	Measurement gap enhancements for XR	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2504341	Discussion on UE Assistance Information (UAI) for Measurement Gaps	Ericsson	discussion	Rel-19

R2-2504409	RRM measurement gaps/restrictions related enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504436	Discussion on UAI for Measurement Gap Enhancements	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504442	Discussion on RAN4 LS on UE assistance information	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.4	Scheduling enhancements

#### 8.7.4.1	LCP enhancements

Further details of handling of the additional priority for LCH, e.g. configuration details, impact on Bj etc.

SR priority [MAC-01]

R2-2503690	TP for using additional LCP priority for SR priority determination	Huawei, HiSilicon, ZTE Corporation, Samsung, Sharp, NEC, CATT 	discussion	Rel-19	NR_XR_Ph3-Core

Proposed TP: [MAC-01]

When the MAC entity is configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold at the time of the SR transmission.

R2-2503361	List of open issues in MAC	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

[MAC-01] SR priority adjustment

The current agreement is to evaluate the potential spec impact of SR priority adjustment and then decide whether to adopt it (i.e. only if it is simple enough to capture).  Companies can use the following TP as the baseline for further discussion:

For the MAC entity configured with lch-basedPrioritization, the MAC entity shall for a pending SR:

1>	if the SR is triggered by a logical channel configured with additionalPriority; and

- if the smallest remaining value of the running PDCP discardTimers among the data available for transmission in this logical channel, evaluated at the time of the first symbol of the next PUCCH resource for the SR transmission, is below or at priorityAdjustmentThreshold configured for the logical channel:

2> consider additionalPriority as the priority of the SR transmission.

R2-2503452	Discussion on LCH priority adjustment for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

DISCUSSION:

- OPPO is concerned that we also need to address SR retransmission, so the procedure is more complex than proposed.

- LGE is concerned that SR transmission suffers from RTT anyway, so it is not that useful to prioritize it.

- Xiaomi is still not convinced by the gains. Prioritization of SR was disucssed in R18 already and nothing changed. Agrees that fallback to original priority may be complex for SR.

- Samsung thinks that main concerns were about complexity but the TPs are as simple as they can get.

- MTK sees this a corner case optimization. If you need to send SR, it means you are already delayed.

- Nokia thinks we can skip this optimization.\

- Samsung and ZTE agrees this is an optimization, but the whole WI is about optimizations. TP is very simple.

- IDT indicates that for BSR we did not do anything so for SR we should follow this principle.

[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

Impact of congestion on priority adjustment (MAC-02)

R2-2503698	Discussions on Delay-based LCP Enhancements	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: For the potential impacts of PSI-based discarding, RAN2 can discuss the following options:

•	Option 1: When PSI-based discarding is activated a DRB, priority adjustment for the corresponding LCH is disabled.

•	Option 2: When PSI-based discarding is activated a DRB, priority adjustment is applied only if there is at least one more important packet in the buffer has a remaining time smaller than the threshold.

R2-2503365	Discussion on LCP enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2.	[MAC-02] If an LCH with priority p is in congestion (i.e. PSI based discard has been activated), then no other LCHs are allowed to adjust their priorities to priority p, even if they meet the priority adjustment criteria.

Proposal 3. 	[MAC-02] If an LCH is in congestion (i.e. PSI based discard has been activated), PDUs with low importance are not considered for priority adjustment.

Priority fallback with consideration of PDU Set integrated handling (MAC-13)

R2-2504373	Further consideration on LCP enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5a: In the 1st round of LCP, PSIHI has no specification impact on LCH priority determination as PDU set and PDCP SDU use the same remaining time threshold.

Proposal 5b: In the 2nd round of LCP, if pdu-SetDiscard is configured, the priority of LCH should fall back to the original priority if there is neither PDCP SDU nor PDCP SDU within a PDU set whose remaining is less than the threshold.

R2-2503426	Consideration on LCP Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5:  (MAC-13) Do not consider priority adjustment on PDU set level in enhanced LCP procedure.

Bj enhancement

R2-2504596	Discussion on Bj enhancement	Ericsson, InterDigital, LG Electronics, Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Bj enhancement shall be incorporated in the Rel19 LCP enhancements to achieve the most flexible and high performing LCP solution.

Proposal 2	Implement the TP provided in Appendix 1.

R2-2503522	LCP Enhancements for XR	Ofinno	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Confirm the working assumption “No Bj enhancement is introduced” as an agreement.

R2-2503475	Remaining issues on LCP enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503509	Open Issues on LCP Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503622	Discussion on Bj adjustments for LCH priority-adjusted data transmission	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503654	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	discussion

R2-2503793	Discussion on Remaining Issues of LCP Enhancements	China Telecom	discussion	Rel-19

R2-2503883	Remaining issues on LCP enhancements	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503891	Open issues on Intra-UE prioritization/LCP enhancements	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503957	Discussion on enhanced LCP for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2503972	LCP enhancements for XR	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2504308	Remaining open issues on LCP in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504410	LCP Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504537	discussion on LCP enhancements for XR	ETRI	discussion	Rel-19

R2-2504608	Discussion on LCP enhancement	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504619	Finalising LCP design for XR Ph3	Samsung R&D Institute UK	discussion

#### 8.7.4.2	DSR enhancements

Further details of enhanced DSR configuration/procedure, data volume calculation etc.

Handling of Control PDUs and retransmission PDUs (PDCP-1, RLC-5)

R2-2503556	Discussions on DSR enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold.

Proposal 3: The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG.

R2-2503834	DSR enhancements for UL scheduling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: The additional delay critical data amount from control PDUs and re-transmission data is included into the smallest reported threshold in the new DSR MAC CE.

Threshold configuration for R19 DSR (RRC-7)

R2-2503973	DSR enhancements for XR	ZTE Corporation, Sanechips	discussion	Rel-19

Proposal 4(RRC-7): New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds, and not define new LC-ID for the new DSR MAC CE.

R2-2503510	Open issues on DSR Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 9	If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

DSR cancellation in DC configuration (MAC-03)

R2-2503366	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1. 	In DC, no enhancements are needed when a pending DSR is canceled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

Proposal 2.	[MAC-03] For Rel-19, define an optional UE capability, which is per band combination, for the support of canceling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 3.	[MAC-03] For UEs that support the capability defined in Proposal 1, network configures UE whether it shall cancel pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

R2-2503476	Remaining issues on DSR enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5. For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

Proposal 6. For Rel-19, cancel the DSR when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Capability dependency (Capability-1)

R2-2503453	Discussion on DSR enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	[UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

R2-2503655	Discussion on DSR enhancements of XR traffic	Xiaomi Communications	discussion

Proposal 6   A UE supporting Rel-19 enhance DRS shall also indicate support of delayStatusReport-r18.

R2-2503427	Consideration on DSR Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503623	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503699	On Data Volume Calculations for Rel-19 DSR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503794	Discussion on Remaining Issues of DSR Enhancements	China Telecom	discussion	Rel-19

R2-2503885	Remaining issues on DSR enhancements	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503912	Enhanced delay status reporting for XR	Lenovo	discussion	Rel-19

R2-2504113	Remaining open issues on DSR enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504374	Further consideration on DSR enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504411	DSR Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504435	Remaining Issues on DSR enhancements in Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504474	Discussion on DSR enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504518	Discussion on DSR enhancements for XR	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504574	Discussion on XR DSR enhancements	III	discussion	NR_XR_Ph3-Core

R2-2504598	Discussion on DSR enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.5	RLC enhancements

Further details of autonmous retransmission and enhanced polling mechanisms and unnecessary retransmission avoidance.

Timely retransmissions

Excessive polling and retransmissions due to polling (RLC-8, RLC-9)

R2-2503624	Discussion on RLC remaining issues for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: (RLC-8) Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold.

Proposal 4: (RLC-9) No additional conditions are needed for the polling enhancement.

DISCUSSION on P3:

- LGE thinks P3 is not needed. It does not matter if multiple polls are triggered and sent. One SR will be sent anyway. Supports P4.

- Vivo clarifies that after reviewing specification, they share understanding from LGE.

- LGE also thinks P3 causes issues in case of RLC segmentation.

- Xiaomi supported original P3 to limit the polling. IDT has similar concerns.

- LGE clarifies that the second poll is due to remaining time condition while the first one was due to legacy conditions. Thinks we do not have to do anything about this.

- Huawei agrees with LGE and prefers not to agree P3.

- Lenovo wonders whether we need to specify that enhanced polling will normally be triggered before autonomous retransmission.

(RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this

(RLC-9) No additional conditions are needed for the polling enhancement.

Autonomous Retransmission coexistence with ARQ (RLC-7)

R2-2503830	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4. Remaining-time based RLC retransmission does not increment RETX_COUNT, but a simple text is added in the RLC specification as in the following TP.

R2-2503600	Considerations on open issues for RLC enhancements	KDDI Corporation (TTC)	discussion

Proposal 4: RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

- DISCUSSION:

- Ofinno, Huawei supports KDDI proposal, it is easier to implement in the current specs.

- Samsung supports LGE because the retransmission is due to latency, not channel conditions.

- Huawei indicates it does not matter so much as there is only one auto reTx.

- QCM also supports LGE.

- Nokia agrees with Huawei, i.e. we can just follow current specifications.

- LGE emphasizes that this reTx is without SR, so it makes more technical sense not to increment. But is OK to go with KDDI proposal.

RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

PDU set impact on timely retransmissions (PDCP-2)

R2-2503830	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6. If pdu-SetDiscard is configured, remaining-time based RLC retransmission and polling are triggered for all SDUs in the PDU set.

Proposal 7. If PSI based SDU discard is activated, remaining-time based RLC retransmission is not triggered.

R2-2503439	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP PDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

DISCUSSION on PDCP-2:

- Ericsson share understanding with Xiaomi.

- Ofinno also agrees with Xiaomi, because each PDU can trigger this separately, we do not have to rely on other PDUs from PDU set.

- QCM suggests an alternative way of capturing LGE’s proposal by keeping the remaining timer per SDU, but redefining the remaining time to per PDU set.

- CMCC thinks auto reTX should only refer to high importance PDUs.

- Lenovo has sympathy for Xiaomi’s proposal as there should be no long intervals between PDUs. Nokia agrees.

- LGE thinks that with Xiaomi’s proposal we are more likely to fail to meet PSDB requirement.

DISCUSSION on P7 from LGE paper:

- Futurewei tends to support LGE’s proposal but it may be too restrictive. FTW thinks that for high importance packets it can still be used. We can add the condition that low importance discard timer is non-zero.

(PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance

Configuration clarifications (RRC-8)

R2-2504666	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core	R2-2503566

Proposal 2 (RRC-8): the remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

- QCM thinks the granularity can be similar to DSR. Different paths can have different RTT so it makes sense to keep this per RLC.

- Lenovo also prefers to have per RLC entity, also DSR is per LCG so we can align this.

- Ericsson has sympathy for Nokia view as PDCP uses this timer, so not sure how we can have different timers for different RLC entities.

- LGE agrees with Nokia’s proposal.

- Apple indicates that several meetings ago Apple raise the same issue for LCP enhancements and we sticked to per RLC, so we can also do this per RLC.

- LGE thinks that LCP and DSR are different as these are MAC features, but here we speak of RLC features.

- Samsung also prefers PDCP, this will simplify the mechanism.

- QCM indicates all parameters are anyway in RRC. Different RLC entities can correspond to different carriers (e.g. in FR1 and FR2), so there is different RTT.

- KDDI prefers simpler configuration.

- Nokia thinks that it will be simpler as having it at PDCP allows us to avoid impact on DC.

The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Auto reTx and polling applicability for low importance SDUs (RRC-3, RRC-4)

R2-2503913	AM RLC enhancement	Lenovo	discussion	Rel-19

Proposal 4. (RLC-9) If PSI based-discard is activated, the discardTimerforLowImportance is not considered for autonomous retransmission and/or enhanced polling.

Unnecessary retransmission avoidance

Impact on RLF (RLC-6)

R2-2503493	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	(RLC-6) RAN2 to discuss whether/how to solve RLC-based RLF miss-detection issue (i.e., maxRetxThreshold cannot be reached) caused by in-flight PDU discard enhancement.

R2-2503624	Discussion on RLC remaining issues for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8: (RLC-6) RAN2 should discuss the following approaches for triggering RLF when avoiding unnecessary retransmission is enabled:

-	Approach 1: Triggering RLF when the RLC entity receives consecutive N SDU discard indications from the PDCP entity.

-	Approach 2: Triggering RLF when the RLC entity receives a total of N SDU discard indications from the PDCP entity within the configured time window.

R2-2503428	Further consideration on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: (RLC-6) There is no specification impact foreseen for RLF.

DISCUSSION:

- Ofinno indicates that in this case increment would not be due to bad channel conditions, so no enhancement is needed.

- Huawei supports proposal from CATT, it is not critical to address it.

- Lenovo also ado not think any special handling is needed. The packets might not have been even transmitted, so makes no sense to trigger RLF.

- LGE also support CATT proposal, there are other ways to detect bad channel conditions. We should not increment if there was no transmission.

- Apple thinks we should not trigger RLF due to discarded packets. No enhancement is needed.

- MTK, Sharp agrees and also thinks other RLF triggers will kick in.

(RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission.

No SDU to retransmit the poll with (RLC-11)

R2-2503507	Discussion on RLC Enhancements for XR	Samsung	discussion	Rel-19

Proposal 1: When the indicated RLC SDU for discard has a sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded, stop and reset t-PollRetransmit, if running. (Adopt TP 1)

R2-2504666	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core	R2-2503566

Proposal 7 (RLC-11):	If the transmitting window is stalled when t-PollRetransmit expires, the poll is re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

Proposal 8 (RLC-11):	When an RLC ACK or a PDCP-discard indication for an SDU is received, if the transmitting window is not stalled, t-PollRetransmit is stopped if, after the reception, all SDUs previously submitted to lower layer have been either ACKed or their transmissions have been stopped due to discard indication from PDCP.

DISCUSSION:

- Huawei supports Nokia’s proposal, it is important to avoid window stalling.

- Ericsson asks whether this is actually current behavior that when window is stalled then we choose any PDU.

- Sharp agrees window stalling should be addressed and support Nokia’s proposal.

- OPPO thinks in case all PDUs are discarded then poll is useless. OPPO understand companies thought there was no window stalling issue.

- LGE indicates that in case the buffer is empty there is no issue. But in case some PDUs are stuck in the buffer there can be problem. But this is corner case.

- Samsung indicates there is a timer at Rx, so an SR will be sent anyway. Ericsson agrees with LGE.

- Sharp think that even if this is corner case, it has critical consequences, that is why we handle it specifically in legacy case.

- Ericsson thinks it is different than legacy because SR will be sent anyway.

- LGE think even Samsung’s proposal is not needed, we can keep current running CR.

- Vivo agrees with LGE.

Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

Capability or capabilities (Capability-2)

R2-2504342	Discussion on UE Capabilities for Unnecessary Retransmission Avoidance	Ericsson, Qualcomm Incorporated, ZTE Corporation, MediaTek Inc., Xiaomi	discussion	Rel-19

Proposal 1	Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Proposal 2	Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. A UE supporting this feature shall also indicate the support of Rx-side aspect.

- OPPO thinks we do not have mandate UE supporting Tx side to also support Rx side. Vivo agrees.

- LGE thinks Tx side requires Rx side, otherwise we can have window stalling issue we just discussed. LGE supports the proposals.

- OPPO indicates that this is for UL so SR is triggered by the network.

Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Impact of discard on PDCP SN gap report (RLC-12)

R2-2503507	Discussion on RLC Enhancements for XR	Samsung	discussion	Rel-19

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 3):

-	For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

-	A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

R2-2503830	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8. No enhancement is needed for PDCP SN gap report handling during UE mobility.

Proposal 9. If the proposal 8 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

Configuration clarifications (RLC-10)

R2-2503830	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. Add RRC configuration guidance for the stopReTxObsoleteSDU and t-RxDiscard, i.e., if stopReTxObsoleteSDU is configured, the t-RxDiscard is configured at the peer receiving entity.

R2-2503439	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

R2-2503367	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503511	Open Issues on RLC Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503557	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503566	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

=> Revised to R2-2504666

R2-2503635	Discussion on RLC AM Enhancements	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503700	Views on Avoidance of Unnecessary RLC Retransmissions	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503701	Discussions on Fast RLC Retransmission	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503795	Discussion on Remaining Issues of RLC AM Enhancements	China Telecom	discussion	Rel-19

R2-2503818	Details on XR RLC autonomous retransmission	Quectel	discussion

R2-2503835	Discussion on RLC enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503974	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2504032	Remaining issues on RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504056	Timely retransmissions for RLC AM	Sony, Canon	discussion	Rel-19	NR_XR_Ph3

R2-2504118	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504401	Discussion on the left issue of RLC enhancements	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504475	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504519	Discussion on RLC enhancements	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.6	XR rate control

Further details of XR rate control, e.g. configuration, MAC CE design, table design etc.

Single or multiple QoS flows (MAC-05)

R2-2503429	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: (MAC-05) RAN2 follows the legacy design not to introduce the flexibility MAC CE format in Rel-19 XR.

R2-2504476	Discussion on XR rate control	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (MAC-05) XR rate control MAC CE can comprise multiple QoS flows.

ID for flow identification (MAC-04)

R2-2503889	Uplink rate control for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: [MAC-04] RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

R2-2503429	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (MAC-04) RAN2 uses the PDU session ID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19 as baseline.

Proposal 2: (MAC-04) RAN2 further discusses whether any optimize for the overhead is needed.

R2-2503558	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4: In the rate control MAC CE, the index (indices) of the QoS flows is included, e.g., via a bitmap.

Proposal 5: The index in the rate control MAC CE can be configured by network based on the QoS flow ID and the PDU session ID, or the mapping between the index in the rate control MAC CE and the QoS flow ID and the PDU session ID can be pre-defined.

DL rate control (MAC-11)

R2-2503512	Open Issues on Rate Control	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Rel-19 Rate Control MAC CE can be used for DL as well as UL.

R2-2504375	Further consideration on XR rate control and query	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: RAN2 will not specify DL rate control and DL rate control query.

Procedure details and prohibit timer configuration (RRC-1, MAC-08, MAC-10)

R2-2503440	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: (RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

Proposal 7: (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

Available bit rate query (MAC-12)

R2-2503523	XR Rate Control	Ofinno	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1[MAC-12] RAN2 to discuss and down-select one of the following options to indicate that the bit rate query is for an available bit rate with no desired bit rate:

Option 1: Use a specific index (e.g., index 0 or a reserved index) in the bit rate table:

The specific index (e.g., index 0 or reserved index) in the bit rate table specifically to indicate a query for an available bit rate with no desired bit rate.

Option 2: Introduce a single-bit field/flag in the Rate Control MAC CE:

The single-bit field/flag can indicate whether the UE queries an available bit rate with no desired bit rate or queries an available bit rate by indicating a desired bit rate.

Option 3: Introduce an additional Rate Control MAC CE format which does not include bit rate field.

R2-2503831	Remaining open issues on rate control signaling for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6. The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

Proposal 7. The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

Impact of Dual-Connectivity (MAC-9)

R2-2503889	Uplink rate control for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: [MAC-09] RAN2 considers the following solutions to apply the indicated recommended bit rate for a QoS flow in DC:

-	For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

-	For split transmission, the recommended bit rate for a QoS flow may be applied to the cells or cell groups where the MAC CE is received.

R2-2504119	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8: (MAC-09) RAN2 to capture a NOTE that each Rate Control MAC CE indicates the recommended bit rate for the service corresponding to the QoS flow.

Proposal 9: (MAC-09) RAN2 to leave the issue of Rate Control in DC to NW implementation or RAN3 discussion.

R2-2503368	Discussion on XR rate control	Qualcomm Incorporated, MediaTek Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503558	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503625	Discussion on remaining issues of XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503702	Views on Remaining Issues of XR Uplink Rate Control	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503796	Discussion on Remaining Issues of XR Rate Control	China Telecom	discussion	Rel-19

R2-2503836	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2503975	XR Rate control details	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2504000	Discussion on XR Rate Control	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504343	More Views on XR Rate Control	Ericsson	discussion	Rel-19

R2-2504434	Discussion on UL rate control for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2504457	Closing the Loop on XR Rate Control	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

R2-2504599	XR Rate Control	Lenovo	discussion	NR_XR_Ph3-Core

## 8.18	LTE-based 5G Broadcast

(LTE_terr_bcast_Ph2; leading WG: RAN1; REL-19; WID RP-250794)

Time budget: 0.25 TU

Tdoc Limitation: 1 tdoc

### 8.18.1	Organizational

Incoming LS, rapporteur input, running CRs, etc.

### 8.18.2	Other

RAN2 signalling impacts to support time-frequency interleavers.

Baseline signalling design

R2-2504260	RAN2 impacts to support time-frequency interleavers for LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU	discussion	Rel-19	LTE_terr_bcast_Ph2-Core

Proposal 1:	Wait for RAN1 for the down-selection regarding the configuration for time interleaving transmission.

Proposal 2:	Wait for RAN1 for the candidate values of the new MCH scheduling period(s).

Proposal 3:	Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}.

Proposal 4:	Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}.

Proposal 5:	Presence of the parent IE containing fields for N and M indicates time interleaving is enabled.

Proposal 6:	Introduce a flag to indicate whether frequency interleaving for MTCH is enabled.

- QCM indicates RAN already agreed per PMCH configuration.

- ZTE thinks there may be some MAC impacts.

- Huawei asks whether we need to agree those or we just wait for L1 parameter list.

- Samsung thinks we need to align terminology with RAN1.

- Huawei thinks that for P6 RAN1 is still discussing whether flag is needed. For P6 we can wait for RAN1.

RAN2 assumes the following needs to be specified in RRC, but the final parameters/values/structure/terminology needs to be confirmed based on L1 parameters list from RAN1:

    - Follow RAN1 agreement (i.e. per PMCH configuration) on the configuration for time interleaving transmission.

    - Wait for RAN1 for the candidate values of the new MCH scheduling period(s).

    - Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}.

    - Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}.

    - Presence of the parent IE containing fields for N and M indicates time interleaving is enabled.

    - FFS (pending RAN1 decision):  Introduce a flag to indicate whether frequency interleaving for MTCH is enabled.

Co-existence with legacy MBMS and MAC impacts

R2-2503506	Configuration and scheduling aspects for LTE-based 5G Broadcast Phase 2	Samsung	discussion	Rel-19

Proposal 1: Co-existence for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time Interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

Proposal 5: RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and include:

a)	Not allowing multiplexing of two MTCHs in same subframe

b)	Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

c)	Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

d)	Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

Proposal 6: RAN2 to specify the HARQ handling for Rel-19 5G Broadcast with regard to differentiating new transmission and retransmission/repetition, allowing for soft combining and addressing HARQ feedback generation aspects.

DISCUSSION on P1:

- Huawei thinks that this may also depend on L1 parameters list, but is generally OK with the proposal.

- QCM asks if the intention is to support UEs supporting TFI and not supporting TFIs in the same cell. Samsung confirms.

DISCUSSION on P5:

- QCM support the intention but the details may depend on exact configuration of TFI.

- ZTE thinks Samsung proposal is OK as MTCHs are scheduled one after another. Even with the same TFI, they should not be multiplexed together.

- Huawei agrees with ZTE. Two MTCHs are not multiplexed in the same subframe, but they can be multiplexed in one MAC PDU.

- Huawei thinks for bullet d) we need to wait for RAN1, e.g. whether we use padding or skip etc.

DISCUSSION on P6:

- ZTE thinks this is not HARQ and this seems a new enhancement.

- Huawei understands there is no such discussion in RAN1, so it is unclear why there is any impact to HARQ operation.

- Samsung clarifies that HARQ is specified in MAC, so RAN2 can handle this.

- QCM thinks RAN1 did not discuss HARQ.

- EBU thinks that HARQ enhancements do not make sense in the current MBMS design.

We aim to support co-existence of legacy (pre-Rel19) and R19 transmission and UEs by defining R19 PMCHs to cater to Time Interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs. FFS exact signalling

RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and may include, e.g.:

- a)	Not allowing multiplexing of two MTCHs in same subframe

- b)	Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- c)	Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

- d) FFS (pending RAN1 discussion):	Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

Impact on cell reselection

R2-2504075	Discussion on time-frequency interleavers for MBMS	Huawei, HiSilicon	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 3: UE supporting time-frequency interleaving should prioritize the cells enabling time-frequency interleaving while legacy UE shouldn’t.

DISCUSSION:

- QCM tends to agree, but is there any spec impact due to this?

- Huawei thinks we may need a new list of frequencies for the UE to prioritize (e.g. frequencies with TFI enabled).

- Samsung thinks that services will be separated, so no need to change frequency prioritization.

- ZTE thinks the proposal is assuming that the same service will be provided using both TFI and non-TFI which is not a practical scenario.

- QCM thinks this is related to co-existence issue.

- Huawei think not all deployments will always support both legacy and non-legacy services.

FFS cell reselection impact (e.g. need to understand whether there is a practical scenario where some changes would be needed)

UE capabilities

R2-2504620	Discussion on TFI for 5G Broadcast	ZTE Corporation, Sanechips	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 5	LS RAN3 and cc RAN1 on inquiring the feasibility to indicate the intended UE category for one specific eMBMS service from core network.

R2-2504260	RAN2 impacts to support time-frequency interleavers for LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU	discussion	Rel-19	LTE_terr_bcast_Ph2-Core

Proposal 7:	Postpone the discussion about UE capabilities to next meeting.

R2-2503390	Configuration to support LTE-based 5G Broadcast	NEC	discussion	Rel-19	LTE_terr_bcast_Ph2

## 8.20	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

### 8.20.2	Other WGs

Including input for LS from S4-250739

R2-2503341	LS on RTP retransmission (S4-250739; contact: Nokia)	SA4	LS in	Rel-19	5G_RTP_Ph2	To:SA2, RAN2

R2-2503567	RTP Retransmissions for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Conclusion

This contribution has discussed the SA4 LS on RTP Retransmissions for XR and has observed that identifying RTX PDUs would be beneficial to the RAN and can easily be done if two QoS flows are setup (one for the source stream and another one for the retransmissions streams).

R2-2503578	Views on LS on RTP retransmission (S4-250739)	CATT	discussion	Rel-19	5G_RTP_Ph2

Proposal 1: RAN2 to reply to SA4 with the following information:

1.	Whether a separate QoS flow is necessary for RTP retransmission PDUs depends on how many and how often RTP PDUs need to be retransmitted, but this is not clear according to the SA4 LS.

2.	It’s up to gNB implementation to map different QoS flows to same/different DRBs, and also up to gNB implementation to achieve the corresponding scheduling to fulfill the QoS requirement. No additional assistance information from application layer seems to be really necessary.

3.	RAN2 work is postponed until SA2 determines what RAN enhancement is needed for mapping between QoS flow and DRB.

R2-2504117	Discussion on LS S4-250739 for RTP retransmission	Huawei, HiSilicon	discussion	Rel-19	5G_RTP_Ph2

Proposal 1: RAN2 to reply to SA4 that there is no additional benefit to RAN from receiving application layer retransmission information if the retransmission PDUs and the source PDUs are mapped to distinct QoS flows and no RAN impact is needed.

DISCUSSION:

- FTW thinks it is unclear what kind of packet will be retransmitted via RTP.

- Nokia thinks this is irrelevant to RAN2. Lenovo agrees and supports proposal from Huawei tdoc.

- CMCC thinks that from R2 point of view it is simplest to rely on separate QFIs.

- OPPO agrees with Huawei proposal.

- Nokia thinks maybe the proposal seems to suggest there is no benefit. ZTE has similar concern.

?? RAN2 thinks there is benefit for RAN to be able to distinguish initial PDUs and retransmissions (e.g. by mapping them to different QoS flows)

?? From RAN2 point of view this can be achieved if the retransmission PDUs and the source PDUs are mapped to distinct QoS flows as in this case no RAN impact is needed.

[AT130][502][XR] Reply to SA4 LS on RTP retransmission (Nokia)

Scope: Discuss the reply LS considering the online discussion

Intended outcome: Agreeable reply LS in R2-2504813

Deadline:  CB session on Thursday