3GPP TSG-RAN2 #130	R2-250XXXX

Malta, May 19 – 23, 2025

Agenda Item:	8.8.2

Source: 	Xiaomi

Title:  	Report of [AT130][304][R19 NR NTN] DL CE (Xiaomi)

Document for:	Discussion and Decision

# Introduction

This document summarizes and provides report on the following offline discussion:

- [AT130][304][R19 NR NTN] DL CE (Xiaomi)

Scope: continue the discussion on further details for the WA and on possible cell re-selection enhancements (e.g. considering the proposals in R2-2503495 and R2-2504400)

Intended outcome: summary of the offline discussion

Offline time: Wednesday 2025-05-2 afternoon coffee break in BO1

Deadline for offline discussion summary (in R2-2504775):  Thursday 2025-05-22 08:00

# Discussion

## Details for the WA

For connected mode SMTC configuration, we made the following WA during the online discussion:

=> WA: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode.

As per offline instruction, this offline is only to discuss detailed solutions for the WA, instead of whether to confirm the WA or not.

According to the contribution from this meeting, the following candidate mechanisms are proposed:

Option 1: UE provide the closest N reference locations/neighbor cells to network [1].

Option 2: Event D1/D2 enhancement [2]:

- option 2.1: Modified event D1/D2, e.g. if network configures distanceThreshFromReference1 as specific value (e.g. 0, 1), UE doesn't check the condition for distanceThreshFromReference1.

- option 2.2: define new event with only a single referenceLocation and distanceThreshFromReference.

Option 3: UE reports the selected SMTCs from configured SMTC set to network [3].

Option 4: Enhanced Coarse location report with coarser granularity [4].

During the online discussion, it seems clear that people want solution without UE consent requirement and simple enough. Rapporteur thinks the above solutions except option 2 may not have UE consent issue. For option 2, some company thinks it may expose UE location as network now has the freedom to configure only a single distance threshold (with minimum value 50m), exploring UE location to network at the accuracy of 50m. Another concern over option 2 is that configuring multiple trigger events may lead to multiple MR before network know how to configure the SMTC. However, rapporteur think for simultaneously triggered events, UE can report the corresponding MRs in one uplink transmission. For subsequently triggered events, it is kind of location update that all the other solutions also require. For option 2.2, concern is raised about its complexity. For option 3, the concern during the online discussion is that if the SMTC selection is up to UE implementation, whether network can trust the reported SMTC or not, because it may impact the mobility performance. Otherwise, SMTC selection criterion may need to be defined, which complicates the solution.

Question 1: Under the WA to introduce a mechanism to assist the NW to configure the SMTCs in connected mode, companies are invited to provide views on which of the above options to take.

## Cell selection/reselection

For cell selection and reselection, the following enhancements were brought up by companies:

Option 1: enhance the cell selection/reselection criterion with separate parameters (e.g. q-RxLevMin, q-OffsetCell) for DL CE UE and non DL CE UE [4].

Option 2: a non DL CE UE deprioritizes the frequency for DL CE cells while DL CE UE doesn't [6].

Option 3: a non DL CE UE deprioritizes DL CE cell while a DL CE UE prioritizes DL CE cell [7].

Option 4: Introduce a new inter/intra frequency exclude cell list for DL CE UE [8][5].

For option 1, the rationale given is that DL CE cell can achieve 3 dB enhanced sensitivity through mechanisms like PDCCH/Msg4 PDSCH/SIB1 repetition, enabling cell access at SS-RSRP levels 3 dB lower than non DL CE cell. Note that, in LTE, we also introduced separate cell selection parameters(q-RxLevMinCE-r13, q-QualMinRSRQ-CE-r13) for coverage enhancement.

For option 2, it is applicable to the scenario that DL CE cells are deployed in a separate frequency. And the argument given is that a UE not supporting DL-CE will experience performance degrade in a DL-CE NTN cell.

For option 3, it is applicable to the case that a frequency layer is mixed with DL CE cells and non DL CE cells. And the argument for this is that the UE not supporting the DL CE may not work well when camp on DL CE cells.

For option 4, the rationale provided is that a non DL CE UE will suffer from additional access delay and power consumption in a DL CE cell if UE cannot finish the cell access during the cell active time. And that UE will simply waste its power searching for the cells while the cell is temporary off. Opponent points out that excluded list doesn’t prevent measurement as UE has to capture SSB first to know PCI and check the exclude list.

For option 1 and 3, some company indicates that, for intra frequency cell reselection, if UE selects a cell with lower RSRP than neighbor cell, it may suffer from inter cell interference.

Question 2: companies are invited to provide views on whether to do any enhancement for cell selection/reselection or not, and if yes, which option to take.

# Conclusions

Proposal 1:

# References

[1]	R2-2503943	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

[2]	R2-2504571	Discussion on Downlink Coverage Enhancements	CSCN, Huawei, HiSilicon, ZTE corporation, Sanechips, CATT	discussion	Rel-19	NR_NTN_Ph3-Core

[3]	R2-2504653	DL coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

[4]	R2-2503495	Discussion on DL coverage enhancement	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

[5]	R2-2504400	Analysis on DL coverage enhancements due to extended SSB periodicity	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

[6]	R2-2503905	Further considerations on NR NTN DL-CE	Lenovo	discussion	Rel-19

[7]	R2-2504006	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

[8]	R2-2504079	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core