3GPP TSG RAN WG2#130	R2-2504933

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121			R1-2504915

St Julian’s, Malta, May 19th – 23th, 2025

Title:	LS on Ambient IoT R2D control information

Response to:

Release:	Rel-19

Work Item:	Ambient_IoT_Solutions

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

Name:	Jingwen Zhang

E-mail Address:	zhangjingwen@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description:

In Rel-19 Ambient IoT, RAN1 discussed the R2D control information for R2D reception and D2R scheduling, and made the following agreements:

For D2R scheduling

General

Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Agreement For CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, provided in the paging message are the same for all the FDMed Msg1 transmissions. Agreement For CBRA, for TDMA of X=2 Msg1 transmissions in response to a R2D transmission triggering random access, the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, provided in the paging message are the same for all the TDMed Msg1 transmissions. Agreement For FDMA of Msg3 transmissions in response to a PRDCH for Msg2: - The bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, provided in the PRDCH for Msg2 are the same for all the FDMed Msg3 transmissions

Frequency domain resource

Agreement For D2R transmission, - The minimum Tb is 1.39μs. - Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: This does not imply the device to pre-store the table. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. Agreement For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates - a single bit duration Tb which is same for all frequency domain resources - a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R - note: the set of R values could be signalled using a bitmap The detailed signalling design is left to RAN2. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement For frequency domain resource for Msg 3 transmission determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, the reader indicates: - a single bit duration, which is same for all frequency domain resources, - a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R - note: the set of R values could be signalled using a bitmap The mapping relationship between device and its Msg 3 frequency domain resource is left to RAN2. - Note: Device could determine its R value for Msg 3 transmission based on its order of random ID in Msg 2 The detailed signalling design is left to RAN2.

Time domain resource

Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s).

TBS-like information

Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. Agreement Confirm the working assumption with following updates Working assumption - For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size except for Msg1 and Msg3 transmission in CBRA and 1st D2R Message in CFRA: - 7 bits for byte-level D2R payload size indication - Regarding the TBS of Msg3 in CBRA, and 1st D2R Message in CFRA - From RAN1 perspective, it is up to other WGs to decide the actual payload value set and how device knows the actual payload size

Code rate and repetition

Agreement For the D2R transmission - 1 bit is used to indicate whether FEC is applied or not. - 1 bit is used to indicate the number (1 or 2) of block-level repetitions.

Amble-related information

Agreement For D2R midamble, for determining the presence and location of midamble(s) at the device: - Reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information - FFS: details of signalling - FFS: whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end - Note: This does not preclude the support of having no midamble present in the D2R transmission Agreement For indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following is adopted: - Unit of interval is number of bits after FEC (if FEC is applied) and repetition (if repetition is applied) - FFS: the candidate values in terms of the unit of interval Agreement - For D2R, for indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following interval values are adopted: - For bit duration of 266.67μs - I = 48 bits, 96 bits, 168 bits, 240 bits - For other supported bit durations of 266.67μs/Y - I = Y * {48, 96, 168, 240} bits - Values of Y: 2, 4, 8, 16, 32, 64, 192 - For signaling via R2D control information, following is adopted: - 1-bit length codepoint is used to indicate whether long or short preamble/midamble is applied at the device, where “0” indicates short preamble/midamble and “1” indicates long preamble/midamble - Midamble interval is indicated by a 2-bit length codepoint - Lowest to highest codepoint value indicates lowest to highest interval value - 1-bit length codepoint is used to indicate whether the midamble is present at the end or not, where “0” indicates no midamble present at the end and “1” indicates midamble present at the end - Note: if the indicated interval is longer than the number of bits after FEC (if FEC is applied) and repetition (if repetition is applied), and 1-bit length codepoint does not indicate midamble present at the end, then there is no midamble.

For R2D reception

Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

# 2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above RAN1 agreements into account when designing the higher layer signalling, including defining R2D TBS information (excluding CRC length) to be included in higher layer signaling, at least for messages with variable size.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #122              25 Aug. – 29 Aug. 2025        Bengaluru, India

TSG RAN WG1 Meeting #122b            13 Oct. – 17 Oct. 2025        Prague, Czech Republic