3GPP TSG RAN WG2#130	R2-2504934

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121			R1-2504924

St Julian’s, Malta, May 19th – 23th, 2025

Title:	LS on Ambient IoT Stage-2 TP

Response to:

Release:	Rel-19

Work Item:	Ambient_IoT_Solutions

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

Name:	Jingwen Zhang

E-mail Address:	zhangjingwen@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description:

RAN1 agreed the following TP for physical layer functions for TS 38.300:

### 16.x.3	A-IoT Physical layer functions

#### 16.x.3.1	Waveform, numerology, time and frequency domain structure

The R2D transmission is a DFT-s-OFDM-based OOK waveform with subcarrier spacing Δf=15∙103 Hz and normal cyclic prefix. The R2D transmission in each OFDM symbol is described by a number of resource blocks, NRBR2D, and one resource block consists of 12 consecutive subcarriers. In each OFDM symbol, there are a number of chips to which modulated symbols are mapped. For in-band operation, the starting position of a A-IoT R2D transmission is aligned in time with the starting position of an NR OFDM symbol.

The D2R transmission is described by a set of chips to which modulated symbols are mapped, and based on backscattering on a carrier wave. The carrier wave is a single-tone sinusoid signal.

#### 16.x.3.2	R2D

##### 16.x.3.2.1	Physical reader-to-device channel

The physical reader-to-device channel (PRDCH) carries an R2D block originating from the A-IoT MAC layer.

The physical-layer processing of PRDCH consists of the following steps:

-	CRC attachment;

-	Line encoding with OOK modulation;

-	Mapping to chips and OFDM symbols.

##### 16.x.3.2.2	Timing acquisition signal

An R2D timing acquisition signal (R-TAS) is transmitted immediately before a PRDCH, and consists of a start indicator part (SIP) followed by a clock acquisition part (CAP). The device determines that an R2D transmission begins upon determining that a SIP of R-TAS has been received. The CAP indicates the number of chips per OFDM symbol and chip duration for PRDCH.

#### 16.x.3.3	D2R

##### 16.x.3.3.1	Physical device-to-reader channel

The physical device-to-reader channel (PDRCH) carries the D2R block originating from the A-IoT MAC layer.

The physical-layer processing of PDRCH consists of the following steps:

-	CRC attachment;

-	Block repetition;

-	Channel coding, which may be omitted;

-	Modulation of OOK or BPSK, resulting in small frequency shift;

-	Mapping to chips.

##### 16.x.3.3.2	D2R amble signals

A D2R preamble signal for timing acquisition, timing tracking and channel estimation, and zero or more D2R midamble signal(s) for timing tracking and channel estimation, are inserted in a D2R transmission. A D2R preamble signal is transmitted immediately before a PDRCH.

# 2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to incorporate the TP in TS 38.300.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #122              25 Aug. – 29 Aug. 2025        Bengaluru, India

TSG RAN WG1 Meeting #122b            13 Oct. – 17 Oct. 2025        Prague, Czech Republic