3GPP TSG RAN WG2#130	R2-2504960

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121	R1-2504885

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	Reply LS on differentiation of sDCI based mTRP and sTRP

Response to:	R2-2411244

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN1

To:	RAN2

Cc:

Contact person:

Name:	Jiayi Yang

E-mail Address:	yangjiayi@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall description

RAN1 would like to thank RAN2 for the LS on differentiation of sDCI based mTRP and sTRP. RAN1 provides the corresponding replies.

- sTRP vs sDCI mTRP

To separate the cells operated as sTRP and sDCI mTRP, it is proposed to add the following highlight sentence. imultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.. But some companies understand that applyIndicateTCI-State with value “none” configured for ControlResourceSet indicates sTRP operation and different channels on a BWP may operate differently.

Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence?

Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

[RAN1’s reply]

Question 2a): Yes.

Question 2b): RAN1 has reached to the following conclusion:

Conclusion

RAN1 has no consensus on the down-selection between the alternatives of the below agreement:

Agreement To differentiate the cells operated as sTRP vs sDCI mTRP, down-select one of the options: - Alt-1: It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Option 1: RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. However, for each list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells or none of the following RRC parameters is configured in any BWP of any serving cell. - “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or - “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or - “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18. - mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config - “repetitionSchemeConfig-r16” in PDSCH-Config - “searchSpaceLinkingId-r17” in searchSpace - “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17 - “cjt-Scheme-PDSCH-r18” in ServingCellConfig - Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’ - Option 2 (No concern): RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Alt 2: It is RAN1 consensus that the number of indicated TCI states can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation.

- Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the information into consideration.

- Date of next TSG-RAN WG1 meeting

TSG RAN WG1 Meeting #122	25th August – 29th August 2025	Bangalore, India

TSG RAN WG1 Meeting #122bis	13th October – 17th October 2025	Prague, Czech Republic