3GPP TSG-RAN WG2 Meeting #130	R2-2503343

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.2.2

Source:		Futurewei

Title:		Open issues on A-IoT paging

Document for:		Discussion and Decision

# Introduction

In meeting#129-bis, RAN2 reached the following agreements with respect to A-IoT paging design [1]:

Agreements on behavior for parallel service requests

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

In this contribution, we discuss some open issues of A-IoT paging.

# Discussions

## Behavior for parallel service requests (related to Issues 1-1 and 1-2)

Two solutions for parallel service requests were discussed in meeting#129-bis: In Solution A, the device essentially ignores the new service request, while in Solution B, the device abandons its current ongoing procedure, if any, and attempts to respond to the new service request.

A main issue for Solution A is the potential deadlock for devices that have not successfully responded to the previous service request. To address the issue, a reader ID or an explicit signaling for announcing the end of a service session is required, adding complexity to both the readers and the devices.

Solution B is simple and works generally well in a single-reader deployment, which is the scenario that Rel-19 A-IoT WI is focused on. A potential issue of Solution B is its operation in a multi-reader deployment. For example, a paging message from a neighboring reader received by the device may interrupt the ongoing procedure of the device with another reader. However, the interruption may cause no harm to the device if the paging message from the neighboring reader is sent only after the reader, with which the device has the ongoing procedure, has already terminated the procedure from the reader’s end. This requires coordination among neighboring readers, which can be handled by network implementations.

Observation. Solution B is simpler and has less issues and hence is a better solution than Solution A for Rel-19 A-IoT.

Proposal 1. RAN2 adopts Solution B for parallel service requests for Rel-19 A-IoT.

However, if Proposal 1 is not agreeable and RAN2 decides to address the issue associated with multi-reader deployment or at least to have the Rel-19 design be forward compatible for multi-reader deployment scenarios, we suggest that some restriction on how the Transaction IDs from the same reader are generated can be considered, for example, the Transaction ID is always increased by 1 for a new service request started by the same reader. In this way, if the device finds that its stored Transaction ID does not match with the Transaction ID in a paging message received but matching with the Transaction ID value in the paging message minus 1, after possible adjustment by a modulo function for wrap-around, then the device considers that the paging message is still received from the same reader except that the reader has terminated the old service request and is requesting a new one in the paging message. On the other hand, if the device finds that its stored Transaction ID does not match either the Transaction ID value in the paging message or the Transaction ID value in the paging message minus 1, after modulo function, the device can consider the paging message is received from a different reader hence choose to ignore it.

Proposal 2. If Proposal 1 is not agreeable, RAN2 introduces some restriction on how the Transaction IDs from the same reader are generated, e.g., always increasing by 1 for new service request.

Proposal 3. If Transaction ID is always increased by 1 for a new service request from the same reader, a device finding that its stored Transaction ID does not match with the Transaction ID in a paging message received but matching with the Transaction ID value in the paging message minus 1, after possible adjustment by a modulo function for wrap-around, considers that the paging message is still received from the same reader except that the reader has terminated the old service request and is starting a new one.

## Content of paging message for CFRA (related to Issue 1-5)

Several PHY layer related parameters are expected to be included in the paging messages that initiates a CBRA procedure. These parameters include value X (for TDM Msg1s), value Y (for FDM Msg1s), and Q-like value (or number of access occasions), which are used only in CBRA procedures, not in CFRA procedures. A question has been raised on whether these three parameters (and maybe other parameters exclusively used for CBRA) should be omitted from the paging message that initiates a CFRA procedure by standardization so that there is no need consider the presence bits for these fields in such paging message.

Since we have already agreed on using a 1-bit as the explicit indication of CBRA vs. CFRA, contents in a paging message that initiates a CFRA procedure should be optimized by omitting all fields not needed for CFRA, e.g., value X, value Y, the Q-like value to begin with, without a need of a present bit for these fields in the paging message. A device uses the CBRA/CFRA indication bit to determine whether the presence bits associated with the fields carrying value X, value Y, and the Q-like value need to be considered when parsing the paging message received.

Proposal 4. Contents in a paging message that initiates a CFRA procedure is to be optimized by omitting all fields not needed for CFRA, e.g., value X, value Y, the Q-like value to begin with, without a need of a present bit for these fields in the paging message.

Proposal 5. A device uses the CBRA/CFRA indication bit in a paging message received to determine whether the presence bits associated with the fields carrying value X, value Y, and the Q-like value should be considered when parsing the paging message received.

# Conclusions

Observation. Solution B is simpler and has less issues and hence is a better solution than Solution A for Rel-19 A-IoT.

Therefore, we propose the following:

Proposal 1. RAN2 adopts Solution B for parallel service requests for Rel-19 A-IoT.

Proposal 2. If Proposal 1 is not agreeable, RAN2 introduces some restriction on how the Transaction IDs from the same reader are generated, e.g., always increasing by 1 for new service request.

Proposal 3. If Transaction ID is always increased by 1 for a new service request from the same reader, a device finding that its stored Transaction ID does not match with the Transaction ID in a paging message received but matching with the Transaction ID value in the paging message minus 1, after possible adjustment by a modulo function for wrap-around, considers that the paging message is still received from the same reader except that the reader has terminated the old service request and is starting a new one.

Proposal 4. Contents in a paging message that initiates a CFRA procedure is to be optimized by omitting all fields not needed for CFRA, e.g., value X, value Y, the Q-like value to begin with, without a need of a present bit for these fields in the paging message.

Proposal 5. A device uses the CBRA/CFRA indication bit in a paging message received to determine whether the presence bits associated with the fields carrying value X, value Y, and the Q-like value should be considered when parsing the paging message received.

# References

R2_129b_ChairNotes_25-04-11_16-45 final.