3GPP TSG-RAN WG2 Meeting #130                                                                R2-2503354

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Further Discussion on S&F Operation

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis [1], the issue of S&F operation was discussed and the following agreements were made.

Agreements: - For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. - We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS. - We don’t introduce any new release cause related to S&F operation.

Moreover, RAN2 discussed whether UE to prioritise frequency/cell corresponding to the satellite(s) indicated in the MME-configured satellite list. However, it was postponed.

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list. - Google supports this. CMCC supports this but thinks this would apply to UEs supporting S&F - IDC thinks we don’t have cell specific priorities but only frequency priorities. Also what does it means that the UE prioritizes. IDC also thinks that NB-IoT does not support priority based cell reselection. - Nokia does not support this and thinks the observations do not refer to cell reselection - QC thinks p1 and p1a are in contradiction (p1 relates to MME configured satellite list) - Novamint supports the principle of p1 and p1a - Samsung wonders if there will be many satellites the UE can prioritize - Xiaomi thinks p1 makes sense if there are only S&F mode cells around. - Come back in the next meeting (the proponents could show the possible spec impact if we decide to go in this direction)

Besides, according to the capability running CR [2], the S&F AS UE capability is introduced. However, it is questioned whether such UE capability is needed.

In the contribution, we will continue to analyse the following issues and give our views.

- Information for the normal mode to S&F mode transition;

- Usage of the S&F monitoring list;

- S&F capability.

- Discussion

## 2.1 Information for the normal mode to S&F mode transition

In RAN2#129bis, RAN2 agreed to introduce indication information for the normal mode to S&F mode transition. However, it is FFS what the details of such information, i.e., whether to reuse the existing t-service or introduce new time information. From our perspective, the decision mainly depends on how UE behaves based on the time information from the normal mode to the S&F mode.

For legacy UE, since UE does not support the S&F operation mode, it is straightforward for UE to trigger the cell reselection before or upon the time for the normal mode to S&F mode transition. While for S&F capable UE, there are two options:

- Option 1: S&F UE still camps on the cell even if the cell changes from normal mode to S&F mode, considering the S&F UE can work in the cell operating in S&F mode;

- Option 2: S&F UE behaves like the legacy UE to trigger the cell reselection measurement before or upon the time for the normal mode to S&F mode transition to have better performance.

If option 1 is taken, new time information may be needed for S&F UE. Otherwise, according to the spec [3], all UE(s) shall perform cell reselction measurement before the t-service.

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

If option 2 is taken, NW can set the t-service to the time for the normal mode to S&F mode transition, and then all UE, regardless of whether UE support the S&F or not shall trigger the cell reselection measurement before the t-service, i.e., the time for the normal mode to S&F mode transition. Moreover, in case the cell changes from normal mode to S&F mode, the feeder link is not available, and this is consistent with the meaning of the t-service [4].

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

Proposal 1: RAN2 to firstly discuss whether the S&F capable UE is required to perform the cell reselection measurement before or upon the normal mode to S&F mode transition.

Proposal 2: If S&F capable UE is required to perform the cell reselection measurement before or upon the normal mode to S&F mode transition, legacy T-service is reused. Otherwise, the new time information is introduced.

## 2.2 Usage of the S&F monitoring list

According to the SA2 CR [5], MME can provide a list of satellite IDs to UE via the NAS message. The satellite IDs are used for UE to decide which cell/satellite can re-attempt to perform the subsequent attach/TAU procedure after the wait time expires. From this perspective, it is clear that the satellite ID provided by NAS is globally unique at least within the PLMN.

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with asatellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

Observation 1: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique within the PLMN.

Regarding the satellite ID used in SIB, according to the discussion in RAN2#122 [6], the number of the satellite ID depends on the maximum number of satellites to be considered for mobility, but not the deployed satellite in the same or neighbour orbits.

5. The satellite ID in the new SIB is an integer of X bits wherein X depends on the maximum number of satellites to be considered for mobility.

Based on the above agreements, our understanding is that the satellite ID used in SIBs is unique within the cell level, i.e., different cells may assign different satellite ID for the same satellite.

Observation 2: The satellite ID used in the RAN is not a global ID within the PLMN, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

Based on the analysis above, the satellite ID in SIB(s) is not globally unique, which only considers the satellite surrounding the current serving cell, while he satellite ID in the S&F monitoring list is globally unique within the PLMN. Besides, there is only a satellite index within the S&F monitoring list. Thus, there is ambiguity for UE to decide which ephemeris information and the association of satellite ID provided in S&F monitoring list with the frequency or cell may not be sufficient. Restricted by such ambiguity, it is not possible to prioritise frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list during the cell reselection procedure. Maybe there is some scenario that the number of the satellite in the orbit(s) is not large, UE can blindly try to consider the frequency/cells of the satellite(s) in the MME-configured satellite list, but this is up to UE implementation and it may not work well.

Proposal 3: RAN2 confirms whether and how UE prioritise the frequency(ies) corresponding to the satellite(s) indicated in the MME-configured satellite listis up to UE implementation.

## 2.3 S&F capability

According to the capability running CR [2], the S&F AS UE capability is introduced.

#### 4.3.38.xx ntn-StoreAndForward-r19 This field defines whether the UE supports the store and forward operation when feeder link is not available as defined in TS 36.331 [5]. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17.

However, such UE capability is already introduced in the NAS [5]. Considering that a NAS-level UE capability has already been introduced, we do not see that this Uu capability is necessary. The network can potentially make use of the NAS level capability to handle the S&F UEs.

When an on-board MME supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

Proposal 4: No need to introduce AS S&F UE capability, NW can rely on the NAS level S&F UE capability to decide how to handle the UE.

- Conclusion

In this contribution, we have further discussed the S&F operation aspect. And we make the following observations and proposals:

Information for the normal mode to S&F mode transition:

Proposal 1: RAN2 to firstly discuss whether the S&F capable UE is required to perform the cell reselection measurement before or upon the normal mode to S&F mode transition.

Proposal 2: If S&F capable UE is required to perform the cell reselection measurement before or upon the normal mode to S&F mode transition, legacy T-service is reused. Otherwise, the new time information is introduced.

Usage of the S&F monitoring list:

Observation 1: To assist UE perform subsequent attach/TAU, the satellite ID provided by the NAS is globally unique within the PLMN.

Observation 2: The satellite ID used in the RAN is not a global ID within the PLMN, i.e., it is allocated by the eNB and it uniquely identifies a satellite within the cell.

Proposal 3: RAN2 confirms whether and how UE prioritise the frequency(ies) corresponding to the satellite(s) indicated in the MME-configured satellite listis up to UE implementation.

S&F capability:

Proposal 4: No need to introduce AS S&F UE capability, NW can rely on the NAS level S&F UE capability to decide how to handle the UE.

- Reference

- Draft_RAN2_129bis_Meeting_Report_v1.

- [Post129bis][313][R19 IoT NTN] capability CR (Qualcomm)

- TS38.331, NR; Radio Resource Control (RRC); Protocol specification.

- TS38.304, NR; User Equipment (UE) procedures in Idle mode and in RRC Inactive state.

- S2-2502513, Remove EN and clarification on UE behaviour.