3GPP TSG-RAN WG2 Meeting #130	R2-2503364

St Julian’s, Malta, 19-23 May 2025

Agenda item:	8.7.3

Source:	Qualcomm Incorporated

Title:	Discussion on measurement gap enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss required signaling support for the UAI for measurement gap cancelation.

# Discussion

There are different types of measurement gaps (MG) designed for different purposes, e.g. positioning MG, MUSIM MGs, inter-frequency MGs, etc. Not every type of MGs should be subject to DCI-based (aka dynamic) MG cancelation.

UE’s mobility status may also affect whether an MG configuration is eligible for cancelation or not. For example, when UE roams between cell edge and cell center, the eligibility of dynamic cancelation for a MG configuration can change accordingly.

Although network is in full control of when to send cancelation DCIs, it is useful for UE to know whether it can expect cancellation DCIs or not. This would enable the UE to better prepare its measurement scheduler to act upon a cancelation DCI in time.

Observation 1.	Knowing whether to expect cancelation DCI can help UE better prepare for changes in its measurement schedule.

Therefore, we think it is desirable to have a mechanism that enables and disables DCI-based cancelation for an MG configuration. The exact signaling method (e.g. whether is RRC configuration or MAC CE signaling) can be FFS, as different options have their respective advantages.

Proposal 1.	Network can configure/signal whether a MG configuration is enabled or disabled with DCI-based cancelation. FFS whether by RRC configuration or MAC CE.

[RRC-05] When UE should trigger UAI for MG cancelation

In RAN4 LS R4-2504972, RAN4 informs RAN2 that UE may send UAIs to indicate a preferred ratio (R) of measurement gaps that may be canceled during a period of 1 sec. The possible values of R are {0, 20, 40, 60}%.

In legacy, the spec (TS 38.331) specifies, only at a very high level, when a UAI message may be triggered. For example, UAI for preferred DRX parameters has the following triggering conditions:

A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters.

As we may see, the conditions are very generic and high level. Nothing specific is used. This is because there can be different reasons why UE may want to change its DRX parameters. For example, it could be due to changes in UE’s traffic patterns as well as UE’s power status. However, it is not trivial to capture these conditions precisely in the spec.

Observation 2.	Generally, triggers for UAI do not need to have specific conditions.

The case with measurement gap cancellation is similar. For example, UE may want to change the cancelation because its mobility status has changed. Or it could be that UE’s traffic pattern has changed, such that bursts of its delay-critical data has become more (or less) likely to collide with its MGs. These conditions are not trivial to capture precisely in the spec. Moreover, since UAI is only assistance input for gNB, it is not necessary to mandate all UE implementations to have exactly the same triggering conditions. Therefore, it is sufficient to specify that UAI for MG cancelation is initiated when UE has a preference for the cancelation or when it changes its preference for the cancelation.

Proposal 2.	[RRC-05] UAI for MG cancelation is initiated when UE has a preference for the cancelation or when it changes its preference for the cancelation.

In RAN4’s LS, RAN4 does not explicitly specify the granularity of the cancelation ratio R, e.g. whether it is per UE or per MG configuration or something else.

If UE is configured with per-FR MGs, then it is possible for network to configure different measurement patterns in different FRs. These MGs may have different measurement patterns because there can be different measurement loads and/or different types of traffic scheduled in different FRs. Therefore, we believe it is necessary to support different cancelation ratio Rs for MGs in different FRs.

However, in legacy, per-FR MGs are not configured with identifiers. As it is not possible to introduce new identifiers for them now, the differentiation between MGs can be signaled implicitly, i.e. UAI is applicable only to the MG in the FR where the UAI is sent.

Proposal 3.	If a UE is configured with per-FR MGs, a UAI for MG cancelation is applicable only to the per-FR MG in the FR where the UAI is sent.

Concurrent MGs have been introduced since R17. Each concurrent MG is configured with a gap ID. Based on the similar argument made earlier, we think it is desirable to support different cancelation ratio Rs for different concurrent MGs. Since each concurrent MG has an ID, it is easy to identify them, i.e. UE can include the gap ID in the UAI to indicate which concurrent MG that it is intended for.

Proposal 4.	If a UE is configured with concurrent MGs, a UAI for measurement gap cancellation includes the gap ID of the MG that it is intended for.

[RRC-06] What are the configurations for controlling UAI reporting

Many UAIs have an associated prohibit timer. We think it is relevant for MG cancelation too, as UE does not need to request a new cancellation ratio frequently. UE is not allowed to send a UAI for cancelation when the prohibit timer is running, or when the prohibit timer is absent or has a value of 0.

Proposal 5.	[RRC-06] Transmission of UAI for MG cancelation is regulated by a prohibit timer. UE is not allowed to send a UAI when the prohibit timer is running, or when the prohibit timer is absent or has a value of 0.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	Knowing whether to expect cancelation DCI can help UE better prepare for changes in its measurement schedule cancelation.

Proposal 1.	Network can configure/signal whether a MG configuration is enabled or disabled with cancelation. FFS whether by RRC configuration or MAC CE.

Observation 2.	Generally, triggers for UAI do not need to have specific conditions.

Proposal 2.	[RRC-05] UAI for MG cancelation is initiated when UE has a preference for the cancelation or when it changes its preference for the cancelation.

Proposal 3.	If a UE is configured with per-FR MG, a UAI for MG cancelation is applicable only to the per-FR MG in the FR where the UAI is sent.

Proposal 4.	If a UE is configured with concurrent MGs, a UAI for measurement gap cancellation includes the gap ID of the MG that it is intended for.

Proposal 5.	[RRC-06] Transmission of UAI for MG cancelation is regulated by a prohibit timer. UE is not allowed to send a UAI when the prohibit timer is running, or when the prohibit timer is absent or has a value of 0.

# Reference

[1] R4-2504972, Reply LS on UE assistance information, Nokia, April 2025.