3GPP TSG-RAN WG2 Meeting #130	R2-2503365

St Julian’s, Malta, 19-23 May 2025

Agenda item:	8.7.4.1

Source:	Qualcomm Incorporated

Title:	Discussion on LCP enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss remaining issues related to priority adjustment.

# Discussion

[MAC-01] Priority adjustment for SR

When an LCH has delay critical data, a DSR is triggered, which then triggers an SR if no PUSCH resource is available for its transmission.

It is also possible that new data in an LCH triggers an SR. If this SR fails to go through after multiple transmissions, data which triggered the SR has then become priority adjustable.

In these two cases, it is beneficial to give the SR higher priority, so that it can be received by gNB sooner.

Observation 1. 	There are use cases where priority adjustment for an SR can help expedite its successful transmission.

There are two possible options that can give a pending SR higher priority in the above cases:

- Option 1. If this SR is triggered by a logical channel that has priority-adjustable data, then during intra-UE prioritization, the priority of the SR should be the same as the additional priority of that logical channel;

- Option 2. The transmission of this SR can use an SR configuration that has higher priority. Typically, SR configurations with higher priority have more frequent PUCCH transmission occasions. That would allow the SR to be transmitted sooner.

The desired SR configuration in Option 2 can be selected as follows. For a pending SR, if there is an SR configuration with a corresponding logical channel whose priority matches the additional priority of the logical channel that triggered the pending SR, then the SR configuration can be used for the transmission of the pending SR.

We observe that Option 2 is applicable in more scenarios than Option 1. During intra-UE prioritization, if UE uses an SR configuration corresponding to the additional priority, no extra priority adjustment is needed but the same benefit can be attained as in Option 1. On the other hand, if intra-UE prioritization is not needed (no overlapping transmissions), the pending SR can still benefit from more frequent PUCCH transmissions occasion in the SR configuration with higher priority.

Observation 2. 	If a pending SR can use an SR configuration that matches the additional priority of its triggering logical channel, it has more benefits than using additional priority only for intra-UE prioritization.

Option 2 has about the same level of spec impact as Option 1. For example, the following TP can be used to specify how a pending SR may adjust its SR configuration:

### 5.4.4 Scheduling Request (text omitted) The MAC entity shall for a pending SR: - if this SR is triggered by a pending BSR (clause 5.4.5) or a pending DSR (clause 5.4.9) associated with a logical channel configured with additionalPriority; and - if there is an SR configuration with a corresponding logical channel whose priority matches the additionalPriority of the logical channel that triggered this SR; and 1> if the smallest remaining value of the running PDCP discardTimers among the data available for transmission in this logical channel, evaluated at the time of the first symbol of the next PUCCH resource in the SR configuration, is below or at priorityAdjustmentThreshold configured for the logical channel: - apply the SR configuration for the transmission of this SR.

It is not hard to see that this TP is very similar to the one for priority adjustment for intra-UE prioritization.

Observation 3. 	Adjusting SR configuration has the same level of spec impact as priority adjustment in intra-UE prioritization.

Since Option 2 has additional benefits but no more spec impact than Option 1, we think it should be considered first.

Proposal 1.	[MAC-01] A pending SR can use an SR configuration that matches the additional priority of its triggering LCH, if the LCH has priority adjustable data.

[MAC-02] Impact of congestion on priority adjustment

Suppose LCH A is allowed to adjust its priority from p1 to p2, due to the presence of priority-adjustable data. It then will compete with UL resources equally with LCH B, which has the same priority p2.

If LCH B is already in congestion (i.e. PSI based discard has been activated), then the priority adjustment for LCH A can worsen the congestion in LCH B, as LCH B has to share UL resources with LCH A. Note that, when in congestion, LCH B itself may or may not have priority-adjust data, as it would depend on available bandwidth and traffic arrival pattern.

We think this scenario can occur, because congestion in a high-priority LCH can propagate to LCHs with lower priorities. It can result in longer queueing delays for data in low-priority LCHs, eventually causing their remaining time to drop below the priority-adjustment threshold.

Therefore, we believe it is desirable to add a restriction to the priority adjustment criteria, i.e. if an LCH with priority p is in congestion, then no other LCHs are allowed to adjust their priorities to priority p.

Proposal 2.	[MAC-02] If an LCH with priority p is in congestion (i.e. PSI based discard has been activated), then no other LCHs are allowed to adjust their priorities to priority p, even if they meet the priority adjustment criteria.

A related issue was raised in R2-2402579. It discussed whether PDUs with low importance should be eligible for priority adjustment.

We think that is a good question to ask. After PSI based SDU discard is activated, PDUs with low importance apply a shorter discard timer, i.e. discardTimerForLowImportance. Consequently, they are more likely to become priority adjustable. But since they have low importance, it is not sensible to prioritize their transmission during congestion. Therefore, to ensure PDUs with high importance to have more scheduling opportunities, PDUs with low importance should not be considered for priority adjustment.

Proposal 3. 	[MAC-02] If an LCH is in congestion (i.e. PSI based discard has been activated), PDUs with low importance are not considered for priority adjustment.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

[MAC-01] Priority adjustment for SR

Observation 1. 	There are use cases where priority adjustment for an SR can help expedite its successful transmission.

Observation 2. 	If a pending SR can use an SR configuration that matches the additional priority of its triggering logical channel, it has more benefits than using additional priority only for intra-UE prioritization.

Observation 3. 	Adjusting SR configuration has the same level of spec impact as priority adjustment in intra-UE prioritization.

Proposal 1.	[MAC-01] A pending SR can use an SR configuration that matches the additional priority of its triggering LCH, if the LCH has priority adjustable data.

[MAC-02] Impact of congestion on priority-adjustment

Proposal 2.	[MAC-02] If an LCH with priority p is in congestion (i.e. PSI based discard has been activated), then no other LCHs are allowed to upgrade their priorities to priority p, even if they meet the priority adjustment criteria.

Proposal 3. 	[MAC-02] If an LCH is in congestion (i.e. PSI based discard has been activated), PDUs with low importance are not considered for priority adjustment.

# Reference

[1] R2-2402579, Remaining details on Intra-UE prioritization/LCP enhancements, Lenovo, April 2025.