3GPP TSG-RAN WG2 Meeting #130	R2-2503367

St Julian’s, Malta, 19-23 May 2025

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss issues related to timely retransmission and avoidance of unnecessary retransmissions.

# Discussion

## Timely retransmission

Status report

To enable the full benefits of polling enhancements in reducing RLC round-trip time, it is important to expedite the corresponding status report too. Otherwise, polls triggered based on remaining time would still be subject to the regulation of legacy mechanisms such as the prohibit timer t-StatusProhibit. As a result, status reports are not returned as soon as they should be.

It is not a good idea to configure a very small t-StatusProhibit for all status reports. For example, when no RLC PDUs have low remaining time, status reports do not need to be sent frequently. They need to be expediated or sent more frequently only when there are time-critical PDUs (those with low remaining times).

Observation 1.	To take full advantage of the enhanced polls in reducing RLC round-trip time, it is important to enhance the corresponding status reports too.

With the enhancements studied so far, there can be a few options for a RLC receiver to recognize there are time-critical PDUs.

For example, a poll is triggered when remaining time of an SDU dropping below a threshold. In this case, if the transmitter can inform the receiver that it is a time-critical poll, the receiver should send a status report immediately without the restriction by t-StatusProhibit or apply a shorter prohibit timer.

For the subsequent discussion, we denote such a status report a time-critical status report.

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Another possible option is to have the receiver trigger a time-critical status report itself. This is possible because of the enhancement for avoiding unnecessary retransmission. If this enhancement is enabled, the receiver starts t-RxDiscard whenever it detects a gap in the RLC sequence numbers. This timer can approximate the remaining time of a missing RLC PDU. Therefore, when the remaining value of this timer drops below a threshold, it implies that the missing PDU has become time critical, and a time-critical status report should be sent back asap.

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

## Avoid unnecessary retransmission

[RLC-06] RLF for low-latency traffic

In legacy, a transmitter triggers RLF when number of retransmissions exceeds maxReTxThreshold.

For a DRB with low delay budget, its PDCP discard timer can have a length much shorter than the time taken to have maxReTxThreshold number of retransmissions. That means an SDU likely gets discarded before that its RETX_COUNT reaches maxReTxThreshold. As a result, RLF will never be triggered.

On the other hand, maxReTxThreshold can’t be too small (e.g. to match the average number of retransmissions before discard), because that can lead to false triggering of RLF.

Observation 2. 	If a low-latency DRB is configured with active discard, it may not be possible for RETX_COUNT to reach maxReTxThreshold.

Observation 3. 	maxReTxThreshold can’t be too small either (e.g. to match the average number of retransmissions before discard), or it can result in false triggering of RLF.

However, if too many SDUs are discarded before they can be successfully sent, the radio link should be considered unusable, and the UE should reselect a better serving cell to ensure on-time delivery of its data.

One solution is to define a new trigger for RLF, considering active discard and low-latency operation. Since individual SDUs have a very limited number of retransmissions to define a reliable trigger for RLF, one can consider using discards among a set of SDUs as a metric. For example, among the last M transmitted SDUs or within the last T msec, if there are N discards due to expiry of PDCP discard timers, then the radio link is either very congested or no longer usable. In either case, it makes sense for UE to trigger RLF and reselect a new serving cell.

Proposal 4. 	[RLC-06] Define a new RLF trigger for DRBs configured with active discard, e.g. RLF is triggered after N active discards (expiry of PDCP discard timer) within the last M SDUs or last T msec.

[capability-02] Whether to have separate or joint UE capability for UE Tx and Rx operations

For avoiding unnecessary retransmissions, RAN2 have adopted a combined approach, i.e. a transmitter stops retransmissions of an outdated SDU based on upper-layer indication of PDU discard, and the receiver abandons obsolete SDUs based on a local timer. However, it is worth noting that the transmitter side enhancement is independent of the receiver side enhancement. Even if there is no upper-layer indication, retransmissions of an outdated SDU can still be avoided, because the transmitter can do so based on the “fake ACKs” from the receiver. The transmitter side enhancement is useful only in that it helps the transmitter stop retransmission of outdated SDUs sooner.

Observation 4. 	Retransmission of outdated SDUs can still be avoided even if only the receiver side enhancements are applied.

Besides the technical feasibility discussed above, it is also beneficial from a UE implementation perspective to define a separate UE capability for the transmitter side enhancement. For example, in UE implementations that prebuild RLC PDUs, PDU discard may require significant changes to the data path and hence are not desired. Therefore, it would greatly facilitate development and testing if the transmitter and receiver enhancements are supported by two separate UE capabilities.

Proposal 5.	[capability-02] Define separate UE capabilities for transmitter and receiver enhancements for avoiding unnecessary retransmissions.

[RLC-12] Impact of discard on PDCP SN gap report

At the RAN2#129 meeting, companies discussed the issue with PDCP SN gap report during mobility events. The following agreements were made:

From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives.

It is possible that a PDCP SN gap report is lost. For example, an SN gap report can be lost during a mobility event, e.g. due to RLC re-establishment (Details may be found in R2-2500491).

In our view, this issue is worth a study in Rel-19, because there can be more PDCP discards in the new release, due to new RLC enhancements such as avoiding unnecessary retransmissions. More PDCP discards will trigger more SN gap reports. The more gap reports there are, the more gap reports that may become lost, even if the loss probability remains the same.

If a SN gap report is lost, the receiver will not be able to know that a missing PDCP SDU within a SN gap has already been discarded. As a result, it will start t-Reordering timer, causing extra delay in the delivery of other SDUs to the upper layers and degrading mobility performance.

Observation 5.	There may be more lost PDCP SN gap reports in Rel-19, which can cause extra delay in the delivery of other SDUs to the upper layers and degrading mobility performance.

Three options were discussed at the RAN2#129 meeting:

-	Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB.

-	Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB.

-	Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

We do not think the first two options would work, because there is no way for source/target gNB to learn or detect that a missing SDU is due to discard, not even with any enhancements for inter-gNB coordination.

Although the third option may work, it is a very inefficient one in our view. This is because a gap report lost during a mobility event is not a typical/average case. Hence it is not sensible to have UE send a SN gap report after every mobility event just to handle a lower-than-average case.

Observation 6.	It is inefficient to have UE send an SN gap report after every mobility event, as a lost report is less frequent than the average case.

In addition, there may be other non-mobility cases where SN gap reports are lost. For instance, transmission errors on the F1 interface may also cause the loss of a gap report. The Alternative 3 above does not help this case at all.

Therefore, we think a good solution should be one that helps UE detect a loss. For example, the target gNB may send a PDCP status report, e.g. after a mobility event or when it suspects a gap report might have lost. On the UE side, if it receives a PDCP status report indicating a PDCP SDU is missing but the SDU has already been discarded, it can infer that the corresponding SN gap report must have been lost. It then triggers a new SN gap report, which helps the gNB advance the reordering window.

Proposal 6. 	[RLC-12] UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Status report

Observation 1.	To take full advantage of the polling enhancement in reducing RLC round-trip time, it is important to enhance the corresponding status reports too.

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Avoiding unnecessary retransmissions

Observation 2. 	If a low-latency DRB is configured with active discard, it may not be possible for RETX_COUNT to reach maxReTxThreshold.

Observation 3. 	maxReTxThreshold can’t be too small either (e.g. to match the average number of retransmissions before discard), or it can result in false triggering of RLF.

Proposal 4. 	[RLC-06] Define a new RLF trigger for DRBs configured with active discard, e.g. RLF is triggered after N active discards (expiry of PDCP discard timer) within the last M SDUs or last T msec.

UE capabilities

Observation 4. 	Retransmission of outdated SDUs can still be avoided even if only the receiver side enhancements are applied.

Proposal 5.	[capability-02] Define separate UE capabilities for transmitter and receiver enhancements for avoiding unnecessary retransmissions.

Impact of discard on PDCP SN gap report

Observation 5.	There may be more lost PDCP SN gap reports in Rel-19, which can cause extra delay in the delivery of other SDUs to the upper layers and degrading mobility performance.

Observation 6.	It is inefficient to have UE send an SN gap report after every mobility event, as a lost report is less frequent than the average case.

Proposal 6. 	[RLC-12] UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.