3GPP TSG-RAN WG2 Meeting #130	R2-2503370

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item		:	8.2.2 (Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on A-IoT paging

Document for	:	Discussion and Decision

1.	Introduction

In this contribution, we discuss our views on A-IoT paging, considering the current progress in RAN2 and the WID [1].

2.	Discussion

2.1	Device behaviour if it gets a new service request while one procedure is still ongoing

In the previous RAN2#129bis meeting [2], RAN2 discussed the device’s behaviour when it receives a new service request while one procedure is ongoing. Two options were considered:

- Option (a): ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

- Option (b): terminate the ongoing procedure and respond to the latest request

The following agreement was made without reaching a consensus on which option to purse.

Agreement – RAN2#129bis - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

As discussed previously, option (b) could lead to the device experiencing a ping-pong problem, particularly in scenarios with multiple readers in future releases. For example, if multiple readers trigger their procedures parallelly, the device would repeatedly restart the newly incoming procedure, being unable to complete any of them and wasting energy. To address this issue in a multiple-reader scenario, option (b) requires coordination between the readers (e.g., the CN coordinates the readers to execute their procedures sequentially rather than in parallel).

Option (a) could help prevent the ping-pong problem without relying on such coordination; however, it comes with certain requirements. The device must be able to identify the end of the ongoing procedure. Additionally, it is needed for device to know which procedure each reader-to-device (R2D) message, including the paging message, is associated with. To this end, certain information, such as the transaction ID, should be included in the R2D messages.

In the RAN2#129bis meeting, several companies expressed their concerns regarding option (b), noting that it is uncertain whether the readers would be perfectly coordinated in the multiple-reader scenario. They pointed out that forward compatibility issues arising from the ping-pong problem could persist in future releases. Some companies then suggested designing Rel-19 A-IoT paging message, along with the associated device behaviour, separately from those of future releases. However, other companies argued that such a separate design is not forward compatible. After the discussion, an agreement was reached stating that A-IoT R2D messages, including the paging message, should allow for extensibility in future releases.

Agreement – RAN2#129bis - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Our understanding of this agreement is that it only addresses the forward compatibility issues related to the format of the R2D messages and does not resolve the forward compatibility issues associated with option (b) (i.e., the ping-pong problem). If there is no reader coordination, even with the extensibility of the R2D messages for future releases, the parallel transmission of R2D messages from different readers could again lead to the ping-pong problem. In other words, the fundamental key for resolving the ping-pong problem is coordination between readers.

We foresee scenarios in future releases where coordination between readers may be imperfect. For example, in topology 2, where UE-readers are deployed, if these UE-readers are mobile and move, the coordination between them at any given moment could become imperfect after their movement. For instance, if two UE-readers with non-overlapping coverage are scheduled to trigger their procedures simultaneously, they may subsequently and unexpectedly move into each other's coverage, resulting in parallel transmission of paging and R2D messages. Additionally, temporary out-of-coverage situations for UE-readers could lead to similar issues, as a temporarily out-of-coverage UE-reader may not be able to participate in coordination. Considering the above discussion, we believe that option (b)'s reliance on network coordination is prone to malfunction, and as a result, option (b) is still not forward compatible.

Besides, to address the unplanned parallel requests mentioned above, option (b) should ensure that devices follow the procedure associated with the last reader they encountered. Therefore, the R2D messages from both the previous reader and the new reader should be distinguishable. To achieve this, like option (a), certain information, such as the transaction ID, should be included within the R2D messages. Given that coordination between readers may be imperfect and must be managed, this information overhead could be necessary for both options (a) and (b). In short, to compensate for the weaknesses of option (b), a similar level of information overhead as that of option (a) is necessary.

Therefore, we prefer option (a) and propose the following.

Proposal 1: A-IoT device should ignore all new requests and R2D messages addressed to itself that are not associated with the ongoing procedure, meaning that option (a) should be supported.

Now, we further discuss the requirements related to option (a). One key requirement is that the device must be able to identify the end of the ongoing procedure. We note that the reader initiates and manages the A-IoT procedure. Therefore, the reader needs to indicate the end of the procedure to the devices. We propose the following, particularly including that the reader should send broadcast R2D messages to signal the end of the procedure.

Proposal 2: RAN2 should introduce a method to indicate to the device the end of the procedure.

Proposal 3: Reader should indicate the end of the ongoing procedure to the A-IoT devices through an R2D message, and the A-IoT devices should adhere to this indication.

At the same time, we should consider the scenario in which an A-IoT device misses the R2D message indicating the end of the procedure. While sending a duplicate indication could be one option, it may become ineffective if the device misses the indication again. Therefore, as a supplementary measure, we propose introducing a timer-based approach to be handled by the A-IoT device.

For example, an A-IoT device could (re)start a timer after receiving an (subsequent) A-IoT paging message or an access occasion trigger message. Depending on the supported timer duration of the A-IoT device, the timer could be set upon receiving a paging message (if the duration is long) or set upon receiving both paging messages and other R2D messages (e.g., access occasion trigger messages) (if the duration is short). This way, even if the indication of the end of the procedure is missed, the A-IoT device could determine the end of the procedure through the expiration of the timer. The detailed device behaviour related to the timer could be discussed further.

Proposal 4: A-IoT device should run a timer to determine the end of the procedure for the case of the indication of the end of the procedure is missed.

Last but not least, to implement option (a), the A-IoT device needs to know which ongoing procedure the received R2D message is associated with. This is essential for pursuing option (a), as the device must identify the specific R2D message relevant to the ongoing procedure among the concurrently transmitted R2D messages. To achieve this, we believe that certain information to identify the association with the ongoing procedure should be included with each R2D message. To this end, we propose that transaction ID should be included in every R2D message, including the A-IoT paging message.

Proposal 5: A-IoT R2D messages (e.g., paging messages) should include transaction ID to specify which ongoing procedure the R2D message is associated with.

2.2	Transaction ID-related issues

During the RAN2#129 meeting [3], RAN2 reached the following agreements regarding the transaction ID. There are still several FFS points that we need to discuss.

Agreements – RAN2#129 Which entity generates the transaction ID - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID Transaction ID Size - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. Multiple Reader Scenario - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

In line with our previously mentioned view supporting option (a), we first discuss how the reader generates the transaction ID. As we explained regarding Proposal 5, we believe that the transaction ID should be included within the R2D messages to properly implement option (a).

The purpose of pursuing option (a) and including the transaction ID within the R2D messages is to achieve forward compatibility, such as preventing the ping-pong problem in a multiple reader scenario. Although RAN2 agreed not to specify something specific for the multi-reader scenario, as summarized above, and decided to rely on the transaction ID, we would like to remind that RAN2 acknowledges the existence of the multi-reader scenario, which has already been considered in the discussion of option (a). Therefore, we believe it is important to consider the multiple-reader scenario when determining how the reader generates the transaction ID. For instance, during the generation of the transaction ID, a portion of it could be used to represent the reader's information.

Proposal 6: RAN2 considers the multiple-reader scenario when determining how the reader generates the transaction ID.

We also need to address the size of the transaction ID. Considering the multiple-reader scenario, the transaction ID should have a sufficient size (e.g., 8 bits) to accommodate information related to reader.

Proposal 7: Considering the multiple-reader scenario, the transaction ID should have sufficient size (e.g., 8 bits) to accommodate information related to the reader.

2.3	Paging message format and contents

As outlined in the WID, RAN2 aims to design a paging message format that can accommodate multiple identifiers within a single paging message for forward compatibility purposes. During the RAN2#129bis meeting, RAN2 agreed to develop an extensible R2D message format (including the A-IoT paging message) to account for future devices and features, as follows.

Agreement – RAN2#129bis - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Additionally, RAN2 agreed on the paging ID length, stating that a field indicating the paging ID length information must always be included alongside the paging ID field.

Agreement – RAN2#129bis - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

To distinguish each identifier in multiple-identifier scenarios, we believe that the ‘length field’ for each identifier should also be included in the A-IoT paging message accompanying the related identifier.

Proposal 8: To ensure forward compatibility with scenarios where multiple paging identifiers can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple paging identifiers.

Proposal 8a: If a reserved field for multiple paging identifiers is defined, the A-IoT paging message should include a ‘length information field’ for each identifier stored in the field.

2.4	Paging and random-access procedure

As summarized below, RAN1 decided to support frequency domain multiple access (FDMA) for device-to-reader (D2R) transmission, while FDMA is not supported for reader-to-device (R2D) transmission [4].

Agreements – RAN1#120 - Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

Based on the above agreements, the following scenario can be considered.

Figure 1. Example scenario of FDMA applied to D2R transmission

As shown in Fig.1, if FDMA is applied, the A-IoT device needs to support the transmission of D2R, e.g., Msg1 transmission, on specific frequencies. In other words, the reader should specify the frequency for D2R transmission to the A-IoT device. We think the simplest approach is to include a list of frequency information for D2R transmission in the A-IoT paging message. Then, when the A-IoT device receives the A-IoT paging message, the device can randomly select an access occasion and the frequency to transmit Msg1 (the same may apply to Msg3).

Proposal 9: A-IoT paging message should include a list of frequency information for D2R transmission, e.g., Msg1 transmission.

Meanwhile, during the RAN2#129 meeting, RAN2 agreed that two approaches could be considered for the R2D transmission, which determines the Msg1 resource(s) after the initial paging message.

Agreement - RAN2#129 From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

We think that it is also necessary to discuss whether, for the very first Msg1 resource, an additional R2D transmission (regardless of whether it is a new R2D message or a reused paging message) is needed, or if the initial paging message itself is sufficient to indicate the first Msg1 resource. Since receiving an R2D transmission consumes device energy, it may be more energy-efficient to have the initial paging message directly indicate the first Msg1 resource, rather than introducing an additional R2D transmission after the paging message. Moreover, since the first Msg1 resource occurs almost right after the initial paging message, there should not be significant scheduling issues for the first Msg1 resource. Therefore, we note that whether the initial A-IoT paging message can directly indicate the first Msg1 resource should be discussed. As mentioned above, we believe that the initial paging message can directly indicate it.

Proposal 10: A-IoT paging message should directly indicate the first Msg1 resource (i.e., the start of the first access occasion) without requiring an additional R2D transmission.

3.	Conclusion

This contribution makes the following observations and proposals:

Proposal 1: A-IoT device should ignore all new requests and R2D messages addressed to itself that are not associated with the ongoing procedure, meaning that option (a) should be supported.

Proposal 2: RAN2 should introduce a method to indicate to the device the end of the procedure.

Proposal 3: Reader should indicate the end of the ongoing procedure to the A-IoT devices through an R2D message, and the A-IoT devices should adhere to this indication.

Proposal 4: A-IoT device should run a timer to determine the end of the procedure for the case of the indication of the end of the procedure is missed.

Proposal 5: A-IoT R2D messages (e.g., paging messages) should include transaction ID to specify which ongoing procedure the R2D message is associated with.

Proposal 6: RAN2 considers the multiple-reader scenario when determining how the reader generates the transaction ID.

Proposal 7: Considering the multiple-reader scenario, the transaction ID should have sufficient size (e.g., 8 bits) to accommodate information related to the reader.

Proposal 8: To ensure forward compatibility with scenarios where multiple paging identifiers can be contained in a single paging message, A-IoT paging message should include a reserved field to accommodate multiple paging identifiers.

Proposal 8a: If a reserved field for multiple paging identifiers is defined, the A-IoT paging message should include a ‘length information field’ for each identifier stored in the field.

Proposal 9: A-IoT paging message should include a list of frequency information for D2R transmission, e.g., Msg1 transmission.

Proposal 10: A-IoT paging message should directly indicate the first Msg1 resource (i.e., the start of the first access occasion) without requiring an additional R2D transmission.

4.	References

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

- R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- R1-2501682, Final Report of 3GPP TSG RAN WG1 #120 v1.0.0