3GPP TSG-RAN WG2 Meeting #130		R2-2503373

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	vivo

Title:	Discussion on data collection

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

At the RAN2#129-bis meeting, the following highlighted agreements were reached regarding UE- and NW-side data collection:

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

In this contribution, we will further discuss AI/ML mobility-specific issues related to data collection.

- Discussion

- 2.1 RS type

In NR, the reference signal for RRM measurement can be SSB or CSI-RS. In the AI-based beam management use case, these two types of reference signals are supported. However, for AI mobility, SSB was selected as the reference signal for simulation since the mobility is typically based on SSB. Thus we understand that at least SSB-based measurement prediction should be supported in AI-based mobility.

Furthermore, RAN2 can discuss whether CSI-RS is also considered.

- For the reference signal type in AI/ML-based mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

- 2.2 UE-side data collection

Regarding UE-side data collection for AI/ML-based beam management, RAN2 reached the following agreements:

Agreements @ RAN2#129 Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). Agreements @ RAN2#129-bis The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Similarly to AI/ML-based beam management, the UE needs to request the data collection configuration for UE-sided model training. We understand that the data collection schemes for beam management can be the baseline for AI mobility.

Therefore, we propose:

- The data collection configuration schemes for AI/ML-based beam management can be the baseline for UE-sided model training of AI/ML-based mobility, including:

- The network configures whether UE is allowed to initiate the request for data collection configuration, which may include a list of candidate configurations.

- The UE sends a request for data collection configuration via UAI (e.g. start/stop, preferred configuration from the list of candidate configurations), if configured.

- RS configuration(s) and associated ID(s) (if needed) can be included in the data collection configuration.

- The network decides when to start/stop the data collection by provision of configuration.

Furthermore, in AI/ML-based beam management, only intra-cell prediction of the serving cell is supported, thus the request for data collection configuration is only for the serving cell. As to AI/ML-based mobility, the prediction of neighbor cells is also required. Correspondingly, the UE should be able to request the data collection configuration for both serving and neighbor cells.

- The UE can request the data collection configuration for both serving and neighbor cells.

- 2.3 NW-side data collection

2.3.1 Data collection framework

As to the NW-side data collection for AI/ML-based beam management, the following description based on agreements were captured in the running CR [1]:

For data collection for NW-side model training, data collection can be initiated by OAM or by gNB. The following enhancements are introduced for data collection for model training over air interface: - The UE can be configured by gNB to log L1 measurement results in RRC message. - The UE can be configured by gNB to report logged data or L1 measurement results in RRC message. Editor's note: FFS if dynamic activation/deactivation is support. - Both periodic and radio condition-based event-triggered data logging are supported. UE can be configured to perform data logging based on the trigger of L3 measurement event. The UE stores the logged data at the AS layer buffer. When the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection purpose, and UE indicates the data availability, if configured, to the network. The UE can also send this data availability indication when a threshold of UE buffer is reached or when UE low power state is detected, if configured. Upon reception of the availability indication, the network can request the UE to transmit the available data, as specified in TS 38.331[12]. - To reduce UE power consumption, when low power state is detected, the UE can indicate the low power state to the network, if configured. Upon reception of the low power state indication, the network should de-configure UE to release the data collection configuration. NOTE 1: It is up to UE implementation how buffer threshold reached and low power state is determined. - Network can configure UE whether the logged data should be kept or not during handover. When configured, UE can retrain the logged data during handover and indicate the availability of logged data to network after handover. Upon UE transitioning to RRC_IDLE/INACTIVE or UE experiencing RLF, UE discards any stored data.

At the last RAN2 meeting, it was agreed to use the NW-side data collection procedures in AI/ML for PHY as a baseline. From our understanding, all the aspects can be reused with some updates:

- The NW-side data collection schemes for AI/ML-based beam management can be reused as the baseline for AI/ML-based mobility, including:

- Data collection can be initiated by OAM (via immediate MDT) or by gNB.

- The gNB configures UE to log multiple instances of RRM measurement results.

- Both periodic and event-triggered data logging are supported.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- UE may indicate the data collection status to gNB, e.g., the buffer becomes full or buffer reaches a threshold if configured.

- When low power state is detected, the UE can indicate the low power state to the gNB, if configured. Upon reception of the low power status indication, the network should de-configure UE to release the data collection configuration.

- During handover the UE can retain the logged data and the UE can indicate the availability of logged data to the network and the network decides whether the logged data should be kept or not.

- Upon UE transitioning to RRC_IDLE/INACTIVE or UE experiencing RLF, UE discards any stored data.

For the event-triggered data logging, the following A1/A2-like events were introduced in the AI/ML-based beam management.

All agreements for NW-side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

The mechanism is utilized to collect the data of specific areas (e.g., cell center or cell edge) within the serving cell. Based on [2], the event-triggered logging could be useful to collect a complete dataset, avoid redundant data collection, save air interface resources, and save UE power. The intention is also valid to AI/ML-based mobility. Furthermore, the events can be further extended to include the events related to neighbor cell. As measurement event A1-A6 were agreed to be supported in AI/ML-based mobility and the measurement event prediction can up to NW implementation, all the events similar to A1-A6 should be supported for event-triggered data logging to improve the efficiency of data collection.

- In the NW-side data collection for AI/ML-based mobility, the following events (similar to events A1-A6) should be supported for event-triggered data logging, including:

- Serving becomes better than a threshold for TTT

- Serving becomes worse than a threshold for TTT

- Neighbour becomes offset better than Serving for TTT

- Neighbour becomes better than a threshold for TTT

- Serving becomes worse than threshold1 and neighbour becomes better than threshold2 for TTT

- Neighbour becomes offset better than SCell for TTT

Another critical issue is whether the new mechanisms are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication. Although the issue was raised in the AI/ML for PHY, we are not sure whether they will consider the forward compatibility for all the enhancements since the issue has been postponed.

Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later.

From our understanding, it’s not expected that UE will implement one dedicated memory for each AI/ML-based feature. That is, the AS layer memory and the associated buffer threshold are shared for all AI-related use cases. Further, as the logged data availability can be based on the buffer status, the logged data availability should be per UE granularity, i.e., not per use case.

For the low-power state indication, the data collection for different training purposes should be with the same priority from UE perspective, i.e., UE expects the network to release all the data collection configurations for training purposes when UE is in a low power state.

- The AS layer memory and the associated buffer threshold are shared for all AI-related use cases (including AI/ML-based beam management and mobility).

- The logged data availability and low-power state indications are per UE, i.e., not per use case. Upon reception of the low power status indication, the network should release all the data collection configurations for training purposes (including AI/ML-based beam management and mobility).

2.3.2 Contents of collected data

Regarding the contents of collected data for the NW-sided model, the data required for model training includes inputs and labels. The inputs and labels vary for different sub-use cases in different scenarios, so the content of data collected on the network side needs to be discussed per sub-use case and scenario. Currently, further study of the specification impact on the following scenarios is agreed for the NW-sided model:

Table 2.3-1: Sub-use cases to continue study for specification and RAN2 impact

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

The input and label of temporal domain prediction case A/B are illustrated in Table 2.3-2. Furthermore, the input is a sequence of multiple measurement instances at consecutive timing. Based on the simulation results, the gain of the cluster approach against the single-cell approach is not clear for temporal domain prediction, the label is the beam-level or cell-level measurements of the same cell.

Table 2.3-2: Input and label for temporal domain prediction case A/B

| Sub-use case | Input | Label |
| --- | --- | --- |
| 1/4 | L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of the same Cell |
| 2 | L3 Cell-level RSRP | L3 Cell-level RSRP of the same Cell |
| 3 | L1-filtered beam-level RSRP | L3 Cell-level RSRP of the same Cell |
| 5 | L3 beam-level RSRP | L3 beam-level RSRP of the same Cell |
| 6 | L1-filtered beam-level RSRP | L3 beam-level RSRP of the same Cell |

For frequency prediction, the input is the measurement from Frequency 1, and the label is the measurement from another frequency, i.e., Frequency 2. Based on the simulation results, the cluster approach can improve the prediction accuracy. Therefore, the input may include measurements from multiple cells.

Table 2.3-3: Input and label for frequency domain prediction

| Sub-use case | Input | Label |
| --- | --- | --- |
| 1/4 | L1-filtered beam-level RSRP of Frequency 1 | L1-filtered beam-level RSRP of Frequency 2 |
| 2 | L3 Cell-level RSRP of Frequency 1 | L3 Cell-level RSRP of Frequency 2 |
| 3 | L1-filtered beam-level RSRP of Frequency 1 | L3 Cell-level RSRP of Frequency 2 |
| 5 | L3 beam-level RSRP of Frequency 1 | L3 beam-level RSRP of Frequency 2 |
| 6 | L1-filtered beam-level RSRP of Frequency 1 | L3 beam-level RSRP of Frequency 2 |

For spatial prediction, the input is the measurement of the part of beams to reduce overall measurement effort. Furthermore, based on the simulation results of AI/ML-based beam management, the performance of intra-cell prediction is good enough and the cluster approach is not needed. Therefore, the label is the measurement of the same cell.

Table 2.3-4: Input and label for spatial domain prediction

| Sub-use case | Input | Label |
| --- | --- | --- |
| 1/4 | Set B of L1-filtered beam-level RSRP | Set A of L1-filtered beam-level RSRP of the same Cell (Set A and Set B are different) |
| 3 | Set B of L1-filtered beam-level RSRP | L3 Cell-level RSRP of the same Cell |
| 6 | Set B of L1-filtered beam-level RSRP | L3 beam-level RSRP of the same Cell |

As shown in Table 2.3-2 to Table 2.3-4, the input/label of different scenarios/use cases are combinations of several types of measurements, i.e., L1-filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP. This means that as long as the network obtains several types of basic measurement results, it can train models for specific scenarios based on implementation.

Specifically, for the temporal domain and spatial domain, only intra-cell prediction is considered. The UE can be configured to collect and report time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP. The network can then train models for all sub-use cases as highlighted red and green in Figure 2.3-1. On this basis, for frequency domain prediction, the UE needs to provide measurement results of one or more cells on another frequency, also including time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP as highlighted blue in Figure 2.3-1.

Figure 2.3-1: Unified solution of NW-side data collection for all sub-use cases

Therefore, we propose:

- For NW-side data collection, introduce a unified solution for all sub-use cases and scenarios, i.e., the sub-use case and scenarios can be agnostic in the specification.

- As to the contents of NW-side data collection:

- To support temporal domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of the same cell at multiple time instances.

- To support spatial domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of the same cell.

- To support frequency domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of two or more cells on different frequencies.

- How to ensure the alignment of L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP can be discussed during the WI phase.

- Conclusion

In the contribution, we have the following observations and proposals:

RS type

- For the reference signal type in AI/ML-based mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

UE-side data collection

- The data collection configuration schemes for AI/ML-based beam management can be the baseline for UE-sided model training of AI/ML-based mobility, including:

- The network configures whether UE is allowed to initiate the request for data collection configuration, which may include a list of candidate configurations.

- The UE sends a request for data collection configuration via UAI (e.g. start/stop, preferred configuration from the list of candidate configurations), if configured.

- RS configuration(s) and associated ID(s) (if needed) can be included in the data collection configuration.

- The network decides when to start/stop the data collection by provision of configuration.

- The UE can request the data collection configuration for both serving and neighbor cells.

Framework of NW-side data collection

- The NW-side data collection schemes for AI/ML-based beam management can be reused as the baseline for AI/ML-based mobility, including:

- Data collection can be initiated by OAM (via immediate MDT) or by gNB.

- The gNB configures UE to log multiple instances of RRM measurement results.

- Both periodic and event-triggered data logging are supported.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- UE may indicate the data collection status to gNB, e.g., the buffer becomes full or buffer reaches a threshold if configured.

- When low power state is detected, the UE can indicate the low power state to the gNB, if configured. Upon reception of the low power status indication, the network should de-configure UE to release the data collection configuration.

- During handover the UE can retain the logged data and the UE can indicate the availability of logged data to the network and the network decides whether the logged data should be kept or not.

- Upon UE transitioning to RRC_IDLE/INACTIVE or UE experiencing RLF, UE discards any stored data.

- In the NW-side data collection for AI/ML-based mobility, the following events (similar to events A1-A6) should be supported for event-triggered data logging, including:

- Serving becomes better than a threshold for TTT

- Serving becomes worse than a threshold for TTT

- Neighbour becomes offset better than Serving for TTT

- Neighbour becomes better than a threshold for TTT

- Serving becomes worse than threshold1 and neighbour becomes better than threshold2 for TTT

- Neighbour becomes offset better than SCell for TTT

- The AS layer memory and the associated buffer threshold are shared for all AI-related use cases (including AI/ML-based beam management and mobility).

- The logged data availability and low-power state indications are per UE, i.e., not per use case. Upon reception of the low power status indication, the network should release all the data collection configurations for training purposes (including AI/ML-based beam management and mobility).

Contents of NW-side data collection

- For NW-side data collection, introduce a unified solution for all sub-use cases and scenarios, i.e., the sub-use case and scenarios can be agnostic in the specification.

- As to the contents of NW-side data collection:

- To support temporal domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of the same cell at multiple time instances.

- To support spatial domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of the same cell.

- To support frequency domain prediction, the UE can be configured to report a list of time-aligned L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP of two or more cells on different frequencies.

- How to ensure the alignment of L1-filtered beam-level RSRP, L3 beam-level RSRP, and/or L3 cell-level RSRP can be discussed during the WI phase.

- Reference

- R2-2503398	Introduction of AI/ML for NR Air interface feature in TS38.300

- R2-2501330	Report on [POST128][019][AI PHY] NW side data collection (Nokia)