3GPP TSG-RAN2 Meeting#130	R2-2503376

# Malta, Malta, May 19th - May 23rd, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

2.1.1 RRC based (De-)Activation

Based on the latest R1 progress

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

For the FFS point on R2, considering the following R2#127bis conclusion

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

It is already confirmed by R2, no further conclusion is needed from R2 side.

- R2 confirmed the feasibility of RRC-based deactivation.

Another issue is that R1 has concluded to use separate capability for RRC-based and MAC-CE based OD-SSB A/D.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) | FFS |
| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2. Supported time domain relation between on-demand SSB and always-on SSB | FFS |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB | 61-2 |
| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) [2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] | FFS |
| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2.Supported time domain relation between on-demand SSB and always-on SSB [3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] | FFS |
| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency | 61-4 |

There is a need to differentiate the behavior for UEs supporting 61-1/2/2a and 61-3/4/4a only

- For UE supporting 61-1/2/2a only, it relies on RRC only to set the A/D status of OD-SSB.

- For UE supporting 61-3/4/4a only, it relies on MAC-CE to set the A/D status of OD-SSB, but that is not sufficient, since the OD-SSB parameter setting anyway comes from RRC first, and thus before MAC-CE arrival, UE has to know whether the configured OD-SSB is activated or not.

Therefore, it is more reasonable to assume UE supporting 61-3/4/4a (MAC-CE based A/D) relies on RRC to set initial state of OD-SSB, but leave to MAC-CE for the subsequent A/D status setting.

For this reason, R1 indicates that R2 can further discuss different RRC-based capability (in R1-2502961), i.e., the capability for RRC setting initial A/D state and RRC setting initial and subsequent A/D state have to be differentiated.

Note: whether there is a need to define separate UE capabilities for different RRC-based signaling mechanisms (if any) for indication of on-demand SSB transmission is up to RAN2

- For 61-3/4/4a, UE relies on RRC to set initial A/D state of OD-SSB, and relies on MAC-CE for subsequent A/D control.

2.1.2 MAC-CE based (De-)Activation

Furthermore, for deactivation: R1 confirmed the two options above, i.e., MAC-CE may or may not be used for OD-SSB deactivation. In 129, one concern raised by companies is that there could be cases where

- For cell-A, the OD-SSB deactivation is controlled by MAC-CE, so a bit 0 in MAC-CE means deactivation;

- For cell-B, the OD-SSB deactivation is controlled by RRC, so a bit 0 in RRC means nothing;

And thus, concern is raised regarding how to the two different types of bit 0 co-exist in the same MAC-CE. In our view, it can be easily solved by a per-cell RRC configuration, i.e., regarding whether it is controlled by MAC-CE or not.

For activation: R1 concluded that

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

·	Scenario #3B and Case #1

o	In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

·	Scenario #3B and Case #2

o	In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

I.e., R1 has concluded to support the ‘updating’ scenario, i.e., change SSB burst periodicity without deactivation first. Note that there is no difference between the two cases from MAC-CE design perspective

- MAC-CE is to activate the OD-SSB, which is currently deactivated

- MAC-CE is to change the periodicity of an OD-SSB, which is currently activated

Since in both cases, a bit-1 would indicate the presence of associated configuration / parameter (including periodicity) field, and OD-SSB is (re)activated as a result of the MAC-CE transmission.

- R2 agree that MAC-CE can be used for SSB periodicity change, i.e., change the periodicity of an OD-SSB that is being transmitted.

For the MAC-CE format design, R1 agreed that

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

The cases can be summarized as follows

| For the concerned cell | The OD-SSB was activated | The OD-SSB was deactivated |
| --- | --- | --- |
| Bit = 1, configuration ID points to an entry with N configured | How for UE to behave upon reception of a configuration ID if the concerned OD-SSB has been activated. | OD-SSB activation, to be deactivated when reaching N or via another explicit de-activation MAC-CE later |
| Bit = 1, configuration ID points to an entry without N configured |  | OD-SSB activation, to be deactivated via another explicit de-activation MAC-CE later |
| Bit = 0 | OD-SSB Deactivation | Keep OD-SSB as deactivated |

In order to simplify UE operation, i.e., no need to remember previous configuration:

Issue-1: How for UE to behave in case the configuration ID is the same, and the new configuration ID points to an entry with N configured

The key issue here is whether to allow differentiation between maintain (no restart of N counting) and re-activation (restart of N counting). It is preferred to always go for re-activation.

Issue-2: how for UE to behave in case the configuration ID is different, and the new configuration ID points to an entry with N configured

Surely it may aim at periodicity change, SSB position re-configuration or N reconfiguration. It is preferred for UE to apply the new configuration and restart N counting.

Issue-3: how for UE to behave in case the configuration ID is the same, and the new configuration ID points to an entry without N configured

No doubt the OD-SSB is kept as activation, no issue related to N at all.

Issue-4: how for UE to behave in case the configuration ID is different, and the new configuration ID points to an entry without N configured

Surely it may aim at periodicity change, SSB position re-configuration. It is preferred for UE to apply the new configuration and stop N counting, i.e., OD-SSB can be deactivated only via explicit manner.

- For OD-SSB MAC-CE, when bit for a cell is set to 1, the associated configuration index is present.

- For OD-SSB MAC-CE, when bit for a cell is set to 1, UE always apply the corresponding OD-SSB configuration and restart N counting (if N is configured for the corresponding configuration ID) or stop/re-set N counting (if N is not configured for the corresponding configuration ID) according to the configuration index.

- For OD-SSB MAC-CE, when bit for a cell is set to 0, the associated configuration index is absent, and OD-SSB is deactivated (if not yet).

  - Issue-2: L3 Measurement of OD-SSB

Comparing Case#1 and Case#2,

- For OD-SSB, hopefully the conclusion of Case#1 can be reused as much as possible to OD-SSB of Case#2, i.e., for the issues of OD-SSB itself, R2 can prioritized the discussion of Case#1 to simplify the discussion.

- On the other hand, R2 can focus on the relationship between OD-SSB and AO-SSB in Case#2, to clarify the Case#2 specific issues, as complementary part of conclusion of OD-SSB.

2.2.1 Case#2-Specific Issues

For case-2, R1 is still working on the feasibility of different frequency scenario

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

R1 already confirmed the feasibility of same-frequency scenario for AO/OD-SSB in Case-2 (when AO-SSB is NCD-SSB), and FFS when AO-SSB is CD-SSB.

Considering that, R2 can proceed for the same-frequency scenario in the coming meeting, e.g., regarding whether to allow combined measurement on always-on SSB and OD-SSB.

Figure 1 An example of Case-2 (AO-SSB and OD-SSB are partially overlapping)

Based on R1 conclusion, Figure-1 gives an example of partially overlapping AO/OD-SSB, e.g., AO-SSB/OD-SSB is of 40ms/20ms periodicity, with same offset, so it leads to 3 SSB bursts which are overlapping between each other. In this case, it is hard to differentiate between AO-SSB and OD-SSB, at the overlapping places.

Furthermore, R2 defined a L3 measurement framework in the following manner, I.e., the configuration of AO-SSB and OD-SSB have to be carried via a same MO.

The network applies the procedure as follows:

[…]

- to ensure that, in the measConfig associated with a CG:

- for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

Combining the two, i.e., same MO and hard to differentiate the SSB attributive (AO-SSB or OD-SSB), it seems easier to perform a combined L3 measurement, and generate a single MR. But that is to be further evaluated by R4, and R2 only needs to focus on MO configuration and MR report.

Considering R4 latest conclusion supporting combined measurement for C1, R2 can firstly conclude on C1. And wait for R4 conclusion on C2.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1

Agreement:

- RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario.

  - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification.

  - FFS for other procedures.

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

- For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 firstly conclude the support of L3 measurement configuration based on a single MO. And wait for further conclusion from R4 on whether to support C2.

If R2 would like to progress more, some key issues need to be considered.

- servingCellMO configuration granularity: to stick to the principle of single-MO for single SSB frequency, it is straightforward to use two separate MOs for inter-frequency case. While the remaining issue is whether to apply this to per-cell MO (for which both intra- and inter-frequency scenarios need to be considered) only or per-BWP MO (for which only intra-frequency scenario needs to be considered so far) as well.

Table 1 AO-/OD-SSB scenarios based on latest R1 conclusion

| AO-SSB Case | AO-SSB is CD or NCD-SSB | AO-SSB is on sync raster or not | Is it applicable to servingCellMO in ServingCellConfig | Is it applicable to servingCellMO in BWP-DownlinkDedicated | RAN1 concluded scenario for AO-SSB and OD-SSB |
| --- | --- | --- | --- | --- | --- |
| Case-1 | CD | Yes | Yes | No | Inter-frequency |
| Case-2 | NCD | Yes | Yes | Yes | Intra-frequency |
| Case-3 | CD | No | Yes | No | Intra-frequency |
| Case-4 | NCD | No | Yes | Yes | Intra-frequency |

- MO Applicability: And when both SSB types are available, for separate frequency case, it is straightforward to rely on the OD-SSB, similar to the legacy scenario when CD-SSB and NCD-SSB co-exist. Following this principle, the applicable SSB would be used to define the intra-/inter-frequency measurement definition.

- Gap applicability: there is no remaining issue for gap necessity, since that no matter OD-SSB is configured as per-cell or per-BWP, it has to stick to the R1 conclusion that AO-SSB and OD-SSB are located in the same BWP. So that the gap necessity can follow legacy rule.

Other than that, i.e., separate OD-SSBs for different active BWPS (which ends up with multiple OD-SSBs per cell), we do not see the motivation to consider multiple OD-SSBs for different frequencies, which is neither discussed in R1 nor in R4 yet (NOTE that this meeting is the last one for R1).

- For OD-SSB Case#2, R2 further discuss whether to include it in BWP-DownlinkDedicated as well. Other than that, R2 does not pursue multiple OD-SSBs with different frequencies for a given cell.

- For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 confirm to rely on OD-SSB for the measurement when OD-SSB is being transmitted (which is anyway pending R4 decision on whether to support or not).

Besides, there was one open issue raised, i.e., how to handle the ssb-ToMeasure

If the od-ssb-PositionsInBurst of OD-SSB and AO-SSB is different, how the UE adapts the ssb-ToMeasure of MeasObiectNR after OD-SSB is activated

According to R1 agreement, the case above only happens for different frequency case, i.e., different frequency for AO and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Considering that

- Within FMW (Fast-Mode-Window), the measurement would be based on the indicated od-ssb-PositionsInBurst via RRC/MAC-CE;

- Out of FMW, the UE fall back to legacy mode as concluded by R4, so that regardless of the OD-SSB periodicity and Positions-In-Burst, the legacy measCycleSCell and ssb-ToMeasure are to be used. And thus the setting of both parameters can be left to network implementation.

- For OD-SSB L3 measurement, the setting of ssb-ToMeasure is up to network implementation.

2.2.2 Common Issues for Case#1/Case#2

For L3 measurement, since it is possible that serving cell and neighbouring cell share the same MO configuration, e.g., in an intra-frequency scenario, it is good to understand the behavior for neighboring and serving cell scenario.

For neighbouring cell, it anyway needs to rely on SMTC window to identify the time location of SSB searching. Besides, when it comes to OD-SSB, it is anyway infeasible for UE to know the neighboring cell OD-SSB (de)activation status. Combing the two, it seems more realistic to rely on Case-2, if network would like to configure the UE to perform neighboring cell measurement. Anyway, there seems not much work that R2 can do for the neighbouring cell measurement under OD-SSB objective, and it aligns with R4 conclusion below

Issue 1-1-2: OD-SSB impact on neighbour cell measurement

Agreement:

- For the RRM measurement on neigboring cell, the legacy meaurement framework is applied.

  - Note: The discussion of SMTC configuration is a separate issue.

- This agreement can be revisited if it is not aligned with further agreement in other WGs.

- For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Another issue is whether to rely on legacy event-based MR triggering or rely on MAC-CE based MR triggering.

We understand that MAC-CE based MR triggering was introduced for FR2 unknown SCell, while OD-SSB can be sent before SCell activation, and thus enable known case, as long as the L3 MR triggering event has been configured properly (regardless of whether the AO and OD-SSB are on the same or different frequencies), which would lead to an even quicker SCell activation compared with the “semi-known” case of SCell A/D MAC-CE triggered MR .

Considering that, the motivation of introducing MAC-CE based MR triggering is not strong. And R4 is discussing this issue

Issue 1-4-11: OD-SSB based L3 reporting

- Proposals

  - Option 1: Qualcomm, CMCC

    - RAN4 not introduce OD-SSB based L3 measurement reporting.

  - Option 2: Huawei

    - RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation.

- Recommended WF

  - Further discussion

Please note that R4 will address the case that SCell A/D MAC-CE triggered MR for OD-SSB in CR phase, which is different from the OD-SSB A/D MAC-CE triggered MR.

Issue 2-1-4: Rel-18 SCell Activation with the L3 reporting during activation

Agreement

- FFS RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

- R2 not pursue OD-SSB A/D MAC-CE triggered MR for OD-SSB.

- Conclusion

Proposal 1	R2 confirmed the feasibility of RRC-based deactivation.

Proposal 2	For 61-3/4/4a, UE relies on RRC to set initial A/D state of OD-SSB, and relies on MAC-CE for subsequent A/D control.

Proposal 3	R2 agree that MAC-CE can be used for SSB periodicity change, i.e., change the periodicity of an OD-SSB that is being transmitted.

Proposal 4	For OD-SSB MAC-CE, when bit for a cell is set to 1, the associated configuration index is present.

Proposal 5	For OD-SSB MAC-CE, when bit for a cell is set to 1, UE always apply the corresponding OD-SSB configuration and restart N counting (if N is configured for the corresponding configuration ID) or stop/re-set N counting (if N is not configured for the corresponding configuration ID) according to the configuration index.

Proposal 6	For OD-SSB MAC-CE, when bit for a cell is set to 0, the associated configuration index is absent, and OD-SSB is deactivated (if not yet).

Proposal 7	For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 firstly conclude the support of L3 measurement configuration based on a single MO. And wait for further conclusion from R4 on whether to support C2.

Proposal 8	For OD-SSB Case#2, R2 further discuss whether to include it in BWP-DownlinkDedicated as well. Other than that, R2 does not pursue multiple OD-SSBs with different frequencies for a given cell.

Proposal 9	For OD-SSB Case#2, for different-frequency scenario between AO-SSB and OD-SSB, R2 confirm to rely on OD-SSB for the measurement when OD-SSB is being transmitted (which is anyway pending R4 decision on whether to support or not).

Proposal 10	For OD-SSB L3 measurement, the setting of ssb-ToMeasure is up to network implementation.

Proposal 11	For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Proposal 12	R2 not pursue OD-SSB A/D MAC-CE triggered MR for OD-SSB.