3GPP TSG-RAN WG2 Meeting #130	R2-2503379

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#129-bis meeting, RAN2 achieved the following agreements about the impacts on random access operation by the evolution of duplex operation:

Output of email discussion [Post129][217] - On RO type signaling for CFRA 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. - When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. - Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. On RSRP threshold for RO type selection (e.g., configuration, related procedure, etc.) - NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. RO type selection before or after RA type selection - Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. On RO type fallback - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we will further analyse the remaining RAN2 impacts to support random access in SBFD symbols.

# 2	Discussion

In the RAN1#120-bis meeting, there was conclusion about RACH configuration Option 1, as following:

Conclusion For RACH configuration Option 1, there is no consensus in RAN1 to support separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs under the same FeatureCombinationPreambles configuration and it’s up to RAN2 to decide whether to support separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs.

As can be seen, the issue of whether to introduce separate featureCombinationPreamblesList is left to RAN2. For RACH configuration Option 1, both Additional-ROs and Legacy-ROs are configured together, and the ssb-SharedRO-MaskIndex is used to configure which ROs can be allocated for a feature/feature combination. According to the RAN1 conclusion, there is no consensus to support separate ssb-SharedRO-MaskIndex configuration for different type of ROs, then the additional-ROs and legacy-ROs can be treated similarly regarding allocating preambles for one feature/feature combination. Note that there was RAN2 agreement that there is no need to introduce SBFD as a new feature within the current PRACH preamble partitioning framework. There would be very limited benefit to introduce separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs in case of RACH configuration Option 1.

In case of RACH configuration Option 2, it shall be possible to support separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs naturally, as the gNB can configure the featureCombinationPreamblesList for SBFD RACH resource independently. There is no point in this case to restrict gNB to use the same featureCombinationPreamblesList for two different configuration instances.

Proposal 1: RAN2 confirms that for RACH configuration Option 1, there is no need to introduce separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs; and for the RACH configuration Option 2, separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs can be supported without restriction.

Note that the featureCombinationPreambles IE is used to define a set of Random Access resources. Thus, RAN2 can confirm that for RACH configuration Option 1, the SBFD RACH resources and legacy SBFD RACH resources associated with same feature or feature combination are treated as the same set of RACH resources.

Proposal 2: RAN2 confirms that for RACH configuration Option 1, the SBFD RACH resources and legacy RACH resources associated with same feature or feature combination are the same set of RACH resources.

While for RACH configuration Option 2, the legacy RACH resources and the SBFD RACH resources are configured separately, by different configuration instances. It can be understood that, the SBFD RACH resources and legacy SBFD RACH resources associated with same feature or feature combination are treated as the two separate sets of RACH resources. That is, there may be one RACH resource set of SBFD type, and another RACH resource set of non-SBFD type in case of RACH configuration Option 2.

Proposal 3: RAN2 confirms that for RACH configuration Option 2, the SBFD RACH resources and legacy RACH resources associated with same feature or feature combination are treated as the two separate sets of RACH resources.

In the current MAC running CR, there are several Editor’s Note related to the RO type fallback specification:

For the Editor’s Note regarding RA resource set reselection after the RO type switching, RAN2 can discuss whether to reselect the RACH resource set and how to reselect the RACH resource after the RO type switching.

According to the above discussion, the RACH resource sets may be separately configured for RACH configuration Option 2. Thus, we understand when the condition of RO type fallback is met, i.e. the PREAMBLE_TRANSMISSION_COUNTER reach the configured fallback threshold, the UE may need to select the set of Random Access resources including the different types of RO for the current RACH re-attempt associated with the same feature or feature combination for this Random Access procedure.

Also considering the Editor’s note related to Msg1 repetition, we think Msg1 repetition and Msg1 repetition number fallback would be common for a RACH procedure, hence it is necessary to support Msg1 repetition number fall back with SBFD RO otherwise the usage of SBFD could be heavily limited. Of course specific considerations are needed as shown in the ENs.

If the current RACH attempt is performed with preamble repetition, the RACH resource set may need to be re-selected, since the RACH resource sets related to different repetition number are different and the thresholds for legacy RO and SBFD RO can be different, even for RACH configuration Option 1.

Proposal 4: After the RO type switching, the UE shall select the set of Random Access resources including different types of RO for the current RACH re-attempt associated with the same feature or feature combination for this Random Access procedure.

Regarding how to determine the target RACH resource set during RO type switching when Msg1 repetition number fallback and SBFD RO are supported together, there could be two straightforward options:

Option 1: the UE reselects the RACH resource set same as the initial RACH resource set selection.

For this option, the UE will compare the RSRP with the threshold configured for the target RO type, which is similar as the selection of the initial RACH resource set. The needed new procedure can simply refer to the initial selection procedure which UE will perform again.

Option 2: the UE reselects the RACH resource set considering the current Msg1 repetition number.

For this option, the previous unsuccessful RACH attempts are also considered, since the UE may have already tried several times under one indicated Msg1 repetition number and have switched to a higher Msg1 repetition number on the current type of RO. If the UE performs RACH attempt with the starting/lower Msg1 repetition number after RO type switching, the access delay may increase because there is a high probability that the UE fails under the lower Msg1 repetition number than current attempt.

As a reference, upon fallback from CFRA with repetition to CBRA with repetition, the UE only selects the RACH resources that associated the same Msg1 repetition number that indicated for CFRA. As another reference, RAN1 has agreed that for RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support to increase the power ramping counter after the RO type switching. This indicates the new RO type’s transmission can “inherit” old RO type’s transmission property.

Thus, we can reuse the same principle for considering Msg1 repetition number after RO type fallback, to select the same or even higher Msg1 repetition number (considering the transmission with the same Msg1 repetition number has failed).

Regarding the last Editor’s note, we think the performance gain/loss of different orders of RO type fallback and Msg1 repetition number fallback is marginal and slightly prefer RO type fallback before Msg1 repetition number fallback as RO type switching may bring imediate performance change.

Proposal 5: After the RO type switching with preamble repetition, UE needs to select RACH resource set with same or higher Msg1 repetition number, i.e. fallback to lower Msg1 repetition number should be avoided.

In the RAN1#120-bis meeting, there was conclusion about RACH procedure, as following:

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary

In the previous RAN2 meetings, there was no conclusion about whether the Msg3 based early indication is supported or not. According to the latest RAN1 agreement, the UE will follow the legacy behaviour when the UE transmits Msg1 in legacy RO. Thus, it can be assumed that the UE will transmit the Msg3 in the legacy resource if the UE doesn’t transmit the Msg1 in SBFD RO and Msg3 based early identification is not supported based on the related RAN1 agreement

Proposal 6: Msg3 based early identification is not supported.

In the previous RAN2 meetings, some contributions (and offline discussions) mentioned the RA-RNTI calculation issue caused by possible overlapping SBFD RACH resource configuration.

In the RAN1#119 meeting, there was agreement about RO determination for the case that additional ROs overlap with legacy ROs:

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

In the RAN1#120 meeting, RAN1 discussed this issue and agreed to not optimize:

Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

Based on the above RAN1 conclusion, we understand RAN2 does not need to enhance the calculation of RACH-RNTI, considering that the contention resolution mechanism of RACH procedure can deal with this issue anyway, if two identical RA-RNTIs are caused by the same RO occasion (same time and frequence occasions). The gNB can recognize different UEs according to the UE identifiers provided in Msg3 (part of 5G-S-TMSI, random value, C-RNTI or I-RNTI), and perform admission control considering the content of Msg3 as UE Contention Resolution Identity for contention resolution. For the case of different time and frequence occasions are used and the identical RA-RNTIs are caused by the same time and frequence indices, the gNB would be able to identify them as different resources used by two UEs and the gNB can, e.g. through the implementation, treat the two UEs separately. In addition, from the perspective of the RAN2, it is unclear whether the overlapping ROs is invalid or not for a UE, i.e. different UE implementations may have different understanding about this situation, RAN2 cannot solve the RA-RNTI conflict issue without the involvement of RAN1. We assume it is suitable to leave this issue/solution to gNB implementation.

Proposal 7: RAN2 do not optimize on the possible RA-RNTI conflict issue.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

Observation 1: RO group is expressed as set of PRACH occasions in MAC spec, and the UE will select the next available set of PRACH occasions for the Msg1 repetition number.

Proposal 1: RAN2 confirms that for RACH configuration Option 1, there is no need to introduce separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs; and for the RACH configuration Option 2, separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs can be supported without restriction.

Proposal 2: RAN2 confirms that for RACH configuration Option1, the SBFD RACH resources and legacy RACH resources associated with same feature or feature combination are the same set of RACH resources.

Proposal 3: RAN2 confirms that for RACH configuration Option 2, the SBFD RACH resources and legacy RACH resources associated with same feature or feature combination are treated as the two separate sets of RACH resources.

Proposal 4: After the RO type switching, the UE shall select the set of Random Access resources including different types of RO for the current RACH re-attempt associated with the same feature or feature combination for this Random Access procedure.

Proposal 5: After the RO type switching with preamble repetition, UE needs to select RACH resource set with same or higher Msg1 repetition number, i.e. fallback to lower Msg1 repetition number should be avoided.

Proposal 6: Msg3 based early identification is not supported.

Proposal 7: RAN2 do not optimize on the possible RA-RNTI conflict issue.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)