3GPP TSG-RAN WG2 Meeting #130	R2-2503384

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.1.2.3

Source:	NEC

Title:	LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meeting, the following agreement was reached for LCM for Positioning use case:

Agreement for Case 1 @RAN2#129bis

1	LMF is responsible for functionality management

2	UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

3	Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

4	An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

5	For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

In RAN1#120bis meeting, the following agreement was reached for model monitoring for Positioning use case:

Agreement

For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by:

• the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

This paper discusses the remaining issues for life cycle management (LCM) for Positioning use case with AIML assistance.

# Discussion

## Performance Monitoring

In RAN1, there were many discussions during RAN1#116bis, RAN1#118 and RAN1#120bis for performance monitoring for AI positioning use case. In summary, the following agreements were reached for model performance monitoring for Position use case for UE sided model.

RAN1#116bis Agreement

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation.

  - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

    - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE.

  - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE.

  - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server).

    - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method.

- Option B. The LMF performs monitoring metric calculation.

  - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

  - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

RAN1#118 Agreement

Model monitoring related

For case 3a, both gNB and LMF performs monitoring metric calculation are feasible from RAN1 perspective. Further selection left to RAN3’s decision. Method to perform the metric is up to gNB/LMF implementation.

For case 1, RAN1 will not discuss Option A-4(monitoring data transferred from PRU to target UE with specification transparent way).

RAN1#119 Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

RAN1#120bis Agreement

For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by:

  - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

During RAN1 discussion, two approaches were discussed among companies for model performance monitoring of AI/ML positioning Case 1, one is label-free monitoring, and the other is label-based monitoring. With label-based monitoring, a sort of assistance data is required from PRUs. However, for label-free monitoring, the entity (e.g. the UE) performing the monitoring decision may require assistance data from LMF to support its performance assessment. RAN2 can wait for the input from RAN1 on the detailed definition of these approaches.

Regardless of label-based or label-free performance monitoring, there may be conditions that can trigger the need for performance monitoring at the UE side. LMF may configure the UE with particular conditions to activate the performance monitoring and to report its outcome to enable efficient LCM decisions at LMF, for the case where the monitoring metric is derived at the LMF-side.

For the cases when the UE is the entity to derive the performance monitoring metric, the UE can at least report to LMF the Model monitoring outcome. For example, as agreed in RAN1#120bis, the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

With the discussion above, it can be seen that the signalling required between UE and LMF over LPP is different from the legacy LPP messages and the corresponding information elements required in the LPP message are quite new. Hence we prefer to use new LPP signalling to host the new information elements for performance monitoring of AI/ML positioning Case 1. Then we have the following proposals:

Proposal-1: The UE is configured by LMF with the triggering conditions to enable performance monitoring for AI/ML positioning Case 1 and to report the outcome of monitoring.

Proposal-2: When the UE is the entity to derive the performance monitoring metric, a new LPP message is used for the UE to report to LMF the Model monitoring outcome (the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method).

During RAN1#120bis, extensive discussions were taken for both option A and option B based performance monitoring, however there is no final agreements yet. RAN2 can wait for RAN1 for how and which assistance data should be transmitted from the LMF to the UE and also the further content of the performance monitoring outcome for AI/ML positioning Case 1.

Proposal-3: Wait for further input from RAN1 for how and which assistance data should be transmitted from the LMF to the UE and about the further content of the performance monitoring outcome for AI/ML positioning Case 1.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: The UE is configured by LMF with the triggering conditions to enable performance monitoring for AI/ML positioning Case 1 and to report the outcome of monitoring.

Proposal-2: When the UE is the entity to derive the performance monitoring metric, a new LPP message is used for the UE to report to LMF the Model monitoring outcome (the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method).

Proposal-3: Wait for further input from RAN1 for how and which assistance data should be transmitted from the LMF to the UE and about the further content of the performance monitoring outcome for AI/ML positioning Case 1.

# Annex: Agreement made by RAN1 for AI/ML based positioning

## RAN1#116-bis agreements

In RAN1#116-bis two options for the performance monitoring agreed between companies:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

## RAN1#118bis agreements

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

## Annex: RAN1#119 agreements

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

  - starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

## Annex: RAN1#120bis agreements

Conclusion

- The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values.

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by:

- No offset is transmitted from gNB to LMF

Conclusion

For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

Agreement

For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by:

- the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Agreement

For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label,

- The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label.

Note: It is assumed that user data privacy of non-PRU UE is preserved.