3GPP TSG-RAN WG2 Meeting #130					        R2-2503391

St.Julians, Malta, May 19th – 23rd , 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#129bis and following agreements were made:

- Handling RRC connected UE

  - NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- Whether legacy UE can camp in NES Cell if it is broadcasting SIB1

  - When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- UL WUS in RRC release

  - Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

  - SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

  - Understanding of RAR format

  - Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- Backoff in RAR

  - Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

  - Paging interruption

  - Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

  - Condition to bar the cell due to SIB1 acquisition failure

  - If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- Area validity

  - WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

  - Area id will be used as legacy.

- Others

  - We do not need a separate new triggering condition of OD-SIB1 acquisition

  - When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution we discuss further details/open issues of on-demand SIB1.

# Discussion

## 2.1. UL carrier for SIB1 request

In legacy SI request is supported for both SUL and NUL. Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL in a cell supporting SUL. UE should select the UL carrier before transmitting SIB1 request. SIB1 request is based on random access and in legacy random access procedure, UL carrier selection is performed at the initiation of random access procedure. So, both NUL and SUL can easily be supported for SIB1 request without any impact to SIB1 request procedure i.e. no special handling or modification to SIB1 request procedure is needed.

Not supporting SUL for SIB1 request has more specification effort:

- a) there is impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) need to be defined in MAC spec.

- b) we need to specify that cell supporting SUL does not provide SIB1 on demand. The reason is that if cell supporting SUL provides SIB1 on demand and UE is not in NUL coverage, UE will not be able to acquire SIB1 in the absence of SIB1 request configuration for SUL.

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

## 2.2 SIB1 request in RRC_CONNECTED

Figure 1

Figure 1 illustrates the scenario of SIB1 request to the selected cell upon RLF. In RAN2#129, it was agreed that when T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

Latest RRC CR specifies the following:

- UE supporting SIB1 request acquires and maintains SIB X in RRC_CONNECTED.

- RRC system information request needs to include SIB X

Following changes are also needed

- Change 1: RRC reconfiguration message needs to include SIB X

  - This is for the case SIBX cannot be broadcasted in active DL BWP

dedicatedSystemInformationDelivery

This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB20, SIB21, SIB25, SIBx to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

- Change 2: Mechanism to validate that last acquired SIB X in current PCell is valid at the time SIB1 request is sent to cell selected for connection re-establishment.

  - One simple approach for validation is that

    - UE may send SIB1 request to Cell B (i.e. cell selected during re-establishment), if the SIB X last acquired from PCell (say Cell A) includes SIB1 request configuration of Cell B and the SIB X is valid in current modification period.

Proposal 2: Discuss and agree the following for SIB1 request to cell selected for connection re-establishment

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X cannot be broadcasted in active DL BWP)

- UE may send SIB1 request to selected cell (say cell B), if the SIB X last acquired from PCell (say Cell A) includes SIB1 request configuration of Cell B and this SIB X is valid in current modification period.

## 2.3 Cell access info in OD-SIB1 configuration

Upon receiving the on demand SIB1 during cell reselection UE may bar the cell based on information in CellAccessRelatedInfo IE and Cell barring bits in SIB1. This basically delays the cell reselection.

In our view it is beneficial to provide the following information in SIBx for a cell whose SIB1 request configuration is included in SIBx.

- List of PLMN identities associated with the cell

- List of tracking area codes associated with the cell

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG (not needed, if OD-SIB1 is not supported for ATG)

- cellBarredNES

- cellBarred-eRedCap1Rx

- cellBarred-eRedCap2Rx

- cellBarredRedCap1Rx

- cellBarredRedCap2Rx

- cellBarredNTN (not needed, if OD-SIB1 is not supported for NTN)

To reduce the signalling overhead,

- List of PLMN identities can be commonly configured. The list of PLMNs per cell can refer to this common list.

- List of tracking area codes can be commonly configured. The list of tracking area codes per cell can refer to this common list.

To not have any restriction on SIB1 request configuration to be applied to multiple cells, cell access related information can be separately signalled in SIBx instead of signalling them per SIB1 request configuration.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

plmn-IdentityListCommon      SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity OPTIONAL,        -- Need R

trackingAreaListCommon       SEQUENCE (SIZE (1..maxTAC)) OF TrackingAreaCode  OPTIONAL,     -- Need R

<add cell access info here>

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

Proposal 3a: Cell access information in SIB X is supported.

Proposal 3b: Following information is optionally included in SIBx for a cell whose SIB1 request configuration is included in SIB X.

- List of PLMN identities associated with the cell

- List of tracking area codes associated with the cell

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG (not needed, if OD-SIB1 is not supported for ATG)

- cellBarredNES

- cellBarred-eRedCap1Rx

- cellBarred-eRedCap2Rx

- cellBarredRedCap1Rx

- cellBarredRedCap2Rx

- cellBarredNTN (not needed, if OD-SIB1 is not supported for NTN)

Proposal 3c:

- List of PLMN identities can be commonly configured. The list of PLMNs per cell can refer to this common list.

- List of tracking area codes can be commonly configured. The list of tracking area codes per cell can refer to this common list.

Proposal 3d: Cell access related information can be separately signalled in SIB X instead of signalling them per SIB1 request configuration.

2.4 SIB1 request upon connection release

UE is in RRC_CONNECTED in Cell A (PCell). Cell A supports SIB X and UE acquires and maintains a valid version of the Cell A’s SIBX. Upon releasing the RRC connection, UE performs cell selection and selects Cell B.

If Cell A’s SIB X include SIB1 request configuration of Cell B, the issue is whether the UE can use that configuration for requesting SIB1 of Cell B. In our view similar to RLF case, UE should be allowed to use SIB1 request configuration from SIB X of Cell A for SIB1 request to selected Cell B after connection release.

Proposal 4: SIB1 request configuration of a cell (say cell B) from SIB X of PCell can be used for SIB1 request to a cell B selected upon connection release.

2.5 Handling of legacy and new excluded cell list

The new excluded cell list for OD-SIB1 supporting UEs was agreed so that OD_SIB1 cells can be included in legacy list inorder to avoid unnecessary measurements/MIB acquisition of OD-SIB1 cells by legacy UEs and reduce cell reselection delay.

The cells which need to be excluded by both legacy UEs and OD-SIB1 supporting UEs are included in new excluded cell list and legacy list.

If this new excluded cell list for OD-SIB1 supporting UEs is not defined and network includes OD-SIB1 cells in legacy list, both legacy UEs and OD-SIB1 supporting UEs will exclude OD-SIB1 cells which is not the intended behaviour.

Issue with current CR

The CR defines that if new excluded cell list is not signalled, OD-SIB1 supporting UEs apply legacy excluded cell list.

Scenario 1: Network want to exclude Cell A, Cell B, Cell C by legacy UEs only as they are OD-SIB1 cells. Network also wants to exclude Cell D by all UEs.

Legacy excluded cell list: Cell A, Cell B, Cell C and Cell D

New excluded cell list: Cell D

Current CR works and there is no issue. Legacy UE excludes Cell A, Cell B, Cell C and Cell D based on legacy list.  OD-SIB1 UEs excludes Cell D based on new list.

Scenario 2: Network wants to exclude Cell A, Cell B, Cell C by legacy UEs only as they are OD-SIB1 cells. There is not other common cell which needs to be excluded by both legacy UEs and OD-SIB1 UEs.

Legacy excluded cell list: Cell A, Cell B, Cell C and Cell D

New excluded cell list: not signalled.

Current CR does not work. Legacy UE and OD-SIB1 supporting UEs excludes Cell A, Cell B, Cell C. This is not the intended behaviour as network wants only the legacy UEs to exclude Cell A, Cell B, Cell C.

To resolve this issue, we propose that the OD-SIB1 supporting UEs ignores the intraFreqExcludedCellList /interFreqExcludedCellList received from a cell in which SIBxx is provided.

Proposal 5: The OD-SIB1 supporting UEs ignores the intraFreqExcludedCellList /interFreqExcludedCellList received from a cell in which SIB X is provided.

# Conclusion

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

Proposal 2: Discuss and agree the following for SIB1 request to cell selected for connection re-establishment

- SIB X can be delivered to UE using RRC reconfiguration message in RRC_CONNECTED (for the case SIB X cannot be broadcasted in active DL BWP)

- UE may send SIB1 request to selected cell (say cell B), if the SIB X last acquired from PCell (say Cell A) includes SIB1 request configuration of Cell B and this SIB X is valid in current modification period.

Proposal 3a: Cell access information in SIBX is supported.

Proposal 3b: Following information is optionally included in SIB X for a cell whose SIB1 request configuration is included in SIB X.

- List of PLMN identities associated with the cell

- List of tracking area codes associated with the cell

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG (not needed, if OD-SIB1 is not supported for ATG)

- cellBarredNES

- cellBarred-eRedCap1Rx

- cellBarred-eRedCap2Rx

- cellBarredRedCap1Rx

- cellBarredRedCap2Rx

- cellBarredNTN (not needed, if OD-SIB1 is not supported for NTN)

Proposal 3c:

- List of PLMN identities can be commonly configured. The list of PLMNs per cell can refer to this common list.

- List of tracking area codes can be commonly configured. The list of tracking area codes per cell can refer to this common list.

Proposal 3d: Cell access related information can be separately signalled in SIBx instead of signalling them per SIB1 request configuration.

Proposal 4: SIB1 request configuration of a cell (say cell B) from SIB X of PCell can be used for SIB1 request to a cell B selected upon connection release.

Proposal 5: The OD-SIB1 supporting UEs ignores the intraFreqExcludedCellList /interFreqExcludedCellList received from a cell in which SIB X is provided.

# References