3GPP TSG-RAN WG2 Meeting #130					        R2-2503392

St.Julians, Malta, May 19th – 23rd , 2025

Agenda item:	8.5.2

Source:	Samsung

Title:	On-demand SSB SCell Operation

Document for:	Discussion and Decision

# Introduction

The Rel-18 work item on network energy savings for NR specified techniques primarily for RRC Connected UEs such as SSB-less SCell operation, cell DTX/DRX mechanism, mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, CHO procedure enhancement(s) etc.

Rel-19 work item aims to specify further network energy savings techniques such as on-demand SSB for SCell operation. WID of Rel-19 network energy saving specifies the following objective for on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

This contribution discusses the details on supporting on-demand SSB on SCell.

# Discussion

## 2.1 Signalling for on demand SSB activation/deactivation

RAN2#129bis agreements

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

For OD-SSB configuration, multiple sets of following parameter can be signalled by RRC where each set is identified by a configuration index.

  - od-ssb-Periodicity

  - od-ssb-sfn-Offset

  - od-ssb- halfFrameIndex

  - od-ssb-PositionsInBurst

  - od-ssb- nrofBurst (optional, present for implicit deactivation)

This is a simple approach (only one configuration index is included in MAC CE) instead of separate configuration sets for each of the parameter.

MAC CE formats based on agreements is shown in figure 1 and figure 2.

    - Cj field is set to 1 to indicate that on demand SSB for SCell identified by ServCellIndex is activated. Cj field is set to 0 to indicate that on demand SSB for SCell identified by ServCellIndex j is deactivated.

    - One configuration index corresponding to each Cj bit set to 1. configuration indexes are included in ascending order of j for which Cj bit is set to 1.

Figure 1 (highest ServCellIndex or SCellIndex for which on demand SSB is configured is < 8)

Figure 2 (highest ServCellIndex or SCellIndex for which on demand SSB is configured is >= 8)

Since the MAC CE is for (de) activation for multiple SCells, following are the valid scenarios

- Scenario 1: UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently deactivated.

  - This is straight forward and UE does not need to take any action for the jth SCell.

- Scenario 2: UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently deactivated.

  - In this case OD-SSB is activated for the jth SCell and UE applies the OD-SSB configuration corresponding to configuration index included in the MAC CE. In case the OD-SSB configuration includes od-ssb- nrofBurst, UE will start timer/counter for implicit deactivation.

- Scenario 3: UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated.

  - If od-ssb-nrofBurst is not included in configuration applied for the activated OD-SSB

    - OD-SSB is deactivated for the jth SCell

  - If od-ssb-nrofBurst is included in configuration applied for the activated OD-SSB

    - Option 1: do nothing (this means explicit deactivation is not supported).

    - Option 2: OD-SSB is deactivated for the jth SCell. In our view, it is simple to allow this operation to have unified operation. If network does not want to deactivate, it can anyway send MAC CE with Cj bit set to 1.

- Scenario 4: UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently activated.

  - Option 1: if configuration index is different from the configuration index of the configuration currently applied for the activated OD-SSB, UE applies the new configuration based on new configuration index received in MAC CE. Otherwise, do nothing.

  - Option 2: UE do nothing.

    - This may not be a good option as it does not allow network to dynamically change configuration of activated OD-SSB.

Proposal 1: If the Cj field is set to 1, configuration index is always included in MAC CE for the SCell with ServCellIndex j. Configuration indexes are included in ascending order of j for which Cj bit is set to 1.

Proposal 2: When UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently deactivated:

- OD-SSB is activated for the jth SCell and UE applies the OD-SSB configuration corresponding to configuration index included in the MAC CE. In case the OD-SSB configuration indicated by configuration index includes od-ssb- nrofBurst, UE will start timer/counter for implicit deactivation.

Proposal 3: When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

- OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

Proposal 4: When UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently activated:

- if configuration index is different from the configuration index of the configuration currently applied for the activated OD-SSB, UE applies the new configuration based on new configuration index received in MAC CE. Otherwise, do nothing.

# Conclusion

Proposal 1: If the Cj field is set to 1, configuration index is always included in MAC CE for the SCell with ServCellIndex j. Configuration indexes are included in ascending order of j for which Cj bit is set to 1.

Proposal 2: When UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently deactivated:

- OD-SSB is activated for the jth SCell and UE applies the OD-SSB configuration corresponding to configuration index included in the MAC CE. In case the OD-SSB configuration indicated by configuration index includes od-ssb- nrofBurst, UE will start timer/counter for implicit deactivation.

Proposal 3: When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

- OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

Proposal 4: When UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently activated:

- if configuration index is different from the configuration index of the configuration currently applied for the activated OD-SSB, UE applies the new configuration based on new configuration index received in MAC CE. Otherwise, do nothing.

# References