3GPP TSG RAN WG2 Meeting #130            	                      R2-2503417

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.10.2

Source:	Xiaomi

Title:	Discussion on Rel-19 SON/MDT open issues

Document for:	Discussion and Decision

Introduction

In the post RAN2 #129bis meeting [Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson) [1], the RRC Running CR captured previous RAN2 agreements pertains to Rel-19 SON/MDT WI, and some issues are left which need to be brought for online decision.

In this contribution, we discuss some RRC left issues and summarize them for discussion, e.g., whether to indicate the correlation of SHR and SPR and avoiding duplication of the information, explicit or implicit indicator to indicate whether a neighbour cell is an LTM candidate cell. Additionally, some companies have different understandings of previous agreements, e.g., how to capture the agreement “UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs” in the draft CR.

Discussion

RRC open issues

MRO for LTM

For LTM related issue, adopting an explicit ltmCandidate indication in SHR would simplify implementation by directly flagging LTM candidates, mirroring the choCandidate approach. However, this deviates from RAN3’s agreement [2] to use a network-based implicit solution (adding a ltmCandidateList in SON reports), creating inconsistency across RLF reports and SHR. Thus, it’s proposed to align with RAN3’s agreements in RRC CR.

MRO for LTM: Omitted… Network based or UE based solution for LTM candidate cell list? SS, Lenovo, Nok, Google, QC, CATT, LGE: Network based solution is enough Network based solution is used for LTM candidate cell list.

Proposal 1: RAN2 to follow RAN3’s agreement to use NW-based solution for LTM candidate cell list, i.e. add a list indicating which LTM candidates the UE had in the SHR.

MRO for CHO with candidate SCGs

3> if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for the source PSCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; 4> set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, if available, of the source PSCell; 4> set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync; 4> set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync;

In current draft RRC CR of Rel-19 SON/MDT, the above part only captured sourcePScellId while missed target pScell ID in SHR for CHO with candidate SCGs case, which is inconsistent with RAN2#129bis agreements “UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs”. During the post email discussion, the email rapporteur explained that a flag included in chowithCandidateSCGInfoList can indicate the pair of PCell and PSCel which was executed, this can implicitly indicate the target PScell ID. Besides, an explicit target PSCell ID + measurements may introduce additional exception handling in the procedure of neighbour cell measurement results. The followings show the options email rapporteur proposed:

Option 1: Add the target cell ID and measurements together with sourcePSCellId and sourcePSCellMea, and exclude target PScell measurements quantities from the procedure of neighbour cell measurement results. (an example is listed as below)

5> if the SS/PBCH block-based (CSI-RS) measurement quantities are available: 6> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, and target PScell if the UE was configured with condExecutionCond and condExecutionCondPScell ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available,……

Option 2: Add a flag in the chowithCandidateSCGInfoList indicating the pair of PCell and PSCell which was executed.

For option 1, actually it mad current RRC specs more complex due to the introduction of exception handling in the procedure of neighbour cell measurement results. For option 2, it’s not clear how to add a flag in chowithCandidateSCGInfoList, maybe it’s correct to introduce a flag in each ChoWithCandidateSCGInfo IE to indicate the executed the pair of PCell and PSCell. However, option 2 is not aligned with RAN2#129bis agreement well because the flag is implicit.

To align with RAN2 agreements directly and minimize the spec impacts, we propose to just introduce an explicit target PScell ID only, without introducing the corresponding cell measurements and handling in the procedure of neighbour cell measurement results. When the network receives the target PScell ID in SHR, it can retrieve the associated target PScell measurement results from neighbour cell measurement results based on its implementation. So, it’s proposed as below:

Proposal 2: To align with RAN2#129bis agreement, RAN2 to explicitly capture target PSCell ID (without measurements) in SHR for successful CHO with candidate SCGs in RRC CR. Network can associate this ID with measurements retrieved from neighbour cell measurement results based on implementation.

In the post email discussion [1], the RRC-1 open issue “the correlation of SHR and SPR and avoiding duplication of the information” is left to be addressed. In our views, it is difficult to achieve redundant reporting in the current specs. For example, it may be necessary to check the SHR one by one, which contents in the SPR report are the same and which are different, and then only the duplicate parts are reported, which will add great complexity for UE. Now there are many duplicate reports in RLF, RA, SHR, and SPR, but this is a historical problem, so there is no need to optimize for this.

Some companies also proposed to add a flag to correlate the SHR and SPR, so that the network can identify which pair of SHR and SPR is from a certain UE. In our view, it’s not needed to introduce such flag to add signalling overhead due to network always do optimization based on cell-based measurements and information in SON reports, network will bind a pair of SHR and SPR together if they contain the same pair of cell ID (Pcell/PScell). Thus, it’s not needed to do the correlation of the SHR and SPR in case of both reports being generated in a CHO with candidate SCG execution scenario.

Proposal 3: (RRC-1) It’s not needed to adopt a correlation solution between SHR and SPR in case of both reports being generated in a CHO with candidate SCG execution scenario.

MRO for SDT

In current CR listed as below, there is an issue related to the maximum value of time since SDT execution. Companies doubt about the case beyond this value and the understanding from UE and network side due to network cannot recognize the right SDT configuration in the case beyond 172800 seconds. As refer to the timeSCGFailure, timeSinceFailure, and elapsedTimeSCG-Failure IE, the maximum value (e.g., xx seconds) in those IEs implies that values exceeding this threshold are treated as "xx seconds or longer.", which can easily fix the issue that the timeSinceSdtExecution value is beyond its value range. So, it’s better to not treat the case that beyond the maximum value of 172800 due to NW can ignore or retrieve SDT configuration based on implementation.

RA-Report field descriptions

Omitted

timeSinceSdtExecution This field logs the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds. FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW can not recognize the right SDT configuration.

Proposal 4: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE only indicate 172800 seconds or longer. The FFS “FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW cannot recognize the right SDT configuration” can be removed.

Conclusion

In this contribution, we propose:

MRO for LTM

Proposal 1: RAN2 to follow RAN3’s agreement to use NW-based solution for LTM candidate cell list, i.e. add a list indicating which LTM candidates the UE had in the SHR.

MRO for CHO with candidate SCGs

Proposal 2: To align with RAN2#129bis agreement, RAN2 to explicitly capture target PSCell ID (without measurements) in SHR for successful CHO with candidate SCGs in RRC CR. Network can associate this ID with measurements retrieved from neighbour cell measurement results based on implementation.

Proposal 3: (RRC-1) It’s not needed to adopt a correlation solution between SHR and SPR in case of both reports being generated in a CHO with candidate SCG execution scenario.

MRO for SDT

Proposal 4: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE only indicate 172800 seconds or longer. The FFS “FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW cannot recognize the right SDT configuration” can be removed.

Reference

[1] [Post129bis][603][SONMDT] Running NR RRC CR for SONMDT (Ericsson)

[2] RAN3_127_agenda_20250221_1300_EOM