3GPP TSG-RAN WG2 Meeting #130	                                                                    R2-2503420

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on the Random Access for A-IoT

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

This paper further discusses the random access related issues for ambient IoT [1]. In details, the following aspects are covered by our contribution,

- Remaining issues on CBRA

- Remaining issues on CFRA

- On the NACK feedback indication

Section 2 gives the detailed discussion, and followed by the Section 3 that summarizes the conclusion of this paper.

- Discussion

- Remaining issues on CBRA

Msg1 resource related issues

On the selection of Msg1 resource

In the current 38.391 A-IoT MAC running CR, it captures the random selection of the access occasion for CBRA as follows,

If Contention-Based Random Access procedure is initiated according to clause 5.2, the A-IoT MAC entity shall: 1> randomly select an access occasion for transmission of the Random ID message among the access occasions configured in A-IoT Paging message; 1> apply the D2R Scheduling Info received from the A-IoT Paging message for the selected access occasion and indicate it to the physical layer; 1> initiate the transmission of Random ID message, as specified in clause 5.3.1.2.

Then there is an open issue on whether to further specify such random selection for example by the counting down mechanism.

| Issue 2-1: Msg1 resource selection | Whether/how to specify the device detailed behaviour of randomly selecting the Msg1 resource based on the R2D trigger message. - Relevant agreements: - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. - Status in running CR: captured as Editor’s Note in 5.3.3.1. | To be discussed by company contributions |
| --- | --- | --- |

From our perspective, the current description is fine on the access occasion selection along the time domain unless there is additional requirement on FDMA case by RAN1. It can be left to device implementation on the details of random selection along the time domain, i.e., no need to specify the counting down mechanism by the device.

Proposal 1: (2-1) RAN2 to not specify the details of the random selection of the access occasion for CBRA unless there is additional requirement on FDMA by RAN1.

Whether to merge the first R2D trigger message into A-IoT paging

In last meeting, we agreed that,

A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

This R2D trigger message is used to indicate the start of the resource for RN16 transmission. Then whether to merge this R2D trigger message into A-IoT paging is FFS,

| Issue 2-2: Paging&first R2D trigger message | Whether the R2D trigger message is needed in CFRA, and whether the first R2D trigger message will be merged into paging message in CBRA. - The Rapp understands the discussion of the R2D trigger message focused on CBRA, and it’s not crystal clear whether the R2D trigger message is also needed in CFRA, and whether it can be merged to paging message if it’s the first R2D trigger message in CBRA. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

According to the RAN1 progress,

For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact FFS: how to define the end of the R2D transmission in 9.4.1

RAN1 only requires one R2D message to serve as baseline to determine the start of access occasion by the Toffset1, so from this perspective, we see no block to use the A-IoT paging message plus the Toffset1 to indicate the first access occasion. Therefore, we propose that,

Proposal 2: (2-2) For CBRA, the A-IoT paging message is used to determine the start of the first access occasion configured by this A-IoT paging with consideration of the Toffset1 defined by RAN1.

Whether to need byte-align for the R2D trigger message

Another open issue is whether to need byte-align for the R2D trigger message,

| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

According to previous agreements, the R2D trigger message needs to include the message type field, which may be only a very small number of bits than an octet, e.g., 3 bits to cover all the type of R2D message, i.e., A-IoT Msg2, A-IoT paging, R2D trigger message, R2D Upper Layer Data Transfer message, and the NACK message for re-access. Until now no other information is needed in the R2D trigger message, so no need to make byte-align for it.

Proposal 3: (2-3) No need to make byte-align for the R2D trigger message.

How to define contention resolution failure

For CBRA, it needs to further clarify how to define the contention resolution failure,

| Issue 2-4: CBRA failure detection | How to determine CBRA failure/contention resolution failure. - In SI, RAN2 agreed to support re-access in case of contention resolution failure as capture in TR 38.796. In WI phase, RAN2 has agreed to re-use the subsequent paging message to trigger re-access, but has not discussed how to determine CBRA failure/ contention resolution failure. - Status in running CR: captured as Editor’s Note in 5.5. | To be discussed by company contributions |
| --- | --- | --- |

Until now we agreed that the subsequent A-IoT paging is used to trigger the re-access for the devices that suffer contention resolution failure in the previous access attempt,

Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

So there are two cases that the device should consider contention resolution failure.

Case 1 – the device receives the A-IoT Msg2 but not including the echoed RN16 before receiving the subsequent A-IoT paging.

Case 2 – the device misses the A-IoT Msg2 due to the R2D reception failure before receiving the subsequent A-IoT paging.

For both two cases the device should perform re-access in the following subsequent A-IoT paging.

Proposal 4: (2-4) RAN2 to consider the following two cases as the contention resolution failure,

Case 1 – the device receives the A-IoT Msg2 but not including the echoed RN16 before receiving the subsequent A-IoT paging;

Case 2 – the device misses the A-IoT Msg2 due to the R2D reception failure before receiving the subsequent A-IoT paging.

Msg2 content

Resource index

Regarding the content of A-IoT Msg2, there is a controversy on whether to need resource index along with the echoed RN16 to avoid the potential RN16 collision by different devices,

| Issue 2-5: random ID differentiation in Msg2 | Whether/how to address random ID collision in Msg2, i.e. multiples devices generate same random ID using different Msg1 resources. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

The cases can be summarized as follows (take two devices for example),

Case 1: the device 1 and device 2 send the same RN16 to the reader in different access occasions, and the reader successfully decodes the two same RN16. If the reader includes one or two same RN16 in the A-IoT Msg2 associated with the corresponding AS ID, it will confuse the devices 1 and 2 whether or which one echoed RN16 with the associated AS ID is used for itself.

Case 2: the device 1 and device 2 send the same RN16 to the reader in different access occasions, but the reader only decodes one of them (for example, device 1) due to the fact another one is conflicted with the other devices. In this case, the reader does not know there is another device 2 sharing the same RN16 with the echoed RN16 in the A-IoT Msg2, which also confuses the device 2 to finish contention resolution.

To solve above issue, we think this can be left to reader implementation with the following reasons,

1. The reason why we use RN16 is that we think this is a large enough number to avoid generating the same RN16 for different devices, as much as possible.

2. Even if above cases occur, for case 1 we can leave it to reader implementation, i.e., the reader does not echo the RN16 in A-IoT Msg2 if it decodes two same RN16 in different access occasions; for case 2, the reader will echo the RN16 in A-IoT Msg2 because it is not aware of the device 2 sharing the same RN16 in Msg1 transmission, which will cause the device 1 and device 2 share the same AS ID and A-IoT Msg3 D2R resource. However, the reader can use the NACK message to trigger the re-access for device 1 and device 2 due to decoding failure caused by this resource conflict.

In summary, there is no critical problem for both cases based on the reader implementation or the NACK mechanism to trigger re-access.

Proposal 5 (2-5): No need to include the A-IoT Msg1 resource index related parameter in the A-IoT Msg2.

On indicating the number of random ID in Msg2

Companies’ views are still split on the indicating the number of random ID in Msg2,

| Issue 2-6: number indication of echoed random IDs in Msg2 | Whether/how to indicate the number of echoed random IDs included in Msg2. - RAN2 agreed that A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices, but there is no discussion on whether/how to indicate the number of echoed random IDs. The Rapp understands this can be considered as signaling design/stage3 issue which should be quite straightforward. Companies can check the proposal in 2.2. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

From our view, not only RN16 is multiplexed in Msg2, but also the AS ID and scheduling resources for Msg3 can be multiplexed in Msg2. The AS ID and the scheduling resource should be associated with the corresponding RN16 so that the device can know the allocated AS ID and Msg3 scheduling resource for itself.

Observation 1: The AS ID and the Msg3 scheduling resource should be associated with the corresponding RN16 to the echoed device.

Based on above requirement, the most straightforward way is to firstly indicate the number of RN16 multiplexed in the Msg2 by an explicit field. Then the bitmap to indicate whether a new AS ID is allocated for the associated RN16 is used, as suggested by the rapporteur summary,

[8/9] Proposal for Issue 2-7: One bit indication is needed for each echoed random ID in Msg2 to indicate whether AS ID is present (i.e., assigned by reader) for this random ID.

By this way, the length of bitmap can be determined based on the number of RN16. It is mandatory that each echoed RN16 is associated with an Msg3 scheduling resource, therefore no need further indication for this field.

Proposal 6: (2-6) The number of multiplexed RN16 in A-IoT Msg2 is explicitly indicated to the device. A bitmap is introduced with the same length as the number of multiplexed RN16 to indicate the allocated AS ID associated with the corresponding RN16.

- Remaining issues on CFRA

Whether to need R2D trigger message for CFRA

In the open issue of 2-2, it also mentions the need of R2D trigger message for CFRA,

| Issue 2-2: Paging&first R2D trigger message | Whether the R2D trigger message is needed in CFRA, and whether the first R2D trigger message will be merged into paging message in CBRA. - The Rapp understands the discussion of the R2D trigger message focused on CBRA, and it’s not crystal clear whether the R2D trigger message is also needed in CFRA, and whether it can be merged to paging message if it’s the first R2D trigger message in CBRA. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

In the procedure of CFRA, the A-IoT Msg1 sends the device ID to the reader. To our understanding, this falls into the category of data transmission, so there is no need of such R2D trigger message to indicate the start of the access occasion, which is used for the A-IoT Msg1 transmission.

Regarding how to indicate the start of the resource for A-IoT Msg1 transmission in CFRA, we can take some reference from A-IoT Msg3 transmission in CBRA.

According to RAN1 agreement,

Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

That implies the device will use the A-IoT Msg2 to serve as the time baseline to determine the start of the X=1 time domain resource for A-IoT Msg3 transmission.

When considering CFRA, we see no block to use the A-IoT paging along with the resource configuration of the D2R Scheduling Info in the content of the A-IoT paging to determine the start of the resource for A-IoT Msg1 transmission. We think this is similar as the A-IoT Msg3 case. Therefore, it is proposed that,

Proposal 7: (2-2) For CFRA, the A-IoT paging is used to determine the start of the resource configured in the A-IoT paging for A-IoT Msg1 transmission.

How to achieve no re-access/no NACK for CFRA

In last meeting, we agreed that for CFRA, there is no re-access and no NACK feedback,

For CFRA, NACK feedback and re-access is not supported. FFS how to achieve

The potential spec impact is still FFS,

| Issue 2-8: no re-access for CFRA | How to achieve “no re-access” for CFRA - Relevant agreements: - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve. - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |
| --- | --- | --- |

The reader knows whether it is triggering CBRA or CFRA as it will indicate the RACH type in the A-IoT paging,

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

So it can be left to reader implementation to not send the NACK feedback if it is triggering CFRA for a single device.

Proposal 8a: (2-8) It is up to reader implementation to not send the NACK feedback if CFRA is triggered by the reader.

Since the data transmission is directly performed in CFRA without contention resolution, at each time the reader does not successfully receives the D2R data, it can re-send the R2D message.

| Data transmission failure in CFRA | Reader behavior |
| --- | --- |
| A-IoT Msg1 failure (i.e., device ID) | Re-send the A-IoT paging containing the D2R Scheduling Info field without transaction ID, i.e., no subsequent paging concept is used for CFRA. |
| D2R data failure (i.e., A-IoT Msg3/5…) | Re-send the R2D Upper Layer Data Transfer message containing the D2R Scheduling Info field (potentially containing the offset indication for segmentation situation). |

Therefore, no re-access is triggered and no further spec impact is found.

Proposal 8b: (2-8) In CFRA, it is up to reader implementation to re-send the R2D message (A-IoT paging or R2D Upper Layer Data Transfer) to trigger the data re-transmission. No further spec impact is found.

- On the NACK feedback indication

Re-access by R2D trigger message or subsequent A-IoT paging

On the re-access triggered by the NACK message, there is an open issue whether the device should perform re-access in the subsequent A-IoT paging or in the R2D trigger message,

| Issue 2-10: NACK before paging or R2D trigger message | For the re-access due to reception of NACK indication before subsequent R2D message, whether the subsequent R2D message is the R2D trigger message or paging message. - Relevant agreements: - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging - Status in running CR: captured as Editor’s Note in 5.5. | To be discussed by company contributions |
| --- | --- | --- |

We had agreed to use the subsequent A-IoT paging as the trigger for re-access,

Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

We prefer to stick to this agreement, i.e., no re-access is performed between two A-IoT paging messages. For the contention resolution failure cases as mentioned in our Proposal 4, we prefer to have a unified design. So it is proposed that,

Proposal 9: (2-10) RAN2 to confirm that only the subsequent A-IoT paging can trigger the access occasion for re-access purpose, regardless of re-access due to NACK feedback or contention resolution failure.

Whether to support multiplexing of multiple devices in NACK feedback

Another open issue for the NACK message is whether to support multiplexing of multiple devices in NACK feedback,

| Issue 2-12: multiplexing for NACK indication | Whether to support multiplexing of information for multiple devices in NACK feedback. - Relevant agreements: - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

Since the AS ID is allocated in A-IoT Msg2 in CBRA and along with the upper layer command in CFRA, it is straightforward to associate the NACK feedback with the AS ID for multiple devices.

Proposal 10: (2-12) Support the multiplexing of multiple devices in the NACK message.

- Conclusion

In this paper, we discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: The AS ID and the Msg3 scheduling resource should be associated with the corresponding RN16 to the echoed device.

Remaining issues on CBRA

Proposal 1: (2-1) RAN2 to not specify the details of the random selection of the access occasion for CBRA unless there is additional requirement on FDMA by RAN1.

Proposal 2: (2-2) For CBRA, the A-IoT paging message is used to determine the start of the first access occasion configured by this A-IoT paging with consideration of the Toffset1 defined by RAN1.

Proposal 3: (2-3) No need to make byte-align for the R2D trigger message.

Proposal 4: (2-4) RAN2 to consider the following two cases as the contention resolution failure,

Case 1 – the device receives the A-IoT Msg2 but not including the echoed RN16 before receiving the subsequent A-IoT paging;

Case 2 – the device misses the A-IoT Msg2 due to the R2D reception failure before receiving the subsequent A-IoT paging.

Proposal 5 (2-5): No need to include the A-IoT Msg1 resource index related parameter in the A-IoT Msg2.

Proposal 6: (2-6) The number of multiplexed RN16 in A-IoT Msg2 is explicitly indicated to the device. A bitmap is introduced with the same length as the number of multiplexed RN16 to indicate the allocated AS ID associated with the corresponding RN16.

Remaining issues on CFRA

Proposal 7: (2-2) For CFRA, the A-IoT paging is used to determine the start of the resource configured in the A-IoT paging for A-IoT Msg1 transmission.

Proposal 8a: (2-8) It is up to reader implementation to not send the NACK feedback if CFRA is triggered by the reader.

Proposal 8b: (2-8) In CFRA, it is up to reader implementation to re-send the R2D message (A-IoT paging or R2D Upper Layer Data Transfer) to trigger the data re-transmission. No further spec impact is found.

On the NACK feedback indication

Proposal 9: (2-10) RAN2 to confirm that only the subsequent A-IoT paging can trigger the access occasion for re-access purpose, regardless of re-access due to NACK feedback or contention resolution failure.

Proposal 10: (2-12) Support the multiplexing of multiple devices in the NACK message.

- References

[1] Draft_RAN2_129bis_Meeting_Report_v2