3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503421

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on the A-IoT Data Transmission

Agenda Item:	8.2.4

Document for:	Discussion

1.	Introduction

In this paper, we give our views on the left over issues related to the A-IoT data transmission according to the WID in [1] and the progress at RAN2#129bis. The following aspects are covered in our discussion,

- Further consideration on the segmentation for D2R

- AS ID release

- MAC PDU format

- No available upper layer data for D2R scheduling

The Section 2 gives the detailed discussion, and followed by Section 3 that summarizes this paper.

2.	Discussion

2.1 Further consideration on the segmentation for D2R

For the re-transmission of the first segment or non-segmented D2R data, the reader will re-send the upper layer command again to avoid the potential missing of the previous R2D transmission by the device,

For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included.

Then there is one FFS whether to include the upper layer command in the R2D message on retransmission of non-first segments at RAN2#129bis as below:

| Issue 3-1: command for non-first segment | Whether upper layer command is included in the R2D message scheduling for non-first segment. - Relevant agreements: - FFS whether the reader always includes the command for retransmission of segments. - Status in running CR: captured as Editor’s Note in 5.4.2. | To be discussed by company contributions |
| --- | --- | --- |

Here is the analysis of pros and cons on the inclusion of the upper layer command in R2D for retransmission of non-first segment.

| Not Include Command |  | Include Command |  |
| --- | --- | --- | --- |
| Pros | Cons | Pros | Cons |
| Reduces air interface overhead; Reduce the processing latency by the device. | The command should be stored by the device for the potential retransmission indicated by the offset indication (e.g., the read command started by position A and ended by position B, the device needs to determine the bitstring to be transmitted by the offset indication and the stored read command [position A, position B]). | The device does not need to store any command information, i.e., the device just follows the R2D message at each time it needs to make response. | Introduce the processing latency by the device. |

From our perspective, both options are workable since it can be left to device implementation whether to store the command information.

Observation 1: The handling of non-initial segment retransmissions is implementation-dependent, with devices autonomously deciding whether to store or respond to received commands.

However, we prefer the way to reduce the device complexity, i.e., at each time the reader wants the device to make response, it includes the corresponding upper layer command in the R2D message. Therefore, it is proposed that,

Proposal 1: (3-1) The upper layer command should be included in the R2D message for retransmission scheduling of non-first segments by the reader.

There is another FFS on retransmission of the first segment and unsegmented message at RAN2#129bis:

| Issue 3-2: offset for first segment | Whether offset is included in the R2D message scheduling for the first segment and unsegmented message - Relevant agreements: - For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. - Status in running CR: captured as Editor’s Note in 5.4.3. | To be discussed by company contributions |
| --- | --- | --- |

Since the upper layer command will be re-sent to the device for the re-transmission scheduling of the first segment or non-segmented D2R data, the offset indication (with value = 0) is not mandatory for this situation. However, we have a slight preference to have a unified design on this functionality. So it is proposed that,

Proposal 2: (3-2) Offset zero is included in the R2D message along with the upper layer command for the retransmission of the first segment or unsegmented D2R message.

2.2 AS ID release

There is an FFS issue on AS ID release at RAN2#129bis:

| Issue 3-3: AS ID release | Whether to specify any additional AS ID release method - Relevant agreements: - FFS other cases for release ASID to avoid keeping it indefinitely. - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |
| --- | --- | --- |

The existing agreements on the AS ID release from the device perspective are as follows,

The device releases the AS ID upon power off (no stage 3 specification impact); The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely.

The intention to release the AS ID in above scenarios is summarized in the following table,

| Upon power off | Straightforward to release the AS ID as it is a temporary ID which should be stored in the volatile memory. |
| --- | --- |
| Upon receiving paging with new transaction ID | One of the intention is the reader wants to stop the previous service and start a new service immediately. In this situation, the device should release the AS ID and update to a new one. However, if the reader does not execute the new service immediately, for example, the reader does not receive the new service request from the CN, it will cause the device to store the AS ID in vain for a time-period. |
| Upon triggering a new msg1 transmission as a result of receiving paging | Straightforward as the device will be allocated to a new AS ID upon performing re-access for CBRA |

As per above analysis, one situation is not covered by the current agreements for the device to release the AS ID in time, i.e., upon terminating the current service while the new service is not arrival.

Observation 2: From the device perspective, it should release the AS ID as soon as possible controlled by the reader since the reader cannot predict when the new service request will come.

Therefore, it is suggested that,

Proposal 3: (3-3) The service end indication (i.e., 1 bit) is included in the last subsequent paging associated with the same transaction ID. The device releases the AS ID upon receiving the service end indication.

2.3 D2R MAC PDU format

There is an FFS issue on D2R padding indication at RAN2#129bis:

| Subgroup: D2R message content for data transmission |  |  |
| --- | --- | --- |
| Issue 3-4: D2R padding indication | How to indicate padding and the Length filed for SDU or padding and its size - Relevant agreements: - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS. - Status in running CR: captured as Editor’s Note in 6.2.2.2. | To be discussed by company contributions |

Firstly, since only one bit indication is used to indicate whether there is more data to transmit, the reader cannot predict the exact remaining data of the device (in case there is no assistance information from CN on the expected D2R message size as this is optionally provided by CN to the reader). From this perspective, there is no big difference to indicate the SDU length or padding. We just prefer to have the SDU length when the D2R MAC PDU includes both MAC SDU and padding.

Secondly, regarding the size of the SDU length field, we think 10-bit is enough as according to SA1 use cases, the maximum 1000-bit is expected to be transmitted from the device to the reader.

Proposal 4: (3-4) A fixed 10-bit SDU length field is included when the MAC PDU includes both MAC SDU and padding bits.

There is another FFS issue on D2R message type at RAN2#129bis:

| Issue 3-5: D2R message type | Whether to support D2R message type - Relevant agreements: - FFS whether we introduce D2R message type. Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits - Status in running CR: captured as Editor’s Note in 6.1.1. | To be discussed by company contributions |
| --- | --- | --- |

There are only two types of D2R PDU in inventory and inventory + command cases:

| R2D Message Type | Contents |
| --- | --- |
| MSG1 | RN16 |
| Data | EPC ID (MSG3), Data following MSG3 |

The message type of MSG1 is not required since the reader is aware of receiving MSG1 according to the detection of the random access occasions. That means MSG1 can be designed as 16-bit fixed size without additional information. For the data transmission, it is always scheduled by the reader. So there is no need to indicate the D2R message type for the D2R transmission.

Proposal 5: (3-5) No need to introduce the field of D2R message type in Rel-19.

2.4 No available upper layer data for D2R scheduling

There is an FFS issue on no available upper layer data for a D2R scheduling at RAN2#129bis post email discussion:

| Issue 3-6: Write operation response | Whether the write command response means ‘successfully completing the write operation’, and whether this may cause a case of ‘no upper layer data is available for a D2R scheduling’ due to long writing time. - During the running CR review, companies identified that in SA2 LS S2-2501241, the service data response for write command is “operation result indication response for 'Write'”. But, it’s not clear whether the operation result means successfully completing the write command or successfully receiving the command message or both. And if the command response means successfully completing the writing operation, there may be an uncertain writing time in different device implementation. Then we need to accommodate this in the data transmission procedure. | To be discussed by company contributions |
| --- | --- | --- |

Based on the companies’ views, one issue that needs to be clarified is for the write command, whether the device sends the “ACK” indication upon receiving or finishing the write command.

According to the SA2 response in the LS S2-2501241, we think it is clear that the device should firstly finish the write operation and then send the “ACK” response to the reader,

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)? SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

Therefore, it is proposed that,

Proposal 6: (3-6) RAN2 assumes the device sends the “the operation result indication response” in NAS message to the reader upon finishing the write operation.

From the device perspective, we think the device can finish the write operation once it does receive the command. So there is no such issue that there may be an uncertain writing time by different device implementation.  However, this may be under the RAN1/RAN4 scope for further confirmation.

Proposal 7: (3-6) Send LS to RAN1/RAN4 to confirm whether there is an issue that D2R data is not available for the PDRCH transmission due to a long write time for the write operation by the device.

Even if there is such issue, we think it is similar as the re-transmission of the first unsegmented data (Only 1-bit ACK indication, so no need segmentation). That is to say, if the device does not finish the write operation upon the arrival of the D2R resource scheduled by the R2D Upper Layer Data Transfer message, it should ignore the scheduling info and not send any upper layer data using this D2R resource. From the reader perspective, it will consider the failure of data reception no matter whether it is caused by R2D command transmission failure or D2R data transmission failure. The expected reader behaviour is to send another R2D Upper Layer Data Transfer message containing the D2R Scheduling Info along with the write command to let the device send the “ACK” indication.

Proposal 8a: (3-6) The device ignores the D2R scheduling if “the operation result indication response” in NAS message is not generated by the device.

Proposal 8b: (3-6) It’s up to reader to re-transmit the R2D Upper Layer Data Transfer message containing a new D2R Scheduling Info along with the write command to device.

Finally, it may be concerned that the device will mis-understand that the write command contained in the R2D Upper Layer Data Transfer message is a new one, which triggers the device to perform the unexpected write operation once again. We think the device can check the memory that the content of the write command had been finished in previous operation so that it can skip the write operation and generate the “operation result indication response” immediately and deliver it to the A-IoT MAC layer for transmission.

Proposal 8c: (3-6) Device directly responds the write command re-transmitted by the reader with the “operation result indication” if the device had finished the same write operation by previous write command.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: The handling of non-initial segment retransmissions is implementation-dependent, with devices autonomously deciding whether to store or respond to received commands.

Observation 2: From the device perspective, it should release the AS ID as soon as possible controlled by the reader since the reader cannot predict when the new service request will come.

Further consideration on the segmentation for D2R

Proposal 1: (3-1) The upper layer command should be included in the R2D message for retransmission scheduling of non-first segments by the reader.

Proposal 2: (3-2) Offset zero is included in the R2D message along with the upper layer command for the retransmission of the first segment or unsegmented D2R message.

AS ID release

Proposal 3: (3-3) The service end indication (i.e., 1 bit) is included in the last subsequent paging associated with the same transaction ID. The device releases the AS ID upon receiving the service end indication.

D2R MAC PDU format

Proposal 4: (3-4) A fixed 10-bit SDU length field is included when the MAC PDU includes both MAC SDU and padding bits.

Proposal 5: (3-5) No need to introduce the field of D2R message type in Rel-19.

No available upper layer data for D2R scheduling

Proposal 6: (3-6) RAN2 assumes the device sends the “the operation result indication response” in NAS message to the reader upon finishing the write operation.

Proposal 7: (3-6) Send LS to RAN1/RAN4 to confirm whether there is an issue that D2R data is not available for the PDRCH transmission due to a long write time for the write operation by the device.

Proposal 8a: (3-6) The device ignores the D2R scheduling if “the operation result indication response” in NAS message is not generated by the device.

Proposal 8b: (3-6) It’s up to reader to re-transmit the R2D Upper Layer Data Transfer message containing a new D2R Scheduling Info along with the write command to device.

Proposal 8c: (3-6) Device directly responds the write command re-transmitted by the reader with the “operation result indication” if the device had finished the same write operation by previous write command.

4. References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR