3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503423

St.Julians, Malta, 19th – 23rd May, 2025

Source:              CATT

Title:	             Random Access in SBFD symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In the past RAN2 meetings, random access in SBFD were discussed and reached many agreements. In this contribution, we will further discuss the following leftover issues:

- Whether random access triggered by LTM can use SBFD symbols?

- For CFRA triggered by other cases except for BFR and RRCReconfigurationWithSync, how to indicate the RO type？

- How to determine the RO type in case of CFRA fallbacks to CBRA?

- Whether the working assumption of RO type selection order can be confirmed?

- Whether the RO type fallback should be within the selected Random Access resources set?

- Whether Msg1 repetition number fallback can be supported within SBFD RO?

- What is the order of RO type fallback and Msg1 repetition number fallback if both are supported?

- Whether early identification in Msg3 needs to be supported when legacy RO is selected?

Discussion

Whether random access triggered by LTM can use SBFD symbols?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Furthermore, in RAN2#129bis meeting, the above working assumption was discussed and reached the following agreement:

Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

Based on the above agreements, it is still FFS whether RA triggered by LTM can be performed in SBFD symbols. According to [2], there are two RA triggers associated with LTM:

- Trigger A: Early UL synchronization with an LTM candidate cell.

- Trigger B: RACH-based LTM cell switch.

The mobility latency of the above two LTM mechanisms are shown as below [2]:

Figure-1 Mobility Latency for RACH-less LTM

Figure-2 Mobility Latency for RACH-based LTM

Based on the above two figures, it is obvious that:

- For Trigger A, if RA is allowed in SBFD symbols, it may reduce the UL synchronization latency.

- For Trigger B, if RA is allowed in SBFD symbols, it may reduce both the UL synchronization latency and the interruption time.

Observation 1: If LTM related RA in SBFD symbols is supported, it can reduce the UL synchronization time. Furthermore, it can also reduce the interruption time for RACH-based LTM cell switch.

In the email discussion of RRC open issues, many companies do not support LTM related RA in SBFD symbols. The main concerns are listed below:

- Only indicate the RO type in EarlyUL-SyncConfig is not enough, the additional RO configuration should also be indicated in the EarlyUL-SyncConfig;

- EarlyUL-SyncConfig is from candidate cell, which may impact RAN3.

We agree with the first bullet listed above, but we think it does impact RAN3 since EarlyUL-SyncConfig is contained in RRCReconfiguration and RRCReconfiguration is used as a container in RAN3.

Observation 2: If LTM related RA in SBFD symbols is supported, it will introduce specification efforts in RAN2, but no additional RAN3 impacts.

Based on the above observation 1 and 2, it is slightly prefers to support LTM related RA in SBFD symbols.  But if majority does not support it, we can also make a compromise.

Proposal 1: (RRC-2) LTM related RA can be supported in SBFD symbols for reducing the UL synchronization time.

RO type indication for CFRA

In RAN2#128 meeting, regarding to initial RO type selection of CFRA, the following agreement was reached:

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

Furthermore, in RAN2#129bis meeting, it was further agreed that:

On RO type signaling for CFRA 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

Besides BFR and RRCReconfigurationWithSync, the following CFRA cases should also be considered:

- Case 1: PDCCH order;

- Case 2: Early UL synchronization with an LTM candidate cell;

- Case 3: RACH-based LTM cell switch.

For Case 1, the dedicated CFRA resources are indicated in PDCCH order. In RAN1#119 meeting, RAN1 already agreed that:

Agreement For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

Based on the above RAN1 agreement, it is obvious that RAN1 already agreed that the RO type can be indicated in the DCI of PDCCH order.

Proposal 2: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is indicated in the DCI of PDCCH order.

For Case 2, as shown in Figure-1, early UL sync is used. The early UL sync configuration is as below: RRCReconfiguration -> LTM-Config -> LTM-Candidate -> ltm-EarlyUL-SyncConfig -> EarlyUL-SyncConfig. Considering the EarlyUL-SyncConfig is derived in the candidate cell, hence, the candidate cell should be responsible for determining which RO type should be used. Hence, it is reasonable to indicate the RO type in EarlyUL-SyncConfig.

For Case 3, EarlyUL-SyncConfig is also needed. If RO type is indicated in it, it is unnecessary to include it in LTM Cell Switch Command MAC CE.

Proposal 3: (RRC-2) If Proposal1 is agreed, for CFRA triggered by early UL synchronization with an LTM candidate cell and RACH-based LTM cell switch, the RO type is indicated in the EarlyUL-SyncConfig.

RO type selection in case of CFRA/CBRA fallback

According to [3], the following two kinds of RA type fallback are supported for CFRA:

- Case 1: CFRA fallbacks to CBRA, and only CBRA is performed afterwards.

  - Case1.1: CFRA is triggered, but there is always no suitable SSB/CSI-RS (i.e. no SSB/CSI-RS satisfy the RSRP requirement) during each RACH attempts or the preamble index indicated in PDCCH order is 0b000000;

  - Case 1.2: CFRA is performed for N(N≥ 1) times. When performs afterwards RA re-attempts, there is always no SSB/CSI-RS satisfy the RSRP requirement, it fallbacks to CBRA.

- Case 2: CFRA fallbacks to CBRA, and CBRA fallbacks to CFRA again…….

For Case 2, after CFRA fallback to CBRA, for RACH re-attempt, once there is suitable SSB/CSI-RS, it can fallback to CFRA again. And during CFRA, if no SSB/CSI-RS satisfy the RSRP threshold, CFRA can fallback to CBRA again…….

According to RAN2 agreement, for CFRA, the RO type can be indicated by NW. In case of CFRA fallback to CBRA, there are two kinds of RO type selection methods:

- Option 1:  Always use the RO type indicated by NW for CFRA;

- Option 2:  When performing CFRA, uses the RO type indicated by NW; when performs CBRA, it follows the RO type selection rule used for CBRA.

For Option 2, if the selected RO type of CBRA is different from the selected RO type of CFRA, “ping-pong” RO type switch is inevitable in case of CFRA/CBRA fallback is performed. This should be avoided.

Furthermore, for Option 2, it should also discuss whether the combination of CFRA/CBRA fallback and CBRA RO type fallback is supported. If it is supported, during the CFRA/CBRA/CFRA…fallback, it should further discuss how to count the number used for RO type fallback. Considering the timeline, it is slightly prefer Option 1.

Proposal 4:  If CFRA is triggered, UE always uses the RO type indicated for CFRA even if CFRA/CBRA fallback is performed.

Order of RO type selection

Regarding to the RO type selection, in RAN2#129bis meeting, the following working assumption was reached:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

As analyzed in [4], if the selection of RO type should be performed before the selection of the set of Random Access resources. As discussed in the last RAN2 meeting, this solution has less spec impacts and complexity compared with other solutions. Hence, suggest RAN2 to confirm the working assumption.

Proposal 5:  Confirm the working assumption that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Whether RO type fallback for CBRA should be within the same set of Random Access resources?

In legacy release, during the RA re-attempts, the set of Random Access resources keeps unchanged even if 2-step RA fallback to 4-step RA.

Observation 3: In legacy RA procedure, only one set of Random Access resources is selected and the selected set of Random Access resources keeps unchanged during RA re-attempts even if 2-step/4-step fallback happens.

With the introduction of SBFD, RAN2 already agreed that the RO type fallback between additional RO and legacy RO is supported. Hence, RAN2 should further discuss whether the RO type fallback should be within the same set of Random Access resources as the initial RA attempt. In our understanding, this rule should not be broken with the introduction of SBFD.

With the introduction of SBFD, there are two kinds of RACH configurations:

- For RACH configuration Option 1, there is both legacy RO and additional RO within one set of Random Access resources, RO type fallback is possible without change the set of Random Access resources.

- For RACH configuration Option 2, additional RACH configuration is used. Similar as 2-step/4-step RA fallback, if both legacy RO and additional RO are configured within the selected set of Random Access resources, fallback is feasible; otherwise, fallback is not feasible.

Proposal 6: (MAC-2) RAN2 confirms that the RA re-attempts should be within the same set of Random Access resources as the initial RA attempt with the introduction of SBFD.

Msg1 repetition number fallback within SBFD RO

According to the email discussion of MAC running CR, one additional issues needs to be discussed is that whether Msg1 repetition number fallback can be supported within SBFD RO or not? In our understanding, this should be supported within SBFD RO to increase the RACH successfully probability.

Proposal 7: (MAC-3) Msg1 repetition number fallback can be supported within SBFD RO.

Order of the RO type fallback and Msg1 repetition number fallback

According to the email discussion of MAC running CR, another issue needs to be addressed is the order of RO type fallback and Msg1 repetition number fallback. In our understanding, the determination of RO type fallback should be performed firstly. And after RO type fallback, it should decide which Msg1 repetition number can be used.

Proposal 8: Once RO type fallback condition is met, UE should first perform RO type fallback and determine the Msg1 repetition number based on the new RO type.

Whether early identification is needed when legacy RO is selected?

In RAN2#127bis meeting, the following agreements were reached:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreements, it is obvious that if additional RO is selected, early identification via Msg1 is possible. If legacy RO is selected, since SBFD is not introduced as a feature combination, early identification via Msg1 is not supported.

Based on the above background, what needs to be discussed next is whether early identification via Msg3 needs to be supported in case of legacy RO is selected.

In last RAN1 meeting, it was agreed that:

For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure

This means Msg3 will not be enhanced. Hence, early identification via Msg3 cannot be supported in case of legacy RO is selected.

Proposal 9: Follow RAN1 agreement, if legacy RO is selected, early identification via Msg3 is not supported.

Conclusion

According to the analysis in section 2, it is proposed:

LTM related RA can use SBFD symbols [Open issue number RRC-2]

Proposal 1: (RRC-2) LTM related RA can be supported in SBFD symbols for reducing the UL synchronization time.

RO type indication for CFRA

Proposal 2: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is indicated in the DCI of PDCCH order.

Proposal 3: (RRC-2) If Proposal1 is agreed, for CFRA triggered by early UL synchronization with an LTM candidate cell and RACH-based LTM cell switch, the RO type is indicated in the EarlyUL-SyncConfig.

RO type selection for CFRA/CBRA fallback

Proposal 4:  If CFRA is triggered, UE always uses the RO type indicated for CFRA even if CFRA/CBRA fallback is performed.

Order of RA type selection

Proposal 5:  Confirm the working assumption that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Random Access resources set during RO type fallback for CBRA [Issue MAC-2]

Proposal 6: (MAC-2) RAN2 confirms that the RA re-attempts should be within the same set of Random Access resources as the initial RA attempt with the introduction of SBFD.

Msg1 repetition number fallback within SBFD RO [Issue MAC-3]

Proposal 7: (MAC-3) Msg1 repetition number fallback can be supported within SBFD RO.

Order of RO type fallback and msg1 repetition number fallback

Proposal 8: Once RO type fallback condition is met, UE should first perform RO type fallback and determine the Msg1 repetition number based on the new RO type.

Early identification in Msg3

Proposal 9: Follow RAN1 agreement, if legacy RO is selected, early identification via Msg3 is not supported.

Reference

- R2-2500588  Remaining issues for RACH in SBFD   Apple

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)

- TS38.321  NR; Medium Access Control (MAC) protocol specification (Release-18)

- R2-2501849  Random Access in SBFD symbols