3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503426

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Consideration on LCP enhancement

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on LCP enhancement including SR priority and retransmission priority in intra-UE prioritization, impact of congestion (i.e. PSI-based SDU discard) on priority adjustment, and priority fallback with consideration of PDU Set integrated handling, our views are also given in this paper.

Discussion

[MAC-01] SR priority in intra-UE prioritization

RAN2#129bis Agreements on LCP enhancements - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough

Based on current progress, the priority of UL grant will apply the additional LCP priority. But it is still FFS whether additional LCP priority is used for SR priority determination.

TS 38.321 5.4.4 Scheduling Request As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR. 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap: 3> if the PUCCH resource for the SR transmission occasion does not overlap with any of a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, an SL-SCH resource, or an SL-PRS resource; or 3> if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or 3> if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU determined as specified in clause 5.22.1.3.1a for the SL-SCH resource; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or in the clause 5.22.1.3.5, or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a or the SL-PRS resource in clause 5.22.1.3.5: 4> consider the SR transmission as a prioritized SR transmission. 4> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup; <Skip> 3> else: 4> consider the SR transmission as a de-prioritized SR transmission.

Regarding legacy procedure in above yellow marked part, the priority of SR is determined by the priority of logical channel triggering SR. If a LCH with additional priority triggers the SR, it is better to apply the additional priority for SR, which could give the SR higher priority to be transmitted for XR data transmission and avoid the obsolete data.

If the triggerred SR is agreed to apply the additional priority of the LCH, one issue is, maybe, there is no LCP procedure for this LCH when SR is triggered, which means that the additional priority of the LCH needs to be specified at SR procedure. Based on above table on SR procedure, the priority value of the logical channel that triggered SR is better to be determined at the time of PUCCH resource of the SR transmission occasion other than the time of SR triggering, which could reflect the real requirements on SR for this logical channel. As shown in the running R19 XR MAC CR at RAN2#129bis post email, the procedure may be added in 5.4.1 UL Grant reception section, and clarify that the additionalPriority of this logical channel shall be applied if the smallest remaining value of the running PDCP discardTimers among its data available for this logical channel evaluated at the time of the first symbol of this transmissoin is below the configured threshold.

Proposal 1: (MAC-01) The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

Proposal 2: (MAC-01) The procedure on the determination of additional priority of LCH that triggered SR could be added in 5.4.1 UL Grant reception section in MAC spec, it is, the additionalPriority of this logical channel shall be applied if the smallest remaining value of the running PDCP discardTimers among its data available for this logical channel evaluated at the time of the first symbol of this transmission is below the configured threshold.

Retransmission priority in intra-UE prioritization

In legacy procedure, the priority of new transmission and retransmission for a MAC PDU is the same.

With the introduction of additional Priority in enhanced LCP procedure, the new transmission may apply the legacy priority, but it is possible that the retransmission for a MAC PDU will become delay critical, it is reasonable to  improve the retransmision priority based on the remaining time. However, for this case, the MAC or PHY needs to track the remaining time and compute priority adjustment at each retransmission for a MAC PDU in HARQ procedure, the complexity will be increased much more compared with the enhanced LCP procedure. So, it is suggested not to consider the retransmission priority adjustment in intra-UE prioritization, the retransmission priority will follow the priority of new transmission in LCP procedure.

Proposal 3: It is suggested not to consider the retransmission priority adjustment in intra-UE prioritization, the retransmission priority will follow the priority of new transmission in LCP procedure.

[MAC-02] Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment

In current discussion, LCP procedure is used based on the running PDCP discardTimers and does not consider the timer discardTimerForLowImportance, which is used for PDCP SDU belonged to a low importance PDU Set. The UE will perform PSI-based SDU discard  based on discardTimerForLowImportance as shown in below procedure.

TS 38.323 5.2.1 Transmit operation At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall: - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured). NOTE 0: Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

Based on above analysis, as the PDCP SDU belongs to the low importance PDU set for PSI-based SDU discard, it is not necessary consider PSI-based SDU discard for LCP procedure, the PDCP SDU belongs to the low importance PDU set will be discarded based on legacy procedure.

Proposal 4: (MAC-02) Not to consider discardTimerForLowImportance for PSI-based SDU discard on LCP priority adjustment.

[MAC-13] Priority fallback with consideration of PDU Set integrated handling

- TS 38.323 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

Based on above legacy procedure, if discard timer expires for a PDCP SDU, all PDCP SDUs belonging to the PDU Set needs to be discarded.

One open issue suggests extending the similar logic to LCP procedure. For example, if PDU Set discarding is configured, some data in the buffer belong to a PDU Set is already considered urgent and apply the additional priority in resource allocation step, while other data to the same PDU set is not urgent based on its remaining time and remained in the buffer after the resource allocation, it is desirable to continue using the additional priority rather than fall back to the default priority. In our view, if the remaining time of other data in a PDU set is not lower than the configured threshold, it means that the other data in this PDU set is not urgent and have sufficient delay tolerance to be scheduled by network in future. Furthermore, if the additional priority is applied for the data in a PDU set without delay critical requirements, it is not fair to other logical channel with delay critical data. So, it is not necessary to consider enhanced LCP on PDU set level.

Proposal 5:  (MAC-13) Do not consider priority adjustment on PDU set level in enhanced LCP procedure.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: (MAC-01) The additional priority of LCH could be considered for the priority of SR in intra-UE prioritization.

Proposal 2: (MAC-01) The procedure on the determination of additional priority of LCH that triggered SR could be added in 5.4.1 UL Grant reception section in MAC spec, it is, the additionalPriority of this logical channel shall be applied if the smallest remaining value of the running PDCP discardTimers among its data available for this logical channel evaluated at the time of the first symbol of this transmission is below the configured threshold.

Proposal 3: It is suggested not to consider the retransmission priority adjustment in intra-UE prioritization, the retransmission priority will follow the priority of new transmission in LCP procedure.

Proposal 4: (MAC-02) Not to consider discardTimerForLowImportance for PSI-based SDU discard on LCP priority adjustment.

Proposal 5:  (MAC-13) Do not consider priority adjustment on PDU set level in enhanced LCP procedure.

Reference

- RAN2#129bis meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei