3GPP TSG-RAN WG2 Meeting #130                                                                   R2-2503427

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Consideration on DSR Enhancement

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss and give our views on the issues on DSR enhancement, including the handling of Control PDU and retransmission data, and UE capability issue.

Discussion

The enhanced DSR agreements in RAN2#129bis [1] are as follows.

[PDCP-1] Handling of Control PDU and retransmission data

In legacy procedure, only one remainingTimeThreshold is used to trigger the DSR reporting. When R18 DSR is triggered, as the Control PDU and retransmission data in PDCP/RLC layer are high priority, so they will be included in delay-critical RLC data volume as follows.

TS 38.323 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. TS 38.322 5.5 Data volume calculation For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM). In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.

RAN2#129bis meeting agreement on DSR is as follows:

RAN2#129bis agreement During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

Now, for Rel-19 DSR, the network could configure multiple reporting thresholds to a LCG, generally, the Control PDU and retransmission data needs to be associated to one reporting threshold. Regarding on above agreements, one option is associated it to the shortest reporting threshold included in a specific DSR, another option is to associate it to the shortest configured reporting threshold. Compared with two options, the first option will avoid the further issue on how to define the remaining time if only Control PDU and retransmission data is associated to a reporting threshold for a LCG. However, we are preferable the second option, since it makes the Control PDU and retransmission data always to be considered as the highest priority data volume and the procedure is also simple, otherwise, it is possible that the PDCP/RLC control PDUs and retransmission data will be associated to a reporting threshold with relaxed remaining time threshold, which is not beneficial for network scheduling.

Proposal 1: (PDCP-1) All PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold.

For the issue based on above second option, if there is only Control PDU and retransmitted data in a LCH, and the data triggered DSR in another LCH is associated to not-shortest configured reporting threshold, how to set the remaining time associated to the shortest configured reporting threshold in R19 DSR needs to be determined. it can be considered as UE implementation to choose a suitable value less than the shortest configured reporting threshold.

Proposal 2: (PDCP-1) For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, it is UE implementation to set the remaining time field for this reporting threshold in the DSR MAC CE.

[UE  capability-01] UE capability

RAN2#129bis agreement: 4. An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

In last RAN2#129bis meeting, RAN2 already achieved some agreements on UE capability. One FFS issue is whether UE supporting this feature shall also indicate support of delayStatusReport-r18. Based on the procedure of R18 DSR and R19 enhanced DSR, it seems that the UE supporting R19 enhanced DSR will naturally support R18 DSR. However, in fact, the UE supporting R19 DSR will compute and report the data volume based on the configured reporting threshold, while the UE supporting R18 DSR needs to compute and report the data volume based on the triggering threshold, the UE implementation for the threshold is different. If these two features are coupled together for UE supporting R19 DSR, the UE needs to support two procedures considering the reporting threshold and  triggering threshold for data volume computing simultaneously, so, it is not reasonable to think a UE supporting R19 DSR could also support  R18 DSR. It is suggested not to couple the R19 DSR with R18 DSR together.

Proposal 3: (UE capability-01) UE supporting R19 enhanced DSR will separately report the capability for R18 DSR.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: (PDCP-1) All PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold.

Proposal 2: (PDCP-1) For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, it is UE implementation to set the remaining time field for this reporting threshold in the DSR MAC CE.

Proposal 3: (UE capability-01) UE supporting R19 enhanced DSR will separately report the capability for R18 DSR.

Reference

- RAN2#129bis meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei