3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503431

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on Multi-hop Discovery and (Re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, the issue on Rel-19 multi-hop SL U2N relay discovery and (re)selection had been discussed[1]. In the current contribution, we further discuss the key leftover issues:

- FFS on PC5 AS conditions for discovery model B.

- FFS on notification message when the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu.

- UE behavior for the idle/inactive remote UE after intermediate relay UE triggers relay reselection as a remote UE.

- FFS on Uu upper bound for intermediate relay UE.

Discussion

[RRC-6] FFS on PC5 AS conditions for discovery model B

In RAN2#129bis meeting, the below agreements were reached for discovery model B:

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

There is one FFS in the intermediate relay UE always establishes itself as a remote UE first. For this ffs, we think the common procedure that majority companies agreed is the access to the network is not needed for discovery, only for RRC connection. Take one step back, in case that the intermediate relay UE establishes itself as a remote UE first, that is to say, the PC5-S between the intermediate relay UE and its parent UE has been establishment, this case had been already been captured into the RAN2#129 meeting’s agreement (For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.). All in all, this FFS can be deleted without further handling from our point of view.

Proposal 1: (RRC-6) The FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first can be deleted without further handling.

[RRC-5] FFS on notification message when the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu.

In RAN2#129bis meeting, the below agreement was reached:

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

The FFS part is if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). Similar issue had been discussed from Rel-17 U2N relay, we fail to see the need to rediscuss this issue again in Rel-19. Considering we are at the end of current work item, we prefer to use directly forwarded copy of the original notification message in the current release.

Proposal 2: (RRC-5) When the intermediate relay UE relay UE received a notification message from the last relay UE indicating a failure on UU, the intermediate relay UE may transmit a notification message downstream towards the remote UE without change the notification message content in Rel-19.

UE behavior for the idle/inactive remote UE after intermediate relay UE triggers relay reselection as a remote UE

In RAN2#129bis meeting, the below agreements were reached:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

On the notification message details, we think the legacy PC5 notification message can be directly reused. For the detailed cause value, there is no need to further distinguish which PC5 reselection happened, hence a general cause value that the parent PC5 link reselection happened is enough.

Proposal 3: When the parent UE in idle/inactive performs reselection, RAN2 uses the legacy PC5 notification message to indicate of the reselection for its child UE.

Proposal 4: Introduce a new cause value showing the parent PC5 link reselection happened in Rel-19.

On other condition/timing that the intermediate relay UE notifies its child node. As what we discussed previous, we prefer there is no condition or timing for the intermediate relay UE notifies its child node in Rel-19.

Proposal 5: RAN2 agrees that no condition or timing is introduced for the intermediate relay UE notifies its child node in Rel-19.

On the child node behavior after receiving this notification, we further distinguish the UE role. If child intermediate relay UE receives its parent UE’s notification, it can decide by itself to perform relay reselection or further send this notification to its child UE. If the remote UE receives its parent UE’s notification, the relay reselection can be triggered.

Proposal 6: During the notification message transfer to its child UE, if the intermediate relay UE receives this notification(cause value of parent PC5 link reselection happened), it is left to intermediate relay UE implementation to trigger relay reselection or further forward this notification message to its child UE.

Proposal 7: During the notification message transfer to its child UE, if the remote UE receives this notification, relay reselection will be triggered.

FFS on Uu upper bound for intermediate relay UE

In RAN2#128 meeting, whether an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is needed or not was discussed and reached the below conclusion:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The first FFS was solved with the below agreement in RAN2#129 meeting:

The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.  FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive.

For the second FFS, in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, some additional restriction should be introduced to avoid the scenario related to the above agreement. RAN2 can leave this issue to SA2, or the UE role can be used as one AS condition to judge that whether a UE can be operated as an intermediate relay UE or not.

Proposal 8: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 9: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: (RRC-6) The FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first can be deleted without further handling.

Proposal 2: (RRC-5) When the intermediate relay UE relay UE received a notification message from the last relay UE indicating a failure on UU, the intermediate relay UE may transmit a notification message downstream towards the remote UE without change the notification message content in Rel-19.

Proposal 3: When the parent UE in idle/inactive performs reselection, RAN2 uses the legacy PC5 notification message to indicate of the reselection for its child UE.

Proposal 4: Introduce a new cause value showing the parent PC5 link reselection happened in Rel-19.

Proposal 5: RAN2 agrees that no condition or timing is introduced for the intermediate relay UE notifies its child node in Rel-19.

Proposal 6: During the notification message transfer to its child UE, if the intermediate relay UE receives this notification(cause value of parent PC5 link reselection happened), it is left to intermediate relay UE implementation to trigger relay reselection or further forward this notification message to its child UE.

Proposal 7: During the notification message transfer to its child UE, if the remote UE receives this notification, relay reselection will be triggered.

Proposal 8: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 9: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Reference

- R2-2502984 Report from session on positioning and sidelink relay	Session chair (MediaTek)	report