3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503433

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Intra-gNB Service Continuity for Multi-hop U2N Relay

Agenda Item:	8.13.4

Document for:	Discussion and Decision

Introduction

In this contribution, based on the agreements at RAN2#129bis meeting as in [1], the details on intra-gNB service continuity for multi-hop U2N relay will be further discussed for Scenario C/D.

Discussion

Second prority Scenarios

In this section, the approach 1 in multi-hop relay CP discussion is considered as baseline.

RAN2#129bis Agreements: RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.

Based on above agreements, the signaling flows and the details for selected steps of scenario C and scenario D are discussed.

Signaling flow and details of Scenario C

Similar as the signaling flow of Rel-17 Intra-gNB U2N Relay D2I path switching, the signaling flow of Scenario C is as below:

Figure-1: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from direct to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details depend on CP discussion.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop relay path.

9.	The data path is switched from direct path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the associated PC5 link for RRCreconfiguration message transmission if needed.

Proposal 1: Suggest RAN2 to adopt Figure-1 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Based on the above Figure-1, most of the steps are same as the Intra-gNB U2N Relay D2I path switching. Hence, in the following, mainly step 1 will be further discussed.

Step 1

RP-242349 Objective: The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

For scenario C/D, the above WID already limit the path switching to a target multi-hop indirect path consisting of the Last Relay UE in “direct” RRC Connected mode and all the other Intermediate Relay(s) in “indirect” RRC Connected mode to the same cell, which means that the network needs to ensure the target Last/Intermediate/First Relay UE will be under the control of network, then the RRCReconfiguration message could be transmitted to each relay UE in the target multi-hop path. This could be totally considered as network implementation to find a target multi-hop relay link with indirect/direct RRC connection. The measurement reporting is sufficient with the legacy source L2 ID and the sidelink measurement quantity result only to the candidate First relay, and serving cell ID of the Last Relay UE, which is already available based on current agreements on R19 multi-hop relay communication. The same conclusion could be used for scenario D.

Proposal 2: (RRC-02) For Remote UE measurement reporting in scenario C/D, it will include legacy reporting content with the L2 ID and the sidelink measurement quantity result of only the candidate First Relay UE, and the serving cell ID of the Last Relay UE.

Proposal 3: It is network implementation to find a target multi-hop relay link with indirect RRC connection for First/Intermediate Relay UE and direct RRC connection for Last Relay UE.

In legacy Rel-17 intra-gNB D2I path switching, the measurement event Y1/Y2 could be reused to let network understand the channel status of remote UE and determine whether a path switching is needed. For Rel-19 intra-gNB direct to multi-hop indirect path switching, the legacy measurement event could be reused since the description on candidate L2 U2N Relay UE should include the candidate First Relay UE, which is already covered by the agreed definition on L2 U2N Relay UE in TS 38.330.

Event Y1: PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) Event Y2: Candidate L2 U2N Relay UE becomes better than threshold

Proposal 4: (RRC-01) The UE will perform the legacy measurement event Y1/Y2, the description on candidate L2 U2N Relay UE in event Y1/Y2 already includes the candidate First Relay UE regarding the agreed definition of L2 U2N Relay UE.

Signaling flow and details of Scenario D

Clarification to Scenario D

Besides the general case with the target First/Intermediate/Last Relay UE(s) different from the source U2N Relay UE, the source Relay UE may be the First Relay UE as in case 1, the Intermediate relay as in case 2 or the Last relay UE in case 3 as below.

- Case 1: Remote-Relay1-gNB(cell1) TO Remote-Relay1-Relay2-Relay3-gNB(cell1/2)

- Case 2: Remote-Relay1-gNB(cell1) TO Remote-Relay2-Relay1-Relay3-gNB(cell1/2)

- Case 3: Remote-Relay1-gNB(cell1) TO Remote-Relay3-Relay2-Relay1-gNB(cell1/2)

For Case 1, similarly to the discussion in Scenario B, it is not needed to support the path switching at group level. This case can be handled in the baseline control plane model, as implementation, the network could trigger the sequential path switches for the remote UE and the source relay UE.

For Case 2, the source U2N Relay UE becomes the target Intermediate Relay, this case can be handled in the baseline control plane model, the network could trigger the sequential path switches for the Remote UE and the source Relay UE.

Proposal 5: (RRC-03) For Scenario D, RAN2 does not specify anything to support the case that the source single-hop Relay UE is the target First/Intermediate Relay UE using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the source Relay UE.

For Case 3, the source Relay UE becomes the target Last Relay UE, similar to the general case, the gNB could send the RRCReconfiguration message to the First/Intermediate Relay UE via the source Relay UE(i.e., the target Last Relay UE) for remote UE, then send the RRCReconfiguration message to the Remote UE via the source Relay UE, the Remote UE will establish the PC5 link to the First Relay UE. Compared to the general case of scenario D, there is no additional impact to spec.

Proposal 6: (RRC-03) For scenario D, RAN2 will support the case that the source single-hop relay UE is the last relay UE on the target path using the Rel-19 I2M path switching with different target last relay UEs as baseline. Spec impact will be minimised.

The signaling flow for Scenario D

Similar as the signaling flow of Rel-18 U2N Relay I2I path switching, the signaling flow of Scenario D is as below:

Figure-2: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from single-hop indirect to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details could be FFS and depend on CP discussion

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop Relay path.

9.  The gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the source L2 U2N Relay UE can be sent any time after step 6 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

10.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 6 or step7.

11.	The data path is switched from single-hop indirect path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the PC5 link if needed.

Proposal 7: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB single-hop indirect to multi-hop indirect path switching.

Based on the above figure, most of the steps are same as the Rel-18 Intra-gNB I2I path switching. Hence, in the following, mainly step 1 will be further discussed:

Step 1

For scenario D, similar to the analysis to scenario C, besides the enhanced measurement result reporting on candidate multi-hop relay path information, the legacy measurement event X2/Y2/Z1, as below could be reused. The term of candidate L2 U2N Relay should include the candidate First Relay UE, which is already covered by the agreed definition on L2 U2N Relay UE in TS 38.330.

Event X2: Serving L2 U2N Relay UE becomes worse than threshold Event Y2: Candidate L2 U2N Relay UE becomes better than threshold Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

Proposal 8: (RRC-02) The UE will perform the legacy measurement event X2/Y2/Z1, the description on candidate L2 U2N Relay UE in event Y2/Z1 already includes the candidate First Relay UE regarding the agreed definition of L2 U2N Relay UE.

Conclusion

According to the analysis in section 2, it is proposed:

For Scenario C

Proposal 1: Suggest RAN2 to adopt Figure-1 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Proposal 2: (RRC-02) For Remote UE measurement reporting in scenario C/D, it will include legacy reporting content with the L2 ID and the sidelink measurement quantity result of only the candidate First Relay UE, and the serving cell ID of the Last Relay UE.

Proposal 3: It is network implementation to find a target multi-hop relay link with indirect RRC connection for First/Intermediate Relay UE and direct RRC connection for Last Relay UE.

Proposal 4: (RRC-01) The UE will perform the legacy measurement event Y1/Y2, the description on candidate L2 U2N Relay UE in event Y1/Y2 already includes the candidate First Relay UE regarding the agreed definition of L2 U2N Relay UE.

For Scenario D

Proposal 5: (RRC-03) For Scenario D, RAN2 does not specify anything to support the case that the source single-hop Relay UE is the target First/Intermediate Relay UE using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the source Relay UE.

Proposal 6: (RRC-03) For scenario D, RAN2 will support the case that the source single-hop relay UE is the last relay UE on the target path using the Rel-19 I2M path switching with different target last relay UEs as baseline. Spec impact will be minimised.

Proposal 7: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB single-hop indirect to multi-hop indirect path switching.

Proposal 8: (RRC-02) The UE will perform the legacy measurement event X2/Y2/Z1, the description on candidate L2 U2N Relay UE in event Y2/Z1 already includes the candidate First Relay UE regarding the agreed definition of L2 U2N Relay UE.

Reference

- Report from session on positioning and sidelink relay, MediaTek, RAN2#129bis meeting.