3GPP TSG-RAN WG2 Meeting #130	R2- 2503446

St. Julians, Malta, 19 – 23 May, 2025

Agenda Item:	8.20.1

Source: 	Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL

Title:  	Discussion on DL MIMO layer capability for 6Rx UE

Document for:	Discussion and decision

# Introduction

RAN2 receives a LS from RAN4 on DL MIMO layer capability for 6Rx UE. In this paper, we focus on the capability signaling design for DL MIMO layer for 6Rx UE.

# Discussion

As captured in RAN4 LS [1], maximum 6 DL MIMO layer is introduced as optional feature:

RAN4 discussed the support of DL MIMO layers operation for UEs equipped with 6Rx chains for FWA and handheld devices and reached the following conclusions in RAN4 #114: - 6-layer performance requirements will be only defined for FWA (Fixed Wireless Access) UE devices. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications During the following RAN#107 meeting it was agreed to: - Introduce 6 MIMO layers support as an optional feature only for FWA, which is captured as the objective in the latest WID RP-250780. During the RAN4#114bis meeting the following further conclusions have been reached: - Introduce UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH in addition to the existing capability of 2, 4 and 8 maximum DL MIMO layers. - The capability signalling shall be defined as optional. - The signalling details are up to RAN2. Note: In the scope of this WI RAN4 will introduce requirements for both FWA and handheld type of 6Rx devices in RAN4 specifications per agreement in RAN4 #114.

As mentioned above, in addition to existing supported maximum 2, 4, and 8 DL MIMO layer, UE capability signaling to indicate support of maximum 6 DL MIMO layer needs to be introduced.

It was further agreed in RAN4 that 6 DL MIMO layer requirement are applicable for FWA 6Rx devices will be explicitly captured in RAN4 specification, i.e., TS 38.101-4 [2].

Proposal 1: Introduce a new capability for maximum 6 DL MIMO layer as optional capability.

As captured in TS 38.101-1 [3], 8 Rx operation is only supported for FWA/CPE/vehicle/industrial devices form factor, i.e., 8 Rx operation is not supported by handheld UE.

Furthermore, it is clear that the maximum DL MIMO layer is limited by the number of Rx supported by a UE. This means, a UE with a lower number of Rx cannot support a higher number of DL MIMO layer. For example, a handheld UE with 4Rx receiver cannot support 8 DL MIMO layer. Based on above, 8 DL MIMO layer is also only applicable for FWA devices.

As observed in TS 38.306 and TS 38.331, common signaling design for 2, 4, and 8 DL MIMO layer is used, without differentiating FWA and handheld UE:

#### – MIMO-Layers The IE MIMO-Layers is used to convey the number of supported MIMO layers. MIMO-Layers information element -- ASN1START -- TAG-MIMO-LAYERS-START MIMO-LayersDL ::= ENUMERATED {twoLayers, fourLayers, eightLayers} MIMO-LayersUL ::= ENUMERATED {oneLayer, twoLayers, fourLayers} -- TAG-MIMO-LAYERS-STOP -- ASN1STOP

There’s no restriction in RAN2 specification that 8 DL MIMO layer is only supported for FWA.

Observation 1: 8 DL MIMO layer can only be supported via 8 Rx receiver, which is only applicable for FWA. Generic signaling design in RAN2 specifications is used to indicate 8 DL MIMO layer in maxNumberMIMO-LayersPDSCH without differentiating FWA and handheld UE.

Based on above observation and discussion, same as 8 DL MIMO layer, RAN2 should also define a generic signaling without differentiating FWA and handheld UE in RAN2 specification, as such requirement differentiation between FWA and handheld UE is captured in RAN4 specification.

Proposal 2: RAN2 specification will not capture 6 DL MIMO layer is only applicable for FWA.

Furthermore, to support backward compatibility, UE shall indicate both maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900, where the legacy gNB that cannot interpret the new capability can use the legacy capability (i.e., maxNumberMIMO-LayersPDSCH) instead. On the other hand, since maxNumberMIMO-LayersPDSCH-v1900 indicates the maximum DL MIMO layer supported by a UE, if it is reported, it is straightforward that the UE can also support 2 and 4 DL MIMO layer. Therefore, when both capabilities are reported together for a CC, the network which is able to decode the maxNumberMIMO-LayersPDSCH-v1900 can ignore the value reported in maxNumberMIMO-LayersPDSCH.

Proposal 3: A UE supporting 6 DL MIMO layer shall indicate both maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 for a CC. When both capabilities are reported, the network which is able to decode the maxNumberMIMO-LayersPDSCH-v1900 ignores the maxNumberMIMO-LayersPDSCH.

Based on the above discussion, CRs (38.306 and 38.331) are implemented in the companion contributions [4] and [5].

# Conclusion

Based on above discussion, we have following observations and proposals:

Proposal 1: Introduce a new capability for maximum 6 DL MIMO layer as optional capability.

Observation 1: 8 DL MIMO layer can only be supported via 8 Rx receiver, which is only applicable for FWA. Generic signaling design in RAN2 specifications is used to indicate 8 DL MIMO layer in maxNumberMIMO-LayersPDSCH without differentiating FWA and handheld UE.

Proposal 2: RAN2 specification will not capture 6 DL MIMO layer is only applicable for FWA.

Proposal 3: A UE supporting 6 DL MIMO layer shall indicate both maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 for a CC. When both capabilities are reported, the network which is able to decode the maxNumberMIMO-LayersPDSCH-v1900 ignores the maxNumberMIMO-LayersPDSCH.

# Reference

[1] R4-2505225 LS on DL MIMO layers capabilities for 6Rx UEs

[2] TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

[3] TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[4] R2-2503447, Introduction of 6 DL MIMO layer

[5] R2-2503448, Introduction of 6 DL MIMO layer